Source	CANADA NEWSWIRE

Date	06/10/2020

Time	05:03:21 AM

Company	 Frontier Lithium Inc.

Title	Electric Avenue target confirmed by Channel Sampling 16 m of 2% Li2O 25

km from the PAK and Spark deposits

CDNX Symbol: FL

Press Release

" BC-FrontierLithiumSample   06-10 1374

#News release via Canada NewsWire, Toronto 416-863-9350 -ME-

Attention co.122896:

^Electric Avenue target confirmed by Channel Sampling 16 m of 2% Li2O 25

km from the PAK and Spark deposits@

Highlights

<<

--  The entire channel of 16 metres grades 2% Li2O with a 12 m zone

averaging 2.35% Li2O;

--  The channel is part of a 1.5 km long pegmatite dyke;

--  Electron-microprobe on Spodumene confirms low impurities similar to PAK;

--  Substantiates fertility of Ontario's Electric Avenue for high-quality

LCT pegmatites hosting rare low-iron spodumene.

>>

SUDBURY, ON, June 10, 2020 /CNW/ - Frontier Lithium Inc. (TSXV: FL) (the

"Company" or "Frontier") is pleased to report channel sampling results from

Pennock, a third LCT (lithium, cesium, tantalum) pegmatite, on the PAK Lithium

Project. A 1.5 km-long pegmatite dyke was located, which is intermittently

exposed and contains white to light grey spodumene and lepidolite. The dyke is

oriented in an east-west direction, cross-cutting the main trend of the

Bearhead Lake Fault and culminates at a 16 m wide exposed "blow-out" near the

western end (Figure 1). During the drilling program on Spark in July 2019, the

support helicopter was used to transport a two-person crew to the main

blow-out area of the Pennock pegmatite to complete the channeling.

Figure 1: Location and Compilation of the Pennock Showing Area

"The overall high lithium grade of the channel coupled with the presence

of the unique mineral texture SQUI (spodumene-quartz-intergrowth) and

corresponding low impurity spodumene is very exciting." states Garth Drever,

VP Exploration. "This is another landmark on the Project supporting the

Electric Avenue as a rare, high-quality North American corridor for LCT

Pegmatites"

Summary information including the location of the channel and mineralized

intercepts is shown in Table 1 while Figure 2 contains photographs of the

blow-out area where the channel was cut.

Table 1: Summary of Channel sampling at Pennock Pegmatite

Channel     Date Drilled                 UTM Zone 15N (NAD83)

Collar Orient            Metres Cut

---

---

Start           End            Easting        Northing

(mASL)                  Azim           Dip         Start        End

---

---

CH-45  2019-07-28       2019-07-29

456,175                     5,844,545                  332.0

200           0           0          16

---

Total metres

cut

16

---

---

Channel         From               To

Length                            Li2O                   Cs2O

Ta2O5        Nb2O5       SnO2        Rb2O         Geology

(m)              (m)             (m)                             (%)

(%)         (ppm)        (ppm)       (ppm)       (%)

---

---

CH-45                 0              16

16                          1.96                  0.004           80

23          70        0.22           LIZ/aplite

---

Including                    1              13

12                          2.35                  0.004           58

18          52        0.19               LIZ

---

Figure 2: Photographs of the Pennock Pegmatite showing Blow-out area and

location of channel sampling

The 16 m channel averaged 1.96% Li2O with a 12 m length at 2.35% Li2O.

Much of the exposed surface has textures similar to the SQUI

(Spodumene-Quartz-Intergrowths) at PAK where the spodumene laths have a

preferred orientation as shown in Figure 2 rather than random "jackstraw"

textures.

The Electron Microprobe work (initially report in Press Release dated

February 28, 2019) was expanded to include polished thin sections from the

summer drill program at Spark and the channel sampling at Pennock. Polished

thin sections were prepared from 14 outcrop, channel and diamond drill hole

samples from the Spark pegmatite and 3 outcrop and channel samples from the

Pennock showing for the purpose of quantitative analysis of spodumene using

electron probe microanalysis. The goal was to quantify the contents of

inherent iron, sodium, and other major elements within the spodumene.

In total, 236 analyses were performed on the Spark thin sections and 43

analysis on the Pennock sections. Figures 3 and 4 summarize the contents of Fe

and Na in spodumene, which are the two most abundant substituting elements.

Spodumene from the Spark pegmatite has iron contents ranging from below

detection (0.03% Fe2O3) to 0.51% Fe2O3, averaging 0.16% Fe2O3. Spodumene from

the Pennock "blow-out" outcrop has iron contents ranging from below detection

to 0.28% Fe2O3, averaging 0.06% Fe2O3. Spodumene from a dike 380 m to the

southeast of the Pennock outcrop has similar Fe2O3 contents averaging 0.05%

Fe2O3. In comparison, spodumene from the Lower Intermediate Zone (LIZ) and

Upper Intermediate Zone (UIZ) at the PAK deposit average around 0.09% and

0.03% Fe2O3 (Figures 3 and 4A), respectively.

Figure 3: Scatter plot showing Al2O3 vs. Fe2O3 in spodumene from results

of electron microprobe; fields for spodumene in the upper intermediate zone

(UIZ; solid line) and lower intermediate zone (LIZ; dashed line) at the

Pakeagama Lake pegmatite (PAK) are shown for

Figure 4: A - Box plot showing the distribution of Fe2O3 within spodumene

from the Spark pegmatite, the Pennock outcrop and dike, the upper intermediate

zone (UIZ) and lower intermediate zone (LIZ) at the Pakeagama Lake pegmatite

(PAK); sample counts are shown in F

Spodumene from the Spark pegmatite and the Pennock showing have sodium

contents ranging from 0.04 to 0.13% Na2O, averaging 0.08% Na2O which is

similar to Spodumene at the PAK pegmatite (Figure 4B). The other major

elements analyzed, which are Ti, Mg, Mn, Ca, and K, were near or below the

lower limit of detection in all the analyses and do not constitute a

significant component of spodumene from either location.

Due Diligence

All scientific and technical information in this release has been

reviewed and approved by Garth Drever, P.Geo., the qualified person (QP) under

the definitions established by National Instrument 43-101. Under Frontier's

QA/QC procedures, channels are cut with a motorized circular diamond saw, and

are 4 cm wide and 10 cm deep. The channels are typically oriented

perpendicular to the strike of the pegmatite internal mineralogical zones and

are cut continuously across the zones. The samples are removed using a hammer

and chisel. The length of a channel is determined by the width of the

pegmatite zone. Sample lengths typically are 1 metre but can be less depending

on zone mineralogy and boundaries. Samples are described, placed into a poly

sample bag with a numbered sample tag and then shipped to the assay lab for

quantitative multi-element analysis.

All samples were assayed by AGAT Laboratories Ltd. ("AGAT"), an ISO

accredited laboratory in Mississauga, Ontario. Sample blanks along with

tantalum, lithium, rubidium and cesium certified reference material was

routinely inserted into the sample stream in accordance with industry

recommended practices. All petrographic sample preparation and electron

microprobe (EMP) analysis was completed at the Queen's Facility for Isotope

Research (QFIR) at Queen's University in Kingston, Ontario under the direct

supervision of Dr. Steve Beyer. Field duplicate samples were also taken in

accordance with industry recommended practices.

About Frontier Lithium Inc.

Frontier Lithium is a Sudbury based, publicly listed, junior mining

company with the largest land position in an emerging premium lithium mineral

district located in the Great Lakes region of northern Ontario. The company is

in late-stage exploration, early-stage development phase with the near-term

goal of building a Demonstration concentrator under an advanced exploration

permit. The company's PAK Lithium Project contains North America's

highest-grade concentration of rare low-iron, lithium bearing mineral

spodumene and is Ontario's largest resource with significant upside potential

to increase. Frontier Lithium's current objective is to demonstrate the

potential of the PAK Lithium Deposit to produce up to 2,000 tonnes Lithium

Carbonate Equivalent ("LCE") of premium spodumene concentrates for glass

producers and for feedstock to produce lithium chemicals.

Frontier maintains a tight share structure with management ownership

exceeding 30% of the Company.

About the PAK Lithium Project

The PAK Lithium Project encompasses 26,774 hectares at the south end of

Ontario's Electric Avenue, which is the largest land package hosting lithium

bearing pegmatites in Ontario. The Project covers 65km of Avenue length and

remains largely unexplored; however, since 2013 the company has delineated two

premium spodumene bearing lithium deposits located 2.3 km from each other at

the southwestern end of the project. Recently, Frontier confirmed the presence

of spodumene with the Pennock Lake pegmatite occurrence a further 30km along

Frontier's PAK Lithium Project.

The PAK deposit has a mineral reserve in proven and probable category of

5.77 MT averaging 2.06% Li2O. The deposit hosts a rare technical/ceramic grade

spodumene with low inherent iron (below 0.1% Fe2O3). The "2019 Discovery of

the year award" winning Spark Deposit was initially discovered in September

2018 and has a mineral resource estimate of 3.2 MT averaging 1.59% Li2O in

indicated and 12.2 MT averaging 1.36% Li2O in inferred category, as per NI

43-101 Technical Report, "PAK prefeasibility study, Spark resource estimation"

by WSP Canada Inc. and Nordmin Engineering Ltd., updated on March 23, 2020.

SOURCE Frontier Lithium Inc.

View original content to download multimedia:

http://www.newswire.ca/en/releases/archive/June2020/10/c2422.html

-0-                           06/10/2020

/For further information: Bora Ugurgel, Manager, Investor Relations, 2736

Belisle Drive Val Caron, ON. P3N 1B3 CANADA, T. +001 705.897.7622, F. +001

705.897.7618/

/Web Site: frontierlithium.com /

(HLKMF FL.)

CO:  Frontier Lithium Inc.

ST:  Ontario

IN:  MNG OTC MIN

SU:

-30-

#