Source	Marketwire

Date	01/23/2014

Time	10:21:13 AM

Company	 NORTHAVEN RESOURCES CORP.

Title	Northaven Resources Corp. Provides Clarification of Prior Disclosure on

Regal Silver Mine

CDNX Symbol: NTV

Press Release

NEWS RELEASE TRANSMITTED BY MARKETWIRED

FOR: Northaven Resources Corp.

TSX VENTURE SYMBOL: NTV

January 23, 2014

Northaven Resources Corp. Provides Clarification of Prior Disclosure on Regal

Silver Mine

VANCOUVER, BRITISH COLUMBIA--(Marketwired - Jan. 23, 2014) - NORTHAVEN

RESOURCES CORP. (TSX VENTURE:NTV) advises that, as a result of a review by the

British Columbia Securities Commission, we are issuing the following news

release to clarify our prior disclosure.

On December 3, 2013, we issued a news release (the "December News Release")

disclosing contents of a historical technical report (the "Report") dated 1971

containing a positive feasibility review and recommendation for production at

that time on its Regal Silver polymetallic mine property 31 km north east of

Revelstoke, B.C. The news release disclosed certain information which we would

like to clarify.

Historical Resource

The December News Release contained a table describing silver-lead-zinc and

tungsten mineralization comprising the Historical Estimate disclosed in the

Report which might have been read as implying that the grades reported for

Tungsten applied to the entire tonnage reported for the silver-lead-zinc

mineralization, rather than being limited to the smaller tonnage reported next

to the reported tungsten grade, which is in fact the case.

While the tungsten mineralization occurs throughout the mine mineralization in

intimate association with the silver-lead-zinc mineralization, no tungsten

grade was reported for the historical silver-lead-zinc estimate. Instead, the

Authors selected certain areas of the veins containing higher grades of

tungsten for determination of a tungsten resource which was reported separately

and recommended for separate treatment as a matter of practical processing,

described as follows:

/T/

-------------------------------------------   ------------------------------

SILVER-LEAD-ZINC MINERALIZATION          TUNGSTEN (WO3) MINERALIZATION

-------------------------------------------   ------------------------------

Silver   Lead   Zinc                         Tungsten

Category(i)       Tons (troz)    (%)    (%)   Category(i)       Tons  (WO3)%

-------------------------------------------   ------------------------------

"Probable                                     "Probable

Reserve"(i)   114,146   3.24   3.32   1.58   Reserve"(i)      9,060    1.09

-------------------------------------------   ------------------------------

"Possible                                     "Possible

Reserve"(i)   323,911   3.15   3.26   1.61   Reserve"(i)      9,700    1.02

-------------------------------------------   ------------------------------

(i) These 1971 estimates pre-date National Instrument 43-101 ("NI 43-101").

The Company has not independently verified the above results and

considers them to be "Historical Estimates" under NI 43-101. A qualified

person has not done sufficient work to classify the historical estimates

as a current mineral resource and the Company is not treating them as

current mineral resources or reserves.

/T/

Comparison of Historical Estimate to Current Resource Estimates

The December News Release indicated that mineralization classified in the

Report as "Probable Reserve" corresponds to "a Probable Mineral Reserve" and

"Possible Reserve" to an "Inferred Mineral Resource" under NI 43-101, based on

calculations and costs at the time of the Report. Readers are cautioned that,

although the Report Authors used both reserve categories in their Historical

Estimate to support their production recommendations at the time, a historic

"Probable Reserve" cannot be compared to a current reserve because there is no

current economic analysis to demonstrate that economic extraction could be

justified. Further, it appears that the mineralization referred to by the

Authors as a historical "Possible Reserve" category may be comparable the

current NI 43-101 "Inferred Resource", which would have precluded its inclusion

in reserve estimates under NI 43-101.

The Resource was calculated based on mineralized vein sections that Authors

concluded had been adequately developed, sampled and check sampled underground,

excluding areas below the cut-off grade, except where necessary to preserve

orderly mining units. The Authors used a cut-off value for silver-lead-zinc

mineralization of $8.91 per ton with assumed metal prices of $1.75/troz for

silver, $0.15/lb for lead and $0.14/lb for zinc and with adjustments for

anticipated metallurgical recovery. A cut off value of $20.32 per ton was used

for Tungsten mineralization using prices of $50/unit for W03 with adjustments

for anticipated metallurgical recovery. Mining and milling costs used were

$3.68 and $2.75/t for silver-lead-zinc mineralization and $15.09 and $2.75 for

Tungsten mineralization, respectively. The Authors appear to have used the

manual polygonal method for calculating resource blocks, using set distances

above and below underground workings and sampling data. Resource blocks

classified as a "Probable Reserve" were calculated mainly by level assays

extending, in most cases, a maximum of 25 feet (7.62 meters) above and below

the assay level.

The Report appears to have been prepared in a competent and diligent manner and

Company considers it to be reliable, subject to verification and upgrading to

current standards.

The Report is also particularly helpful as a guide for further exploration and

development. The Report indicates there is excellent potential for extending

known mineralized veins in and around the Mine. Although there are at least 5-6

veins identified in the mine area, only one ("No 5 vein") provided 95% of the

tonnage in the Historical Estimate, with most of the other veins being excluded

due to limited exploration or development. None of the veins reported in the

Mine have been drilled to depth and all remain open in both strike directions.

Since the Report clearly identifies the location and grades of mineralization

used to calculate the Historical Estimate, it may significantly reduce the time

and expense required to explore and develop the Mine. Readers are again

cautioned that a Qualified Person has not done sufficient work to classify the

Historical Estimate as current mineral resources or mineral reserves and the

Company is not treating the Historical Estimate as current mineral resources or

mineral reserves.

Retraction of previous disclosure of quantities and grades in Exploration

Targets

The Regal Silver Mine lies approximately 2 miles (3.2 km) southwest of a

northwesterly-trending syncline forming part of a sedimentary/structural

sequence extending for several hundred kilometers in both direction being

itself part of a northeast trending sedimentary occurrence characterized by

northeasterly compressional overthrusting and resultant northwesterly-trending

high-angle reverse and normal extensional faulting.

The Allco Silver Workings are located approximately 6.5 kilometers to the

northwest, along the same structure. Both projects and the intervening ground

are all entirely well within the Company's property holdings. The rocks

underlying the entire area are comprised of the argillitic shales and limestone

of the Lardeau and Badshot Formations. At the Regal Silver Mine, repetitive

mineralized quartz veins occur within the mine area which are sub-parallel to

the synclinal axial plane and dip from 65 degrees north east near surface, with

dips flattening to 25 degrees at depths of 400 meters below surface. In the

historical mine, 6 veins were identified over a vertical height of 1200 feet

(265m) to the #10 level, with thicknesses varying from 1 foot (0.3m) to 40 feet

(12.2m), apparent strike lengths of up to 2,500 feet (762m) and inter-vein

spacings of 50 (15.2m) to 150 feet (45.7m). The veins are described as being

predominantly white to grey non-vitreous quartz with pyrite and pyrrohotite

being the principal sulphide minerals. Scheelite is found in intimate

association with the pyrite. Scattered throughout and locally concentrated in

the veins are nests, bands and pods of coarsely crystalline argentifierous

galena, resinous sphalerite, tetrahedrite and ruby silver.

Based on mapping, sampling and limited underground workings, the Allco Silver

showing also has geological features similar to Regal Silver, including

northwest trending quartz vein structures within an anomalous 500 meter wide,

3000 meter long zone, which remains open along the structure.

Veins occur at the Regal Silver mine in a zone over 300 meters wide and have

been mapped for at least 762 meters along the same structure. Examination of

the veining at both projects suggests they are structurally controlled and from

the geological mapping appear to have wide significant strike lengths and

numerous vein occurrences with over 20 separate locations mapped up to 1970.

Limited production at the Allco showing produced shipments of 213 tons of ore

containing 11 troy ounces of gold, 11,211 troy ounces of silver and 173,157

pounds of lead. Although these spectacular grades are not likely representative

of reasonably repeatable grades, they do show remarkably similar mineralizing

suites and similar metal ratios to those reported at Regal Silver, suggesting a

common mineralizing event.

The structural features at Allco and Regal Silver are consistent with the

occurrence of normal and reverse faulting resulting from regional

overthrusting, particularly at the contact between the underlying argyilic and

limestone units. These fault zones would create zones of dilatancy,

particularly in brittle rocks such as the Lardeau shales and Badshot

Limestones, which were later mineralized by hydrothermal solutions arising from

depth. The existence of this geological model is also supported by the

information obtained from the ZTEM geophysical program conducted by the Company

in 2011 (previously reported) which has revealed a significant number of

north-west tending subsurface linear anomalies which appear to have close

connection with known mineralized occurrences in the area.

If the mineral occurrences at both the Allco and Regal Silver are the result of

the same structural and mineralizing events outlined above, then, as a

conceptual model, there may exist a zone of roughly continuous sub-parallel

mineralized quartz veins representing the traces of steep reverse to normal

faulting striking approximately 315 degrees (NW), dipping from 65 to 25 degrees

northeast between and including both the Regal Silver and the Allco Showings.

Based on this model and on the geological mapping within the Regal Silver mine

and at the Allco Silver showings, we would expect the veins to show similar

characteristics to those mapped. This in turn gives rise to an exploration

target having an areal extent of possibly 300m X 6,500m between the two

projects, containing similar vein structures.

The Company retracts an exploration target size and range of grades in the

December News Release, and a target tonnage disclosed in its MD&A dated

November 28, 2013. On further consideration, there is insufficient mapping,

sampling and assaying throughout the target area for us to arrive at a

considered estimate of tonnage and grade ranges for these targets. Accordingly,

we retract our earlier statements as to target size and instead state that we

believe the area to be geologically permissible of an exploration target of

substantial size, with possible mineralization at grades similar to those found

at either the Regal Silver Mine or the Allco Silver Workings. Readers are

cautioned that this exploration target is conceptual in nature and it is

uncertain if further exploration will result in the estimation of ranges of

quantity and grade in the exploration target, or the target being delineated as

a mineral resource.

The Company plans to conduct exploration work to test the above target as well

as the historic Regal Silver mine area in the coming year, subject to funding.

This news release has been reviewed with respect to technical information by

Phil Southam, P. Geo., an independent Qualified Person under National

Instrument 43-101.

On behalf of the Board of Directors of

NORTHAVEN RESOURCES CORP.

Allen D. Leschert, CEO

All other technical data herein is based on information contained in technical

reports or summaries prepared by independent third parties which has not been

independently verified by the Company, and which remains subject to

confirmation by subsequent evaluation. Neither the TSX Venture Exchange nor its

Regulation Service Provider (as that term is defined in the policies of the TSX

Venture Exchange) accepts responsibility for the adequacy or accuracy of this

news release. This news release may contain assumptions, estimates, and other

forward-looking statements regarding future events. Such forward-looking

statements involve inherent risks and uncertainties and are subject to certain

factors, many of which are beyond Northaven's control, which may cause actual

results or performance to differ materially from those currently anticipated in

such statements.

- 30 -

CONTACT INFO:

Northaven Resources Corp.

Malcolm Fraser

President

604 696-3608

malcolm@northavenresources.com

or

Northaven Resources Corp.

Allen Leschert

CEO

604 696-3600

allen@northavenresources.com

www.northavenresources.com

INDUSTRY: Manufacturing and Production - Mining and Metals

SUBJECT: MEX - MINING EXPLORATION UPDATE

If you no longer want to receive announcements from us, please do not reply to

this e-mail.

Instead simply click http://marketwire.com/mw/fmu.jsp?id=464166191