Source	Marketwire

Date	01/07/2016

Time	05:01:36 AM

Company	 KAMINAK GOLD CORPORATION

Title	Kaminak Gold Corporation Announces Positive Feasibility Study Results on

Yukon Coffee Gold Project

CDNX Symbol: KAM

Press Release

NEWS RELEASE TRANSMITTED BY MARKETWIRED

FOR: Kaminak Gold Corporation

TSX VENTURE SYMBOL: KAM

January 7, 2016

Kaminak Gold Corporation Announces Positive Feasibility Study Results on Yukon

Coffee Gold Project

After-Tax NPV5% of C$455 million and IRR of 37 %; Average Life of Mine Annual

Gold Production of 184,000 Ounces and All-In Sustaining Costs of US$550/oz Au

VANCOUVER, BRITISH COLUMBIA--(Marketwired - Jan. 7, 2016) - Kaminak Gold

Corporation ("Kaminak" or the "Company") (TSX VENTURE:KAM) is pleased to

announce the results of a Feasibility Study prepared in accordance with

National Instrument 43-101 (NI 43-101) for the Company's 100% owned Coffee Gold

Project (the "Coffee Project") located 130 km south of Dawson City in Yukon

Territory, Canada. The Feasibility Study indicates that the Coffee Project

represents a robust, rapid pay-back, high margin, ten year open pit mining and

heap leach project that works in the current gold price environment. As such,

Kaminak intends to move forward into mine permitting to support mine

construction, which is planned for mid-2018. The Company is well financed ($28

million as of September 30, 2015) to undertake an aggressive work-plan in 2016

to meet these objectives.

At a gold price of US$1,150/oz and an exchange rate of C$1.00 to US$0.78, the

Coffee Project base case estimate (the "Base Case") generates an after-tax net

present value (NPV) at a 5% discount rate of C$455 million and an internal rate

of return (IRR) of 37%. The proposed mine will operate over an initial ten year

mine-life with average annual gold production in excess of 200,000 ounces for

the first five years (excluding the initial 3 month ramp-up period), and

average annual life-of-mine gold production of 184,000 ounces. Initial capital

expenditure to fund construction and commissioning is estimated at C$317

million, with a life-of-mine capital cost of C$478 million (including C$60

million in closure costs). The all-in sustaining cash costs (as defined per

World Gold Council guidelines, less corporate G&A) is estimated to be US$550

per ounce of gold produced. The project is expected to have a significant

impact on Yukon's GDP, generating over $2 billion of gross revenue and

contributing 480 permanent, high paying jobs.

Eira Thomas, Kaminak President and CEO commented: "This feasibility study

firmly establishes the Coffee Project as one of the world's best undeveloped

gold projects by value and margin that works in the current gold-price

environment. The Coffee Project further benefits from being a simple, open pit,

heap leach mining opportunity, situated near infrastructure that delivers low

all in sustaining costs and pays back capital in under two years". She further

noted, "Kaminak feels privileged to be working in the pro-mining jurisdiction

of Yukon where we enjoy strong relations with all levels of government,

including our local First Nations, with whom we have worked alongside,

collaboratively since 2010."

Accompanying this news release is a video corporate presentation given by Eira

Thomas, Kaminak President & CEO, discussing the Feasibility Study and available

for viewing by clicking the following link:

https://www.webcaster4.com/Webcast/Page/1369/12606, or by visiting the Kaminak

website, www.kaminak.com.

Furthermore, Kaminak is hosting a live Q&A conference call on Thursday, January

7th at 11:00 a.m. Eastern time (8:00 a.m. Pacific time) with the Kaminak

Executive and Feasibility Study team. Participants may join the call by dialing

toll-free North America (866) 393-4306 or International (734) 385-2616 and

providing the company name (Kaminak Gold Corporation) to the operator. A

recorded playback of the call will be available two hours after the call's

completion until January 21st, 2016 by dialing (855) 859-2056 and entering the

conference ID#: 17824557, and on the Kaminak website.

Coffee Project Feasibility Study Overview

The Feasibility Study was initiated in July 2014 after the release of the

Preliminary Economic Assessment in June 2014 and commenced with infill

drilling, geotechnical investigations and other fieldwork to support the study.

The Feasibility Study was prepared and led by JDS Energy and Mining Inc. (JDS),

an established Yukon mine builder, in collaboration with a broad range of

industry leading consultants (see Contributors below),

Highlights (all currencies are reported in Canadian dollars unless otherwise

specified):

/T/

--  A pre-tax NPV5% and IRR of $762 million and 50% respectively;

--  An after-tax NPV5% and IRR of $455 million and 37% respectively;

--  A mine life of ten years with peak annual gold production of 228,000

ounces in project Year 4 and average, steady state, annual gold

production of 193,000 ounces (Years 1-9);

--  2,157,000 ounces of gold mined at head grade of 1.45 g/t Au (Probable

Mineral Reserve of 46.4 Mt at 1.45 g/t Au, containing 2.157 Moz Au.);

--  1,862,000 ounces of gold produced after average metallurgical gold

recoveries of 86.3%;

--  Total cash cost estimated at US$482 /oz Au (including royalties,

refining and transport) and an all-in sustaining cost (as defined by the

World Gold Council less Corporate G&A) estimated at US$550/oz Au,

generating an operating margin of US$600/oz or 52%;

--  Initial and sustaining capital costs, including contingency, for a 100%

owner-operated mine are estimated at $317 million and $161 million

(including $60 million in closure costs) respectively; and

--  A payback of 1.5 years pre-tax and 2.0 years after-tax after the

commencement of first commercial gold production;

/T/

The Feasibility Study proposes four open pits mined by conventional shovel and

truck methods at a nominal ore mining rate of 5 million tonnes per annum for

approximately ten years (with Year 1 being the first full year of commercial

gold production). A total of 312 Mt of material will be mined to produce 46.4

Mt of ore (strip ratio of 5.7 : 1). Run-of-mine ore will be crushed to a 2-inch

feed size and placed on a heap leach pad. Gold will be extracted from the

leachate by an Adsorption-Desorption-Recovery (ADR) carbon plant.

The site will be accessed principally from Dawson City, Yukon, by a 214 km

single-lane, gravel road with pullouts. The cost associated with upgrading

existing road and to construct approximately 37 km of new road along the

proposed 214 km route is estimated at approximately $25 million. Electrical

power will be generated on site by diesel-powered generators. Project

construction time from site mobilization to first commercial production of gold

is estimated to be 18 months, excluding access road construction which will

take approximately 9 months.

Comparison to the June 2014 Preliminary Economic Assessment (PEA)

The PEA presented a broadly similar operation to the Feasibility Study. The key

differences from the PEA presented in the Feasibility Study include:

/T/

--  Total ounces unchanged, with a shorter mine life, higher head grade,

lower reserve tonnage, and higher strip ratio;

--  Increased average total mining rate from approximately 64,000 tonnes per

day (tpd) to approximately 92,000 tpd;

--  A ridge-top heap leach pad with a shorter initial construction period

and lower capital cost per tonne of ore leached than the valley-fill

impounding heap leach facility proposed in the PEA;

--  An access road from Dawson City utilizing a high proportion of existing

gravel roads; and

--  Improved economics due to a shorter construction period, shorter mine

life and higher average gold grade.

/T/

Coffee Gold Feasibility Study Assumptions and Economic Results

The tables below summarize the various assumptions, operational parameters and

economic results of the Feasibility Study. All money values are nominal 2015

Canadian dollars unless otherwise stated.

The following assumptions were applied only in the derivation of the after-tax

valuation of the project:

/T/

--  Pre-development and sunk costs relating to the project (including but

not restricted to exploration and geotechnical investigations, resource

evaluation, fieldwork, metallurgical testwork, environmental and

baseline studies costs, technical studies);

--  Canadian Exploration Expense (CEE) and Canadian Development Expense

(CDE) tax pools with appropriate opening balances;

--  Applicable private and Yukon Crown royalties;

--  Federal and Yukon Territory tax rates; and

--  Specific capital cost class Capital Cost Allowance (CCA) rates over the

life of the project.

/T/

The following inputs were assumed for the project:

/T/

--  Gold Price of US$1,150 / oz.

--  Exchange Rate (US$ : C$) of 0.78

--  Diesel Price (delivered to site) of $0.85/ L for stationary equipment

and $0.89 / L for mobile equipment.

/T/

Feasibility Study Operating Parameters

/T/

----------------------------------------------------------------------------

Run of Mine Ore Delivered to Heap Leach (Mt)                            46.4

----------------------------------------------------------------------------

Run of Mine Ore Average Daily Production (Ktonnes,)                       14

----------------------------------------------------------------------------

Run of Mine Ore Average Annual Production (Mt)                           5.0

----------------------------------------------------------------------------

Average Diluted Ore Grade Delivered to Heap Leach (g/t Au)              1.45

----------------------------------------------------------------------------

Waste (Mt)                                                               265

----------------------------------------------------------------------------

Strip Ratio (Waste : Ore)                                              5.7:1

----------------------------------------------------------------------------

Gold Contained (Moz.)                                                  2.157

----------------------------------------------------------------------------

Gold Produced (Moz.)                                                   1.862

----------------------------------------------------------------------------

Gold Recovery (%)                                                       86.3

----------------------------------------------------------------------------

Average Gold Production Years 1 and 2 (Koz. per year), excluding 3

month ramp-up period                                                    194

----------------------------------------------------------------------------

Initial Capital Cost (including 8.9% contingency, excluding working

capital)                                                            $  317M

----------------------------------------------------------------------------

Sustaining Capital (including 9.3% contingency and closure costs)    $  161M

----------------------------------------------------------------------------

Life-of-mine Capital (including 9.1% contingency)                    $  478M

----------------------------------------------------------------------------

Unit Operating Costs (per tonne ore leached)

----------------------------------------------------------------------------

Mining $ 15.26

----------------------------------------------------------------------------

Processing $  4.97

----------------------------------------------------------------------------

Site Services $  0.97

----------------------------------------------------------------------------

General & Administrative $  2.89

----------------------------------------------------------------------------

Total Operating Costs (per tonne ore leached)                        $ 24.10

----------------------------------------------------------------------------

/T/

Economic Summary at US$1,150/oz. gold:

/T/

----------------------------------------------------------------------------

Total LOM Operating Cash Flow                                      $  1,614M

----------------------------------------------------------------------------

Total LOM Pre-Tax Free Cash Flow                                   $  1,113M

----------------------------------------------------------------------------

Average Annual Pre-Tax Free Cash Flow                              $    137M

----------------------------------------------------------------------------

LOM Income Taxes                                                   $    430M

----------------------------------------------------------------------------

Total LOM After-Tax Free Cash Flow                                 $    682M

----------------------------------------------------------------------------

Average Annual After-Tax Free Cash Flow                            $     93M

----------------------------------------------------------------------------

----------------------------------------------------------------------------

Discount Rate                                                             5%

----------------------------------------------------------------------------

Pre-Tax NPV                                                        $    762M

----------------------------------------------------------------------------

Pre-Tax IRR                                                              50%

----------------------------------------------------------------------------

Pre-Tax Payback (after start of commercial gold production)              1.5

years

----------------------------------------------------------------------------

After-Tax NPV                                                      $    455M

----------------------------------------------------------------------------

After-Tax IRR                                                            37%

----------------------------------------------------------------------------

After-Tax Payback (after start of commercial gold production)            2.0

years

----------------------------------------------------------------------------

/T/

All-in Cash Costs Including Sustaining Capex

/T/

----------------------------------------------------------------------------

On-Site Mining & Rehandle                                           $   707M

----------------------------------------------------------------------------

On-Site Processing                                                  $   230M

----------------------------------------------------------------------------

Surface and Infrastructure                                          $    45M

----------------------------------------------------------------------------On-S

ite G&A                                                         $   134M

----------------------------------------------------------------------------

Refining                                                            $     2M

----------------------------------------------------------------------------

Royalties                                                           $    29M

----------------------------------------------------------------------------

Sustaining Capex                                                    $   100M

----------------------------------------------------------------------------

Closure                                                             $    60M

----------------------------------------------------------------------------

Total  $  1309M

----------------------------------------------------------------------------

All-in Cash + Sustaining Cost (C$/oz)(i)                                 705

----------------------------------------------------------------------------

All-in Cash + Sustaining Cost (US$/oz)(i)                                550

----------------------------------------------------------------------------

(i)All-in Sustaining Costs are presented as defined by the World Gold

Council ("WGC") less Corporate G&A. Calculated as (Refining Costs + 3rd

party royalties + operating costs + sustaining capital costs + closure

capital costs) / payable Au oz.

/T/

The Base Case set out above represents the value of the project from the known

resources. The Base Case assumes an owner-operated open pit mine, access to

site via a gravel road (approximately 295 days per annum) and diesel power

generation. Mining will take place 365 days per annum, and the heap leach pad

is planned to be operated 275 days per annum with no crushing and stacking of

ore occurring during January through March.

Sensitivities

/T/

----------------------------------------------------------------------------

Gold Price

US$/oz(ii)           $1,000  $1,100  $1,200  $1,250  $1,300  $1,400  $1,500

----------------------------------------------------------------------------

Pre-Tax NPV 5% C$M       504     676     848     934    1021    1193    1365

----------------------------------------------------------------------------

After-Tax NPV 5% C$M     295     402     509     562     616     722     828

----------------------------------------------------------------------------

Pre-Tax IRR              37%     46%     55%     59%     63%     71%     79%

----------------------------------------------------------------------------

After-Tax IRR            27%     34%     40%     43%     46%     52%     58%

----------------------------------------------------------------------------

Pre-Tax Payback

Years                   2.2     1.7     1.3     1.1     1.0     0.8     0.7

----------------------------------------------------------------------------

After-Tax Payback

Years                   2.7     2.2     1.8     1.6     1.5     1.2     1.1

----------------------------------------------------------------------------

(ii)At an exchange rate of C$1.00 to US$0.78

/T/

Opportunities to Enhance Value

Value-enhancing opportunities, such as contract mining, the acquisition of high

quality, second hand mining and ancillary equipment, crushing and process

plant, and accommodation quarters, will be further investigated as the project

moves into detailed engineering and procurement. As operations ramp up and a

greater understanding of the ore and heap leach facility performance evolve,

extending the duration of the crushing / stacking period, the period and

temperature of solution heating, and increasing the size of the ore feeding the

heap leach pad will be evaluated. Current oil prices indicate that diesel-only

power generation is the most cost effective power option. As diesel prices

increase, partial replacement of diesel with LNG will become more cost

effective.

There is potential for resource expansion along strike and to depth, with two

existing inferred gold resources (Kona North and Sumatra), and seven separate

drill discoveries in the early stages of evaluation located proximal to the

proposed mine site which contain more than 25 km of priority gold in soil

anomalies remaining to be systematically drill-tested.

Coffee Project Feasibility Study Major Components

Metallurgy

Ore samples for metallurgical testing were taken from bulk surface samples and

drill core composites. A total of 40 column tests were completed at both

ambient lab and simulated cold-climate temperatures, composited from 4500

samples, representing 29,608 kg from 123 drill holes and 8 trenches (at 1/2",

1", 2" and 6"crush sizes). In addition to column leach tests, metallurgical

testing included bottle roll leaching, flotation, column percolation and drain

down, multi-element head assay analyses, column-leach head and tail assay

screen analyses, ball mill work indices, crushing impact, and abrasion indices.

Fire assays for gold and silver were also conducted. Major conclusions from the

test program include:

/T/

--  Coffee Project ores generally leach very rapidly with low reagent

consumption

--  Ore agglomeration is not required;

--  Cyanide soluble assays from over 14,000 samples confirmed that cyanide

soluble recovery is a reliable method to determine gold recovery;

--  Differences in recovery observed between ambient lab temperature leach

tests and simulate cold climate leach tests were negligible;

--  Optimal gold recovery is achieved at an ore size of 80% passing a 50 mm

screen size (P80 of 50 mm; 2"); and

--  An average plant gold recovery of 86.3% was determined from

metallurgical testwork results and after the application of discount

factors (approx. 3%) to simulate plant operating conditions.

/T/

Geology and Mineralization

The Coffee Project area is underlain by a package of metamorphosed Paleozoic

rocks of the Yukon-Tanana Terrane comprising metasedimentary schist and

orthogneiss which form a west-northwest plunging antiform with gently dipping

limbs. The Palaeozoic sequence is intruded by Mid Cretaceous granite and

younger intermediate dikes.

These units are host to gold mineralization that is hydrothermal in origin and

structurally controlled, forming steeply dipping strike extensive mineralized

lodes in broadly E-W, N-S and NE -SW trending structural corridors.

Mineralization is associated with both polyphase brecciation and intense

sulphidation of mica-rich host rocks resulting in the formation of gold-bearing

arsenian pyrite. Extensive oxidation occurred due to ingress of meteoric water

downwards and along the structural deformation zones. This process resulted in

the oxidation of the arsenian pyrite along rims and cracks causing the release

of micron-scale free gold.

Mineral Resource Estimate

The Feasibility Study is based on an Indicated Mineral Resource estimate

undertaken by Robert Sim, P.Geo., an independent Qualified Person of SIM

Geological Inc. (Kaminak News Release, September 23rd, 2015). The mineral

resource estimate incorporated 1,682 diamond core and reverse circulation drill

holes completed from 2010 to 2015 for a total of 280,000 metres. This estimate

defines a total Indicated mineral resource of 63.7 million tonnes averaging

1.45 g/t Au for 2,968,000 ounces of contained gold, and a total Inferred

mineral resource of 52.4 million tonnes at an average grade of 1.31 g/t Au for

2,212,000 ounces of contained gold. These mineral resources are inclusive of

mineral reserves. Cut-off thresholds used in the resource estimate reflect the

projected metallurgical characteristics at various degrees of oxidation; 0.3

g/t Au for Oxide and Upper Transition, 0.4g/t Au for Middle Transition and at 1

g/t Au for Lower Transition and Sulphide types.

A series of resource block models, covering the limits of the four deposit

areas, were produced with gold grades estimated by ordinary kriging into model

blocks measuring 10m along strike, 2.5m across strike and 5m in the vertical

dimension. A capping strategy was employed to control the effects of

potentially anomalous, high-grade, sample data during block grade

interpolation. The ratio of cyanide soluble gold verses total gold content was

utilized to assign the various Oxide ore types in the block models. Mineral

resources are classified according to their proximity to sample locations and

are reported according to the CIM Definition Standards for Mineral Resources

and Mineral Reserves.

Mineral Reserve Estimate

The Mineral Reserve for the Property was estimated by Dino Pilotto, P.Eng., an

independent Qualified Person of JDS.

Cut-off grades, ranging from 0.27 to 1.40 g/t, were determined for each deposit

and ore type and based on appropriate mine design criteria and the adopted

mining method. The Mineral Reserves were estimated at a gold price of US$1,200

/ oz., and an exchange rate of C$1.00 to US$0.87, with the application of

dilution and recovery factors appropriate to an open pit mining method. The

factors applied to generate the pit shells were the best estimate of gold price

and exchange rate at the time the pit shells were generated. These factors were

not those utilized to generate the final project economics, but reserve

estimates using the lower gold price and exchange rate indicate that there were

not any material differences. Pit optimizations and analyses were conducted to

determine the optimal mining shells.

As input to the initial pit limit optimization and subsequent mine scheduling,

and in order to reflect the selectivity of the mining method chosen when

compared to the block model parameters, an external mining dilution was

calculated and applied for each of the various deposits. This external mining

dilution was based on a calculation of the number of waste blocks adjacent to

an ore block in the mineral inventory block model. Only blocks which were

contained within the resource classification of Indicated and above the given

gold cut-off grade were considered as ore blocks.

Probable Mineral Reserves are estimated at 46.4 Mt at 1.45 g/t Au, containing

2.157 Moz Au.

Mining

An open pit, shovel and truck mining method was applied to the Coffee Project

deposits. Mining is scheduled at the start of Year -1 in order to ensure the

delivery of a minimum of 3.5 Mt of ore to the heap leach pad prior to the first

winter shut-down, and to provide sufficient quantities of waste material to

facilitate construction activities. Mining of the deposit is planned to produce

a total of 46.4 Mt of ore and 265 Mt of waste (5.7:1 strip ratio) over a

ten-year project production life, including the initial year of pre-production.

Mining is planned to continue throughout the year. During the crusher and heap

leach shutdown in the first three months of each year uncrushed ore will be

stockpiled. The mine plan focuses on achieving consistent annual total material

production rates of between 34 and 40 Mt, while ensuring a steady-state ore

production rate of 5 Mt per annum.

The Supremo pit contributes approximately 71% (32.9 Mt) of the total ore and

commences mining in Year 2 of the project. It is planned to remain in

production to the end of mine life. The Latte (25%, 11.5 Mt), Double Double

(2%, 1.1 Mt) and Kona (2%, 0.9 Mt) pits will contribute early production and

will be completed by Year 3. This schedule endeavours to prioritize the early

production of higher value material where practical.

Open pit mining operations will use a fleet comprising industry-standard 16 m3

shovels, 12 m3 front-end loaders, 4 m3 excavators, and 144 t haul trucks. This

fleet will be supplemented by appropriately sized drills, graders, and dozers.

A 5 m bench height was selected for mining in ore and waste with overall 20 m

effective bench heights based on a quadruple-bench configuration. The handling

of the fine ore from the crusher to the heap leach pad is included in the open

pit scheduling and operating cost estimation.

Waste rock will be placed in three engineered waste rock facilities proximal to

the pits from which the waste is sourced. Some waste rock will be backfilled

into mined out pits at Latte, Supremo and Double Double in order to create

causeways and facilitate ore haulage routes to the crusher. Waste rock from the

Kona pit, due to its geochemical characteristics, will be placed in a temporary

waste rock facility adjacent to the pit during mining and then backfilled into

the mined-out Kona pit.

Processing

The process includes a two-stage crushing plant feeding a heap leach operation.

Gold is extracted by an adsorption-desorption-recovery (ADR) carbon plant. The

process is based on a heap leach processing rate of 5.0 million dry tonnes per

year at an average feed grade of 1.45 g/t and an anticipated overall recovery

of 86.3%. The crushing rate will be approximately 18,200 tpd over the 9-month

operating period. The process plant, located in close proximity and at a lower

elevation to the heap leach facility, minimizes pumping by allowing gravity

feed of pregnant solution.

Crushed ore will be trucked to the heap leach facility and spread by dozer.

Crushing and heap leach stacking will be suspended between January and March in

order to maintain the thermal integrity of the heap. The heap leach facility

will operate all year with the solution heated as necessary to maintain thermal

NEWS RELEASE TRANSMITTED BY MARKETWIRED

FOR: Kaminak Gold Corporation

TSX VENTURE SYMBOL: KAM

January 7, 2016

Kaminak Gold Corporation Announces Positive Feasibility Study Results on Yukon

Coffee Gold Project

After-Tax NPV5% of C$455 million and IRR of 37 %; Average Life of Mine Annual

Gold Production of 184,000 Ounces and All-In Sustaining Costs of US$550/oz Au

VANCOUVER, BRITISH COLUMBIA--(Marketwired - Jan. 7, 2016) - Kaminak Gold

Corporation ("Kaminak" or the "Company") (TSX VENTURE:KAM) is pleased to

announce the results of a Feasibility Study prepared in accordance with

National Instrument 43-101 (NI 43-101) for the Company's 100% owned Coffee Gold

Project (the "Coffee Project") located 130 km south of Dawson City in Yukon

Territory, Canada. The Feasibility Study indicates that the Coffee Project

represents a robust, rapid pay-back, high margin, ten year open pit mining and

heap leach project that works in the current gold price environment. As such,

Kaminak intends to move forward into mine permitting to support mine

construction, which is planned for mid-2018. The Company is well financed ($28

million as of September 30, 2015) to undertake an aggressive work-plan in 2016

to meet these objectives.

At a gold price of US$1,150/oz and an exchange rate of C$1.00 to US$0.78, the

Coffee Project base case estimate (the "Base Case") generates an after-tax net

present value (NPV) at a 5% discount rate of C$455 million and an internal rate

of return (IRR) of 37%. The proposed mine will operate over an initial ten year

mine-life with average annual gold production in excess of 200,000 ounces for

the first five years (excluding the initial 3 month ramp-up period), and

average annual life-of-mine gold production of 184,000 ounces. Initial capital

expenditure to fund construction and commissioning is estimated at C$317

million, with a life-of-mine capital cost of C$478 million (including C$60

million in closure costs). The all-in sustaining cash costs (as defined per

World Gold Council guidelines, less corporate G&A) is estimated to be US$550

per ounce of gold produced. The project is expected to have a significant

impact on Yukon's GDP, generating over $2 billion of gross revenue and

contributing 480 permanent, high paying jobs.

Eira Thomas, Kaminak President and CEO commented: "This feasibility study

firmly establishes the Coffee Project as one of the world's best undeveloped

gold projects by value and margin that works in the current gold-price

environment. The Coffee Project further benefits from being a simple, open pit,

heap leach mining opportunity, situated near infrastructure that delivers low

all in sustaining costs and pays back capital in under two years". She further

noted, "Kaminak feels privileged to be working in the pro-mining jurisdiction

of Yukon where we enjoy strong relations with all levels of government,

including our local First Nations, with whom we have worked alongside,

collaboratively since 2010."

Accompanying this news release is a video corporate presentation given by Eira

Thomas, Kaminak President & CEO, discussing the Feasibility Study and available

for viewing by clicking the following link:

https://www.webcaster4.com/Webcast/Page/1369/12606, or by visiting the Kaminak

website, www.kaminak.com.

Furthermore, Kaminak is hosting a live Q&A conference call on Thursday, January

7th at 11:00 a.m. Eastern time (8:00 a.m. Pacific time) with the Kaminak

Executive and Feasibility Study team. Participants may join the call by dialing

toll-free North America (866) 393-4306 or International (734) 385-2616 and

providing the company name (Kaminak Gold Corporation) to the operator. A

recorded playback of the call will be available two hours after the call's

completion until January 21st, 2016 by dialing (855) 859-2056 and entering the

conference ID#: 17824557, and on the Kaminak website.

Coffee Project Feasibility Study Overview

The Feasibility Study was initiated in July 2014 after the release of the

Preliminary Economic Assessment in June 2014 and commenced with infill

drilling, geotechnical investigations and other fieldwork to support the study.

The Feasibility Study was prepared and led by JDS Energy and Mining Inc. (JDS),

an established Yukon mine builder, in collaboration with a broad range of

industry leading consultants (see Contributors below),

Highlights (all currencies are reported in Canadian dollars unless otherwise

specified):

/T/

--  A pre-tax NPV5% and IRR of $762 million and 50% respectively;

--  An after-tax NPV5% and IRR of $455 million and 37% respectively;

--  A mine life of ten years with peak annual gold production of 228,000

ounces in project Year 4 and average, steady state, annual gold

production of 193,000 ounces (Years 1-9);

--  2,157,000 ounces of gold mined at head grade of 1.45 g/t Au (Probable

Mineral Reserve of 46.4 Mt at 1.45 g/t Au, containing 2.157 Moz Au.);

--  1,862,000 ounces of gold produced after average metallurgical gold

recoveries of 86.3%;

--  Total cash cost estimated at US$482 /oz Au (including royalties,

refining and transport) and an all-in sustaining cost (as defined by the

World Gold Council less Corporate G&A) estimated at US$550/oz Au,

generating an operating margin of US$600/oz or 52%;

--  Initial and sustaining capital costs, including contingency, for a 100%

owner-operated mine are estimated at $317 million and $161 million

(including $60 million in closure costs) respectively; and

--  A payback of 1.5 years pre-tax and 2.0 years after-tax after the

commencement of first commercial gold production;

/T/

The Feasibility Study proposes four open pits mined by conventional shovel and

truck methods at a nominal ore mining rate of 5 million tonnes per annum for

approximately ten years (with Year 1 being the first full year of commercial

gold production). A total of 312 Mt of material will be mined to produce 46.4

Mt of ore (strip ratio of 5.7 : 1). Run-of-mine ore will be crushed to a 2-inch

feed size and placed on a heap leach pad. Gold will be extracted from the

leachate by an Adsorption-Desorption-Recovery (ADR) carbon plant.

The site will be accessed principally from Dawson City, Yukon, by a 214 km

single-lane, gravel road with pullouts. The cost associated with upgrading

existing road and to construct approximately 37 km of new road along the

proposed 214 km route is estimated at approximately $25 million. Electrical

power will be generated on site by diesel-powered generators. Project

construction time from site mobilization to first commercial production of gold

is estimated to be 18 months, excluding access road construction which will

take approximately 9 months.

Comparison to the June 2014 Preliminary Economic Assessment (PEA)

The PEA presented a broadly similar operation to the Feasibility Study. The key

differences from the PEA presented in the Feasibility Study include:

/T/

--  Total ounces unchanged, with a shorter mine life, higher head grade,

lower reserve tonnage, and higher strip ratio;

--  Increased average total mining rate from approximately 64,000 tonnes per

day (tpd) to approximately 92,000 tpd;

--  A ridge-top heap leach pad with a shorter initial construction period

and lower capital cost per tonne of ore leached than the valley-fill

impounding heap leach facility proposed in the PEA;

--  An access road from Dawson City utilizing a high proportion of existing

gravel roads; and

--  Improved economics due to a shorter construction period, shorter mine

life and higher average gold grade.

/T/

Coffee Gold Feasibility Study Assumptions and Economic Results

The tables below summarize the various assumptions, operational parameters and

economic results of the Feasibility Study. All money values are nominal 2015

Canadian dollars unless otherwise stated.

The following assumptions were applied only in the derivation of the after-tax

valuation of the project:

/T/

--  Pre-development and sunk costs relating to the project (including but

not restricted to exploration and geotechnical investigations, resource

evaluation, fieldwork, metallurgical testwork, environmental and

baseline studies costs, technical studies);

--  Canadian Exploration Expense (CEE) and Canadian Development Expense

(CDE) tax pools with appropriate opening balances;

--  Applicable private and Yukon Crown royalties;

--  Federal and Yukon Territory tax rates; and

--  Specific capital cost class Capital Cost Allowance (CCA) rates over the

life of the project.

/T/

The following inputs were assumed for the project:

/T/

--  Gold Price of US$1,150 / oz.

--  Exchange Rate (US$ : C$) of 0.78

--  Diesel Price (delivered to site) of $0.85/ L for stationary equipment

and $0.89 / L for mobile equipment.

/T/

Feasibility Study Operating Parameters

/T/

----------------------------------------------------------------------------

Run of Mine Ore Delivered to Heap Leach (Mt)                            46.4

----------------------------------------------------------------------------

Run of Mine Ore Average Daily Production (Ktonnes,)                       14

----------------------------------------------------------------------------

Run of Mine Ore Average Annual Production (Mt)                           5.0

----------------------------------------------------------------------------

Average Diluted Ore Grade Delivered to Heap Leach (g/t Au)              1.45

----------------------------------------------------------------------------

Waste (Mt)                                                               265

----------------------------------------------------------------------------

Strip Ratio (Waste : Ore)                                              5.7:1

----------------------------------------------------------------------------

Gold Contained (Moz.)                                                  2.157

----------------------------------------------------------------------------

Gold Produced (Moz.)                                                   1.862

----------------------------------------------------------------------------

Gold Recovery (%)                                                       86.3

----------------------------------------------------------------------------

Average Gold Production Years 1 and 2 (Koz. per year), excluding 3

month ramp-up period                                                    194

----------------------------------------------------------------------------

Initial Capital Cost (including 8.9% contingency, excluding working

capital)                                                            $  317M

----------------------------------------------------------------------------

Sustaining Capital (including 9.3% contingency and closure costs)    $  161M

----------------------------------------------------------------------------

Life-of-mine Capital (including 9.1% contingency)                    $  478M

----------------------------------------------------------------------------

Unit Operating Costs (per tonne ore leached)

----------------------------------------------------------------------------

Mining $ 15.26

----------------------------------------------------------------------------

Processing $  4.97

----------------------------------------------------------------------------

Site Services $  0.97

----------------------------------------------------------------------------

General & Administrative $  2.89

----------------------------------------------------------------------------

Total Operating Costs (per tonne ore leached)                        $ 24.10

----------------------------------------------------------------------------

/T/

Economic Summary at US$1,150/oz. gold:

/T/

----------------------------------------------------------------------------

Total LOM Operating Cash Flow                                      $  1,614M

----------------------------------------------------------------------------

Total LOM Pre-Tax Free Cash Flow                                   $  1,113M

----------------------------------------------------------------------------

Average Annual Pre-Tax Free Cash Flow                              $    137M

----------------------------------------------------------------------------

LOM Income Taxes                                                   $    430M

----------------------------------------------------------------------------

Total LOM After-Tax Free Cash Flow                                 $    682M

----------------------------------------------------------------------------

Average Annual After-Tax Free Cash Flow                            $     93M

----------------------------------------------------------------------------

----------------------------------------------------------------------------

Discount Rate                                                             5%

----------------------------------------------------------------------------

Pre-Tax NPV                                                        $    762M

----------------------------------------------------------------------------

Pre-Tax IRR                                                              50%

----------------------------------------------------------------------------

Pre-Tax Payback (after start of commercial gold production)              1.5

years

----------------------------------------------------------------------------

After-Tax NPV                                                      $    455M

----------------------------------------------------------------------------

After-Tax IRR                                                            37%

----------------------------------------------------------------------------

After-Tax Payback (after start of commercial gold production)            2.0

years

----------------------------------------------------------------------------

/T/

All-in Cash Costs Including Sustaining Capex

/T/

----------------------------------------------------------------------------

On-Site Mining & Rehandle                                           $   707M

----------------------------------------------------------------------------

On-Site Processing                                                  $   230M

----------------------------------------------------------------------------

Surface and Infrastructure                                          $    45M

----------------------------------------------------------------------------

On-Site G&A                                                         $   134M

----------------------------------------------------------------------------

Refining                                                            $     2M

----------------------------------------------------------------------------

Royalties                                                           $    29M

----------------------------------------------------------------------------

Sustaining Capex                                                    $   100M

----------------------------------------------------------------------------

Closure                                                             $    60M

----------------------------------------------------------------------------

Total  $  1309M

----------------------------------------------------------------------------

All-in Cash + Sustaining Cost (C$/oz)(i)                                 705

----------------------------------------------------------------------------

All-in Cash + Sustaining Cost (US$/oz)(i)                                550

----------------------------------------------------------------------------

(i)All-in Sustaining Costs are presented as defined by the World Gold

Council ("WGC") less Corporate G&A. Calculated as (Refining Costs + 3rd

party royalties + operating costs + sustaining capital costs + closure

capital costs) / payable Au oz.

/T/

The Base Case set out above represents the value of the project from the known

resources. The Base Case assumes an owner-operated open pit mine, access to

site via a gravel road (approximately 295 days per annum) and diesel power

generation. Mining will take place 365 days per annum, and the heap leach pad

is planned to be operated 275 days per annum with no crushing and stacking of

ore occurring during January through March.

Sensitivities

/T/

----------------------------------------------------------------------------

Gold Price

US$/oz(ii)           $1,000  $1,100  $1,200  $1,250  $1,300  $1,400  $1,500

----------------------------------------------------------------------------

Pre-Tax NPV 5% C$M       504     676     848     934    1021    1193    1365

----------------------------------------------------------------------------

After-Tax NPV 5% C$M     295     402     509     562     616     722     828

----------------------------------------------------------------------------

Pre-Tax IRR              37%     46%     55%     59%     63%     71%     79%

----------------------------------------------------------------------------

After-Tax IRR            27%     34%     40%     43%     46%     52%     58%

----------------------------------------------------------------------------

Pre-Tax Payback

Years                   2.2     1.7     1.3     1.1     1.0     0.8     0.7

----------------------------------------------------------------------------

After-Tax Payback

Years                   2.7     2.2     1.8     1.6     1.5     1.2     1.1

----------------------------------------------------------------------------

(ii)At an exchange rate of C$1.00 to US$0.78

/T/

Opportunities to Enhance Value

Value-enhancing opportunities, such as contract mining, the acquisition of high

quality, second hand mining and ancillary equipment, crushing and process

plant, and accommodation quarters, will be further investigated as the project

moves into detailed engineering and procurement. As operations ramp up and a

greater understanding of the ore and heap leach facility performance evolve,

extending the duration of the crushing / stacking period, the period and

temperature of solution heating, and increasing the size of the ore feeding the

heap leach pad will be evaluated. Current oil prices indicate that diesel-only

power generation is the most cost effective power option. As diesel prices

increase, partial replacement of diesel with LNG will become more cost

effective.

There is potential for resource expansion along strike and to depth, with two

existing inferred gold resources (Kona North and Sumatra), and seven separate

drill discoveries in the early stages of evaluation located proximal to the

proposed mine site which contain more than 25 km of priority gold in soil

anomalies remaining to be systematically drill-tested.

Coffee Project Feasibility Study Major Components

Metallurgy

Ore samples for metallurgical testing were taken from bulk surface samples and

drill core composites. A total of 40 column tests were completed at both

ambient lab and simulated cold-climate temperatures, composited from 4500

samples, representing 29,608 kg from 123 drill holes and 8 trenches (at 1/2",

1", 2" and 6"crush sizes). In addition to column leach tests, metallurgical

testing included bottle roll leaching, flotation, column percolation and drain

down, multi-element head assay analyses, column-leach head and tail assay

screen analyses, ball mill work indices, crushing impact, and abrasion indices.

Fire assays for gold and silver were also conducted. Major conclusions from the

test program include:

/T/

--  Coffee Project ores generally leach very rapidly with low reagent

consumption

--  Ore agglomeration is not required;

--  Cyanide soluble assays from over 14,000 samples confirmed that cyanide

soluble recovery is a reliable method to determine gold recovery;

--  Differences in recovery observed between ambient lab temperature leach

tests and simulate cold climate leach tests were negligible;

--  Optimal gold recovery is achieved at an ore size of 80% passing a 50 mm

screen size (P80 of 50 mm; 2"); and

--  An average plant gold recovery of 86.3% was determined from

metallurgical testwork results and after the application of discount

factors (approx. 3%) to simulate plant operating conditions.

/T/

Geology and Mineralization

The Coffee Project area is underlain by a package of metamorphosed Paleozoic

rocks of the Yukon-Tanana Terrane comprising metasedimentary schist and

orthogneiss which form a west-northwest plunging antiform with gently dipping

limbs. The Palaeozoic sequence is intruded by Mid Cretaceous granite and

younger intermediate dikes.

These units are host to gold mineralization that is hydrothermal in origin and

structurally controlled, forming steeply dipping strike extensive mineralized

lodes in broadly E-W, N-S and NE -SW trending structural corridors.

Mineralization is associated with both polyphase brecciation and intense

sulphidation of mica-rich host rocks resulting in the formation of gold-bearing

arsenian pyrite. Extensive oxidation occurred due to ingress of meteoric water

downwards and along the structural deformation zones. This process resulted in

the oxidation of the arsenian pyrite along rims and cracks causing the release

of micron-scale free gold.

Mineral Resource Estimate

The Feasibility Study is based on an Indicated Mineral Resource estimate

undertaken by Robert Sim, P.Geo., an independent Qualified Person of SIM

Geological Inc. (Kaminak News Release, September 23rd, 2015). The mineral

resource estimate incorporated 1,682 diamond core and reverse circulation drill

holes completed from 2010 to 2015 for a total of 280,000 metres. This estimate

defines a total Indicated mineral resource of 63.7 million tonnes averaging

1.45 g/t Au for 2,968,000 ounces of contained gold, and a total Inferred

mineral resource of 52.4 million tonnes at an average grade of 1.31 g/t Au for

2,212,000 ounces of contained gold. These mineral resources are inclusive of

mineral reserves. Cut-off thresholds used in the resource estimate reflect the

projected metallurgical characteristics at various degrees of oxidation; 0.3

g/t Au for Oxide and Upper Transition, 0.4g/t Au for Middle Transition and at 1

g/t Au for Lower Transition and Sulphide types.

A series of resource block models, covering the limits of the four deposit

areas, were produced with gold grades estimated by ordinary kriging into model

blocks measuring 10m along strike, 2.5m across strike and 5m in the vertical

dimension. A capping strategy was employed to control the effects of

potentially anomalous, high-grade, sample data during block grade

interpolation. The ratio of cyanide soluble gold verses total gold content was

utilized to assign the various Oxide ore types in the block models. Mineral

resources are classified according to their proximity to sample locations and

are reported according to the CIM Definition Standards for Mineral Resources

and Mineral Reserves.

Mineral Reserve Estimate

The Mineral Reserve for the Property was estimated by Dino Pilotto, P.Eng., an

independent Qualified Person of JDS.

Cut-off grades, ranging from 0.27 to 1.40 g/t, were determined for each deposit

and ore type and based on appropriate mine design criteria and the adopted

mining method. The Mineral Reserves were estimated at a gold price of US$1,200

/ oz., and an exchange rate of C$1.00 to US$0.87, with the application of

dilution and recovery factors appropriate to an open pit mining method. The

factors applied to generate the pit shells were the best estimate of gold price

and exchange rate at the time the pit shells were generated. These factors were

not those utilized to generate the final project economics, but reserve

estimates using the lower gold price and exchange rate indicate that there were

not any material differences. Pit optimizations and analyses were conducted to

determine the optimal mining shells.

As input to the initial pit limit optimization and subsequent mine scheduling,

and in order to reflect the selectivity of the mining method chosen when

compared to the block model parameters, an external mining dilution was

calculated and applied for each of the various deposits. This external mining

dilution was based on a calculation of the number of waste blocks adjacent to

an ore block in the mineral inventory block model. Only blocks which were

contained within the resource classification of Indicated and above the given

gold cut-off grade were considered as ore blocks.

Probable Mineral Reserves are estimated at 46.4 Mt at 1.45 g/t Au, containing

2.157 Moz Au.

Mining

An open pit, shovel and truck mining method was applied to the Coffee Project

deposits. Mining is scheduled at the start of Year -1 in order to ensure the

delivery of a minimum of 3.5 Mt of ore to the heap leach pad prior to the first

winter shut-down, and to provide sufficient quantities of waste material to

facilitate construction activities. Mining of the deposit is planned to produce

a total of 46.4 Mt of ore and 265 Mt of waste (5.7:1 strip ratio) over a

ten-year project production life, including the initial year of pre-production.

Mining is planned to continue throughout the year. During the crusher and heap

leach shutdown in the first three months of each year uncrushed ore will be

stockpiled. The mine plan focuses on achieving consistent annual total material

production rates of between 34 and 40 Mt, while ensuring a steady-state ore

production rate of 5 Mt per annum.

The Supremo pit contributes approximately 71% (32.9 Mt) of the total ore and

commences mining in Year 2 of the project. It is planned to remain in

production to the end of mine life. The Latte (25%, 11.5 Mt), Double Double

(2%, 1.1 Mt) and Kona (2%, 0.9 Mt) pits will contribute early production and

will be completed by Year 3. This schedule endeavours to prioritize the early

production of higher value material where practical.

Open pit mining operations will use a fleet comprising industry-standard 16 m3

shovels, 12 m3 front-end loaders, 4 m3 excavators, and 144 t haul trucks. This

fleet will be supplemented by appropriately sized drills, graders, and dozers.

A 5 m bench height was selected for mining in ore and waste with overall 20 m

effective bench heights based on a quadruple-bench configuration. The handling

of the fine ore from the crusher to the heap leach pad is included in the open

pit scheduling and operating cost estimation.

Waste rock will be placed in three engineered waste rock facilities proximal to

the pits from which the waste is sourced. Some waste rock will be backfilled

into mined out pits at Latte, Supremo and Double Double in order to create

causeways and facilitate ore haulage routes to the crusher. Waste rock from the

Kona pit, due to its geochemical characteristics, will be placed in a temporary

waste rock facility adjacent to the pit during mining and then backfilled into

the mined-out Kona pit.

Processing

The process includes a two-stage crushing plant feeding a heap leach operation.

Gold is extracted by an adsorption-desorption-recovery (ADR) carbon plant. The

process is based on a heap leach processing rate of 5.0 million dry tonnes per

year at an average feed grade of 1.45 g/t and an anticipated overall recovery

of 86.3%. The crushing rate will be approximately 18,200 tpd over the 9-month

operating period. The process plant, located in close proximity and at a lower

elevation to the heap leach facility, minimizes pumping by allowing gravity

feed of pregnant solution.

Crushed ore will be trucked to the heap leach facility and spread by dozer.

Crushing and heap leach stacking will be suspended between January and March in

order to maintain the thermal integrity of the heap. The heap leach facility

will operate all year with the solution heated as necessary to maintain thermal

integrity. The facility will be stacked progressively, in well-defined stages,

from the east to the west.

Ore leaching will commence in the third quarter of Year -1. At the end of the

fourth quarter approximately 3.5 Mt of ore will be available for leaching. Gold

recovery will continue through the first quarter of Year 1 and ore crushing and

stacking recommence in Q2 of Year 1. Progressive closure of the heap leach pad

will commence in Year 4 as depleted areas of the facility become available for

decommissioning, covering, and reclamation. The seasonal placement of synthetic

geomembrane covers will aid in maintaining the thermal profile and controlling

precipitation run-off.

Process water for leaching will be sourced from site contact water exclusively,

except at the start of operations when a small amount of water may be sourced

from surrounding creeks. Event and rainwater storage ponds will be constructed

between the heap leach pad and the process plant.

Infrastructure and Project Execution

The following key infrastructure will support the mine and process facilities:

/T/

--  A 214 km access road from the Klondike Highway turn-off 15km east of

Dawson City, utilizing and upgrading existing roads where possible and

requiring only approximately 37 km of new road construction to connect

portions of existing isolated road. The Stewart and Yukon rivers will becrossed

by barge in summer and ice roads in winter. During the fall

freezing and spring breakup periods, each approximately 6 and 4 weeks

respectively, the site will not be accessible by road. On-site storage

for consumables accommodates a period of up to ten weeks without site

access;

--  A 1,200 m gravel airstrip;

--  Truck work shop, warehouse, administration and camp accommodation;

--  Bulk explosives storage and magazines;

--  Power plant (installed capacity of 9 MW), initially diesel-fired but

capable of partial LNG substitution;

--  Diesel bulk storage tanks (for power generation, mining equipment and

ANFO) with a ten week capacity;

--  Potable water, fire water and sewage treatment systems; and

--  Potable water to be trucked from the Yukon River;

/T/

Project operational time-line

/T/

----------------------------------------------------------------------------

Project   Project Activities

Year

----------------------------------------------------------------------------

Year -3   Road construction starts.

----------------------------------------------------------------------------

Year -2   Q1-Q2: Delivery of equipment, infrastructure, and consumables.

Q3-Q4: Infrastructure earthworks, camp and facility construction.

Pre-stripping starts.

----------------------------------------------------------------------------

Year -1   Q1: Crusher and process plant construction begins. Mining begins.

Q3: Heap leach pad loading commences; leaching begins late Q3.

Q4: First commercial gold production begins.

----------------------------------------------------------------------------

Year 1    First full year of operations and commercial gold production.

----------------------------------------------------------------------------

/T/

Total manpower complements during construction and operations are estimated to

peak at 480 and 435, respectively. During construction a peak, on-site labour

force of 250 is anticipated. The peak, on-site operational labour complement is

anticipated to be 225. Personnel will generally operate on a 2 week on - 2 week

off shift rotation on a fly-in - fly-out basis.

Contributors

The Feasibility Study was undertaken and led by JDS Energy and Mining Inc.,

under the direction of project lead, Angus Christie, PhD., (JDS) with oversight

and contributions from James Scott, MSc., P.Geo (Advanced Projects Manager,

Kaminak) and Fred Lightner, P.E. (Director of Mining, Kaminak).

The following organizations contributed to the Feasibility Study:

/T/

----------------------------------------------------------------------------

Discipline                                 Responsible Consultant

----------------------------------------------------------------------------

Project management, mining, reserve        JDS Energy and Mining Inc.

estimation, process engineering,

infrastructure, and financial modelling

----------------------------------------------------------------------------

Geological modelling and resource          Sim Geological Inc.

estimation

----------------------------------------------------------------------------

Metallurgical testwork                     Kappes, Cassiday & Associates

----------------------------------------------------------------------------

Crushing and abrasion testwork             ALS Canada

----------------------------------------------------------------------------

Geomechanical and geotechnical             SRK Consulting (U.S.), Inc.

----------------------------------------------------------------------------

Heap leach facility                        RRD International Corp. and The

Mines Group Inc.

----------------------------------------------------------------------------

Water balance, geochemistry, and source    Lorax Environmental Services Ltd.

terms

----------------------------------------------------------------------------

Water management                           SRK Consulting (Canada) Inc.

----------------------------------------------------------------------------

Access road                                JDS Energy and Mining Inc. and

Onsite Engineering Ltd.

----------------------------------------------------------------------------

Permafrost                                 Tetra Tech EBA

----------------------------------------------------------------------------

/T/

Next Steps

With the results of the Feasibility Study now in hand, Kaminak will begin

advancing the Project through the permitting process in Yukon. The Project will

be subject to an environmental and socio-economic assessment under the Yukon

Environmental and Socio-economic Assessment Act (YESAA), administered by the

Yukon Environmental and Socio-Economic Assessment Board (YESAB). Kaminak has

assembled a team of experts to assist the Company in the development of a

Project Proposal which the Company plans to submit to YESAB in Q3 2016. Once

through YESAB's adequacy review, the Company will submit applications for a

Quartz Mining Licence and Water Licence, the two major licences required for

the construction and operation of a mine.

Kaminak's disclosure of a technical or scientific nature in this press release

has been reviewed and approved by James Scott, MSc., P.Geo, Advanced Projects

Manager for Kaminak Gold Corporation, who serves as a Qualified Person under

the definition of NI 43-101.

On behalf of the Board of Directors of Kaminak

Eira Thomas, President and CEO

Kaminak Gold Corporation

For further information about Kaminak Gold Corporation or this news release,

please visit our website at www.kaminak.com.

Neither the TSX Venture Exchange nor its Regulation Services Provider (as that

term is defined in the policies of the TSX Venture Exchange) accepts

responsibility for the adequacy or accuracy of this release.

The technical information in this press release for the Coffee Project is based

on the following technical report:

Technical Report and Feasibility Study for the Coffee Project, 2016, Gord

Doerksen, P.Eng., Angus Christie, Dino Pilotto, P.Eng., Kelly McLeod, P.Eng.,

Trevor Herd, Victoria Viveash, Rob Sim, P.Geo., Mike Levy, P.E., Anthony Crews,

P.E., David Flather, Tom Sharp, P.Eng.

The authors of the Feasibility Study are independent of the Company and are

qualified persons for the purposes of NI 43-101.

The technical report will be filed under the Company's profile on or before

February 19th, 2016 on SEDAR at www.sedar.com.

Robert Sim, P.Geo., Consultant to the Company and a Qualified Person as defined

by NI 43-101 has reviewed and approved the contents of this news release

related to the mineral resource estimate. The mineral resource estimate

referenced in this press release was prepared in September 2015 by Robert Sim,

P.Geo., an independent Qualified Person as defined by NI 43-101.

Dino Pilotto, P.Eng., of JDS Energy and Mining Inc., a Consultant to the

Company and a Qualified Person as defined by NI 43-101 has reviewed and

approved the contents of this news release related to the mineral reserve

estimate. The mineral reserve estimate referenced in this press release was

prepared in December 2015 by Dino Pilotto, P.Eng., an independent Qualified

Person as defined by NI 43-101.

Cautionary Note Mineral Reserves and Mineral Resources:

The terms "Mineral Resource", "Mineral Reserve", "Probable Mineral Reserve",

"Indicated Mineral Resource", and "Inferred Mineral Resource" used in this news

release are Canadian mining terms as defined in accordance with NI 43-101 under

the guidelines set out in the CIM Standards. Mineral Resources which are not

Mineral Reserves do not have demonstrated economic viability.

This news release uses the terms "Inferred Resources" and "Indicated

Resources", which have a great amount of uncertainty as to their existence, and

great uncertainty as to their economic and legal feasibility. It cannot be

assumed that all or any part of an Inferred and/or Indicated Mineral Resource

will ever be upgraded to a higher category. Under Canadian rules, estimates of

Inferred Resources may not form the basis of feasibility or other economic

studies. Kaminak advises U.S. investors that while this term is recognized and

required by Canadian regulations, the U.S. Securities and Exchange Commission

does not recognize it. U.S. investors are cautioned not to assume that part or

all of an Inferred and Indicated resource exists, or is economically or legally

minable.

Caution Concerning Forward-Looking Statements

Certain disclosures in this release, including statements relating to

management's assessment of plans, projects and intentions with respect to the

further development of the Coffee Project and future exploration programs; the

projected price of gold and the associated viability of the Coffee Project; the

intent of Kaminak to move forward with the permitting phase of the Coffee

Project; the estimated after-tax NPV and IRR associated with the Coffee

Project; the anticipated mine-life, annual gold production, capital costs, and

all-in sustaining costs associated with the Coffee Project; the expected impact

of the Coffee Project on Yukon's GDP; the performance of the Coffee Project

with respect to gross revenue, creation of jobs, and pay back capital; the

estimated timeframe associated with project construction; the Company's

intention to further examine value-enhancing opportunities such as contract

mining and second hand equipment, among others; the Company's intention to

submit a project proposal to the YESAB in Q3 2016; and the Company's intention

to apply for and ability to obtain required permits in a timely manner

constitute forward-looking statements that are subject to numerous risks,

uncertainties and other factors relating to Kaminak's operations as a mineral

exploration company that may cause future results to differ materially from

those expressed or implied in such forward-looking statements. Generally, these

forward-looking statements can be identified by the use of forward-looking

terminology such as "expects", "anticipates", "believes", "intends",

"estimates", "potential", "possible" and similar expressions, or statements

that events, conditions or results "will", "may", "could" or "should" occur or

be achieved.

Forward-looking statements are based on the assumptions, opinions and estimates

of management as of the date such statements are made, and they are subject to

a number of known and unknown risks and uncertainties which may cause the

actual results, performance or achievements of the Company to be materially

different from any future results, performance or achievement expressed or

implied by such forward-looking statements. In particular, such risks include

fluctuations in commodity prices and currency exchange rates; mineral resource

and reserve estimates; operational risks associated with mineral exploration

and production; title matters; possible variations in resources, grades, or

recovery rates; uncertainty of estimates of capital and operating costs;

changes in project parameters as plans continue to be refined; delays in

obtaining government approvals or licences; failure of equipment or processes

to operate as anticipated; uncertainty surrounding the ability to obtain

financing and to market the products; accidents or labour disputes, and other

risks and uncertainties disclosed under the heading "Risk Factors' in the

Company's most recent Annual Information Form, and in the forthcoming Coffee

Project technical report, both of which will be available for download at

http://www.sedar.com.

Readers are cautioned not to place undue reliance on forward-looking

statements. Except as required by law, Kaminak expressly disclaims any

intention or obligation to update or revise any forward-looking statements,

whether as a result of new information, future events, or otherwise.

- 30 -

CONTACT INFO:

Kaminak Gold Corporation

Tony Reda

Vice-President of Corporate Development

toll free: 1.844.646.4520 / direct: 604.646.4534

info@kaminak.com

www.kaminak.com

INDUSTRY: Manufacturing and Production - Mining and Metals

SUBJECT: MEX - MINING EXPLORATION UPDATE

----------------------------------------------

Suite 900, 25 York Street, Toronto, ON M5J 2V5

Toll Free:888-299-0338

Phone: 416-362-0885

info@marketwired.com

----------------------------------------------

If you no longer want to receive

announcements from us, please do not reply to this e-mail.

Instead simply click

http://marketwire.com/mw/fmu.jsp?id=553696941