Source	Business Wire

Date	02/11/2019

Time	08:02:42 AM

Company	 BLACK DRAGON GOLD CORP.

Title	[EXTERNAL] Black Dragon Gold Corp.: Positive Preliminary Economic

Assessment f...

CDNX Symbol: BDG

Press Release

Feb. 11, 2019 16:02 UTC

Black Dragon Gold Corp.: Positive Preliminary Economic Assessment for  Salave

Underground

NEWS RELEASE SUBMITTED BY BUSINESS WIRE

FOR: Black Dragon Gold Corp.

TSX VENTURE SYMBOL: BDG

VANCOUVER, British Columbia--(BUSINESS WIRE)-- Black Dragon Gold Corp.

(ASX/TSX-V: BDG) ("Black Dragon" or the  "Company") is pleased to announce the

positive results of  the Preliminary Economic Assessment ("PEA") completed on

its 100% owned  Salave Gold Project ("Salave" or "Project") located in Asturias

in  northern Spain. The PEA is based on the recently completed Mineral

Resource Estimate completed by CSA Global (See October 25, 2018 News  Release).

All figures are in United States Dollars unless otherwise  stated.

This press release features multimedia. View the full release here:

https://www.businesswire.com/news/home/20190211005527/en/

The PEA demonstrates robust economics for an underground mining  operation with

a 14-year mine life.

Paul Cronin, Managing Director and CEO commented,

"The completion of the PEA is a major milestone on the path to  development of

the Salave Project and the metrics support our belief  that Salave can

potentially generate strong returns for shareholders. It  forms the first step

in our permitting process, presenting a new  optimised process on a zero-

discharge basis that minimises the visual  and surface impact of the project.

The robust results of this PEA underline the potential economic  viability of

the current Salave resource to be mined over an initial 14  year mine life, and

our successful drilling campaign last year indicates  strong potential for

growth in mine life at Salave.

This study supports that Salave can produce over 1.1Moz (560 kt of  concentrate

averaging over 59 g/t Au), providing a number of marketing  options for export

and refining, minimising the need for additional  plant and equipment, and

hence reducing the Projects footprint. The  relatively low upfront capex also

opens alternative financing  opportunities which will ensure that both

shareholders and the local  community benefit from the success of this

Project."

KEY PEA OUTCOMES

Pre-Tax NPV at 5% discount rate: US$ 296.2 million.

After-Tax NPV: US$ 230.0 million

Pre-Tax Internal Rate of Return ("IRR"): 28%

After-Tax Internal Rate of Return ("IRR"): 25%

After-Tax Payback: 3.8 years

Pre-Production Capital Cost, including contingency: US$ 95.3 million

Life of Mine ("LOM") Sustaining Capital Cost: US$19.3 million

Estimated Average LOM Total Cash Cost: $729/ounce (oz) Au

Estimated Average LOM All-In Sustaining Costs ("AISC"): $752/oz Au

CAUTIONARY STATEMENT

The PEA is a preliminary technical and economic study of the potential

viability of the Salave Gold Project. It is preliminary in nature and  includes

Inferred Mineral Resources that are considered too  speculative geologically to

have economic considerations applied to  them that would enable them to be

categorized as Mineral Reserves.  There can be no assurance, and there is no

certainty, that the  preliminary economic assessment contained therein will be

realised.  Further exploration and evaluation work and appropriate studies are

required before Black Dragon will be in a position to estimate any Ore

Reserves or to provide any assurance of an economic development case.

The production target and forecast financial information referred to  in this

PEA are comprised of Measured and Indicated Mineral Resources  (73%) and

Inferred Mineral Resources (27%). The proportion of Inferred  Mineral Resources

is not determinative of the project viability and  does not feature as a

significant proportion early in the mine plan.

Metallurgical recoveries have been based on test work data and costs  have been

estimated by independent consultants generally from budget  quotations,

factored estimates or cost data from similar  operations/projects. Cost

estimate accuracy for the PEA is in the  order of +/-35%.

The PEA is based on the material assumptions outlined herein and in  the

report. These include assumptions about the availability of  funding. While BDG

considers all of the material assumptions to be  based on reasonable grounds,

there is no certainty that they will  prove to be correct or that the range of

outcomes indicated by the PEA  will be achieved. To achieve the range of

outcomes indicated in the  PEA, among other things, funding of in the order of

US$100 million  will likely be required. Investors should note that there is

no  certainty that Black Dragon will be able to raise that amount of  funding

when needed.

It is also likely that such funding may only be available on terms  that may be

dilutive to or otherwise affect the value of BDGs  existing shares. It is also

possible that BDG could pursue other  value realisation strategies such as a

sale, partial sale or joint  venture of the project. If it does, this could

materially reduce BDGs  proportionate ownership of the project. Given the

uncertainties  involved, investors should not make any investment decisions

based  solely on the results of the PEA.

NEXT STEPS

Submission of the Project Description for the Environmental and Social  Impact

Assessment ("ESIA") in February 2019;

Additional geophysics over the entire Investigation Permit at Salave   April

2019;

Issuance of the ESIA Terms of Reference in June 2019;

Soil Geochemistry testing on potential drill targets  June 2019;

Pre-Feasibility Study  October 2019.

PEA KEY ASSUMPTIONS AND INPUTS

Assumed gold price: US$1,250/oz

Exchange Rate of $1.15 /

Life of Mine: 14-years

Main Underground Mining Method: Vertical Retreat & Sub-Level Stoping

Average Diluted Head Grade: 3.87 g/t Au

Total Underground Dilution: 43%

LOM Plant Throughput 9.19 Mt

Access Ramp Gradient of 15% at a 5.0m x 5.5m profile

Mineralised Zone Development at a 4.0m x 4.5m profile

Average Mining and Processing throughput: 2,000 tonnes per day ("tpd")

Flotation Plant Recoveries: 97%

Average Annual Production (LOM): 79,200 oz Au in concentrate at an  average

grade of 59.7 g/t Au

LOM recovered gold in concentrate production: 1,108,420 oz;

Refining and Processing Charges: US$368/t concentrate or US$188/oz Au

Table 1 - PEA Summary Parameters

Input 	 	Unit

Physical Parameters

Total Mineralised Material Tonnes Mined (LOM) 		Mt

9.19

Average Annual Throughput (LOM) 		ktpa

656.3

Head Grade 		Au g/t		3.87

Gold Recovery to Concentrate 		% 		97%

Mine Life 		years		14

Gold Grade of Concentrate 		Au g/t		59.71

Total Concentrate Produced 		kt		560.5

Total Ounces in Concentrate 		koz		1,108.4

Average Annual Production (LOM) 		koz		79.2

Cost Parameters

Mining Costs 		US$/t		40.68

Processing Costs 		US$/t		14.00

General & Administrative 		US$/t		2.71

Total Costs 		US$/t		57.39

Pre-Production Capital Costs

Mine Development & Infrastructure 		US$m

29.7

Mining Equipment 		US$m		11.2

Tailings 		US$m		1.3

Process Plant 		US$m		28.3

Owners Costs & EPCM 		US$m		12.5

Contingency (15%) 		US$m		12.4

Total Pre-Production Capital 		US$m		95.3

Sustaining Capital 		US$m		19.3

LOM Cash Costs 		US$/oz		729.15

LOM AISC 		US$/oz		752.80

MINERAL RESOURCE ESTIMATE

An updated NI 43-101 Mineral Resource Estimate, effective 22 October  2018 is

included in this PEA and has been filed on SEDAR and the ASX  market

announcements platform (See October 25, 2018 News Release).

Tonnes	 			Au

Category		Mt		g/t	 	koz

Measured 		1.03 		5.59 		185

Indicated 		7.18 		4.43

1,023

Measured & Indicated 		8.21 		4.58

1,208

Inferred 		3.12 		3.47 		348

Notes:1. Rounding may cause apparent discrepancies2.  Resource Estimate

conducted by CSA Global of Perth Australia ("CSA")  with an effective date of

October 22, 2018. Classification of the MRE  was completed based on the

guidelines presented by Canadian Institute  for Mining (CIM, May 2014), adopted

for Technical Reports which adhere  to the regulations defined in Canadian NI

43-101. The Mineral Resource  Estimate was also prepared in accordance with the

Australasian Code for  Reporting of Exploration Results, Mineral Resources and

Ore Reserves,  2012 edition ("2012 JORC Code").3. The Mineral Resource

Estimate was first announced on 25 October 2018. Black Dragon confirms  that it

is not aware of any new information or data that materially  affects the

information in the previous announcement and that all  material assumptions and

technical parameters underpinning the Mineral  Resource Estimate continue to

apply and have not materially changed.4.  A cut-off grade of 2 g/t Au has been

applied when reporting the Mineral  Resource Estimate.5. Mineral Resources that

are not Mineral  Reserves do not have demonstrated economic viability but do

have  reasonable prospects for eventual economic extraction.6. The  quantity

and grade of reported Inferred Resources in this estimation are  conceptual in

nature and there has been insufficient exploration to  define these Inferred

Resources as an Indicated or Measured Resource. It  is uncertain if further

exploration will result in upgrading them to an  Indicated or Measured Resource

category, although it is reasonably  expected that the majority of the Inferred

Resources could be upgraded  to Indicated Mineral Resources with further

exploration.7. The  Mineral Resource Estimate underpinning the production

targets in this  announcement was prepared by a Competent Person under the 2012

JORC Code.8.  The title of the report is "Salave Gold Project Mineral Resource

Update  for Black Dragon Gold Corp.", with an effective date of October 22,

2018, and it was authored by Ian Stockton, B.Sc (Geol)., MAusIMM, FAIG,  Dmitry

Pertel, MSc (Geol), MAIF, GAA, and Galen White, B.Sc, FAusIMM,  FGS.

POTENTIALLY EXTRACTABLE PORTION OF  MINERALISATION FOR MINE PLANNING

The mine plan supported by the PEA demonstrates that approximately 81.1%  of

the total 2018 updated Mineral Resource tonnage is amenable to  underground

extraction. For purposes of mine planning, the potentially  extractable portion

of the Mineral Resources are comprised of 9.19  million tonnes at a diluted

grade of 3.87 g/t Au, containing just over  1.1 million ounces of gold. The

mineralised material modelled to be  mined in the PEA contains Mineral

Resources classified in the Inferred  category (28%) that are too speculative

geologically to have economic  considerations applied that would enable them to

be categorized as  Mineral Reserves. These Inferred Resources will require

further  exploration and definition to meet the criteria to be classified as

Indicated or Measured Mineral Resources before being considered for  conversion

to Mineral Reserves at the next level of detailed economic  study.

MINE PLAN

Given environmental and community considerations, the PEA has only  evaluated

underground mining operations. The primary mining method  selected for detailed

analysis in this study was the vertical retreat  mining ("VRM"). Sub-level

stoping was considered as a secondary method  applicable to specific vertical

thin geometries (<15m length). Rock and  paste fill will be used as backfill to

maximize mining recovery.

The mine design was based on basic economic assumptions to create  mineable

stope outlines. A value of 2 g/t was assumed as mine cut-off  grade. Mining

dilution and mineralised material loss factors were also  applied to each

mining shape to reflect the selected mining method.

The mine production rate targets a 0.70 Mtpa of RoM. A conceptual mine  layout

was designed including stopes and development as illustrated in  Figure 1, with

60m levels and 3 x 20m sub-levels. The total mineralised  material from stopes,

drives and sill pillar recovery (50%) will total  9.2Mt at 3.87 g/t Au.

A long term mine schedule was created integrating stopes and development  as

shown in Figures 2 and 3. Mineralised zones were sequenced to  prioritise

higher grades at lower operating costs.

MINERAL PROCESSING

In order to minimise potential social and environmental issues,  processing of

Salave mineralised material has been limited to crushing,  grinding and

flotation, with concentrates exported via local ports. Mine  feed will be

crushed on surface at a rate of 0.7 Mtpa, and then be  processed via

conventional SAG and ball milling followed by sulphide  flotation and

thickening.

The run-of-mine will feed a primary jaw crusher with a capacity of 400  t/h

with a physical availability of 70% with design factor of 20%.

From an intermediate stockpile, the coarse material will feed the mill  circuit

which consists of a conventional SAG and ball mill configuration  working in

close circuit with the cyclones.

The flotation circuit consists of a number of cells of 300-400 m3  capacity

(9-10 cells of 40 m3 each) with two conditioning  tanks for pH stabilisation

and reagents.

The final stage consists of tailings thickening to minimise the fresh  water

consumption and to re-cycle process water. A paste thickener  will achieve a

product of 70-75% of solids.

Based on flotation test work conducted to date, it is assumed that 97%  of the

gold head grade will be recovered in the flotation concentrate  that will be

thickened, filtered and bagged for shipping to customers.

INFRASTRUCTURE AND TAILINGS

Power to the project is available from Tapia, which is linked to the  Asturias

main distribution grid, and an existing network of power lines  enter the

property that are connected to the Spanish national  transmission grid. Water

for both domestic and plant usage can be  sourced from wells, the Porcia River

(2.5km east of the property) or the  reticulated water supply that is currently

in place near the plant  location.

A Tailings Management Facility ("TMF") will be constructed at surface  for

temporary storage of plant tailings. The paste and backfill of the  mine will

minimise the amount of tailings storage at surface, and  various options for

complete tailings disposal are being evaluated. The  TMF design will involve

water recovery in the processing plant and  transportation to geo-membrane

lined facility eliminating any risk for  potential surface and ground water

contamination.

Surface facilities to support the Salave Project will include an

administration and engineering building, security, warehouse, fuel and

explosive storage, fire protection, maintenance shops with a site design  to

accommodate for 50 full time staff.

CAPITAL COSTS AND SENSITIVITIES

Input (US$M) 	 	Pre-Production

Sustaining	 	LOM

Development 		29.7 		15.7

45.3

Equipment & Infrastructure 		11.2

3.6 		14.8

Tailings 		1.3 		0.0 		1.3

Process Plant 		28.3 		0.0

28.3

Owner Costs & EPCM 		12.5 		0.0

12.5

Contingency (15%) 		12.4 		0.0

12.4

Total Capex 		95.3		19.3

114.6

Sensitivity Analysis

After-Tax NPV (US$M)

% relative to the

Base Case

Parameter		-20% 	 			Base

Case 	 			+20% 		-20%

Base Case 	 	+20%

Gold price 		95.8

230.0 				361.0 		-58%

0% 		57%

Processing costs 		244.5

230.0 				163.4

6% 				0% 		-29%

Mining costs 		296.1

230.0 				192.3

29% 				0% 		-16%

Capex 		245.3

230.0 				214.6

7% 				0% 		-7%

Gold Price Sensitivities

Macro Parameters 				Unit

-20%

Base Case	 			+20%

Gold Price

US$/oz

1,000

1,250 				1,500

Pre-Tax

NPV5% 				US$M

122.2

296.2 				469.2

IRR 				%

16%

28% 				40%

Post-Tax	NPV5%

US$M

95.8

230.0 				361.0

IRR 				%

14%

25% 				36%

Payback 				Years

6.3

3.8 				2.6

PROJECT FUNDING

The Board of BDG believes there is a reasonable basis to assume the  necessary

funding for the Salave Gold Project will be obtained for the  following

reasons:

The Company has been able to raise funding for its exploration over  the past

years in order to progress its project. In the last two years  BDG has raised

over $14.5 million via equity placements. These raises  indicate a clear base

of support from new and existing shareholders  and third-party investors. The

Company considers it will be able to  raise funding for the next stage of the

Project, which will advance  the Project to the completion of a detailed

Feasibility Study.

The positive outcomes delivered by the PEA give confidence to the  Board in the

ability of the Company to fund the development capital  through conventional

debt and equity financing. A mix of debt and  equity is the most likely funding

model so 100% of the capital  expenditure will not need to be borrowed. The

Board has a strong  financing track record in funding start up mining

operations, and in  their view, it is reasonably expected that when the project

parameters  in this PEA are met, that funding will be able to be arranged.

Notwithstanding this, the normal risks for the raising of capital will  apply

to the Company, such as the state of equity capital and debt  markets, the

results of the Feasibility Study and the price of gold.

The Company believes that its funding opportunities will be improved  at the

completion of the Feasibility Study as a result of:

(i) confidence in the possibility to increase the Mineral Resource  Estimate

that would serve to improve the mine life of the Project;

(ii) confirmation of earlier metallurgical test work to support,  optimise and

potentially improve concentrate grades; and

(iii) finalisation of further engineering studies to improve the  accuracy of

the assessed capital and operating costs

(iv) offtake contracts for concentrates to improve revenue and  treatment

charge assumptions

The funding models being considered will depend on the outcomes of the

Feasibility Study, but as set out above will likely be conventional  debt and

equity financing, but may include convertible notes, gold  streaming,

prepayment of royalties and other options for projects of a  similar nature.

The raising of equity by the Company may be dilutive to existing  shareholders,

but that will depend on the price at which the then  funding is completed.

Where the market capitalisation of the Company  is low as against the amount of

equity that is required to be raised  at the time, there is a high likelihood

that shareholders will be  substantially diluted. This is to be balanced

against the reasonable  expectation of the Company that as the Project becomes

more advanced,  the value of the Company is more likely to increase, resulting

in the  actual dilution to existing shareholders being less. The reality is

that in this case, although the percentage holding of each shareholder  will be

reduced, the value of that holding will be assessed against a  Company that is

anticipated to have a higher market capitalisation at  the time of the raising.

PEA KEY RECOMMENDATIONS

CRS Ingenieria ("CRS"), in Madrid were the principal authors of the PEA  and

have made the following recommendations for further evaluation that  may

improve the economics of the project:

The mineralisation style indicates that both vertical retreat mining  (VRM) and

room and pillar (RP) are applicable to Salave. For this  study, a combination

of VRM and SLS was selected, configured with  60m-height panels, however, CRS

recommends assessing the benefits of  RP for individual panels.

The production rate of Salave used for the PEA was 0.70 Mtpa. While  this

production capacity is optimal under current assumptions of  mining method and

cut-off grade, CRS suggests the evaluation of  alternative cut-off strategies

that may lead to review the production  rate.

Evaluate mining methods by panel and create integrated layouts.

Develop detailed geotechnical studies to estimate stope and room  dimensions

and modifying factors such as dilution and mineralised  material loss.

Develop a volume balance of waste and paste over the LoM sequence.

Further investigate low grade materials by additional drilling to  verify

geological continuity.

Sill pillars may be recovered if specific technical and economic  studies

demonstrate that economic extraction could reasonably be  justified under

realistic conditions. CRS recommends the completion of  detailed geotechnical

studies to confirm the viability of sill pillar  recovery.

Complete an economic study considering obtain free gold through  panning before

shipment the product.

Develop a detailed market study to identify potential clients for the  Salave

gold concentrates.

QUALIFIED PERSONS AND COMPETENT PERSONS STATEMENT

The information in this announcement that relates to the PEA for the  Salave

Gold Project is based on and fairly represents information and  supporting

documentation prepared by CRS Ingenieria and CSA Global.  Paulo Laymen (P.Eng.,

M.AusIMM., B.Eng., M.Eng.) of CRS Ingenieria  supervised the preparation of the

PEA, is independent of the Company and  a qualified person as defined by

National Instrument 43-101 and has  reviewed and approved the technical

disclosure reported herein. Dmitry  Pertel (P.Geo., MSc (Geol), MAIF, GAA) and

Belinda van Lente (P.Geo.) of  CSA Global were responsible for the Mineral

Resource Estimate and are  independent of the Company and qualified persons as

defined by National  Instrument 43-101 and have reviewed and approved the

technical  disclosure reported herein.

The NI 43-101 Technical Report will be fined on SEDAR within 45 days of  this

release.

BLACK DRAGON GOLD CORP.1000 Cathedral Place925 West Georgia  StreetVancouver,

BC V6C 3L2, Canada,T- +44 20 79934077 F-  +44 20

71128814info@blackdragongold.comwww.blackdragongold.com

ABOUT BLACK DRAGON GOLD

Black Dragon Gold "BDG" is the 100% owner of one of the largest  undeveloped

gold projects in Europe, the Salave project. Salave is  situated in the North

of Spain in the province of Asturias. The Salave  project has an updated

combined Measured and Indicated Mineral Resource  of 8.21 million tonnes

grading 4.58 g/t Au, containing 1.21 million  ounces of gold, plus Inferred

resources totalling 3.12 million tonnes  grading 3.47 g/t Au, containing

348,000 ounces of gold.

A full technical report summarizing the Mineral Resource estimate  completed by

CSA Global is available on the companys web site and  posted on SEDAR. In

addition to the current Mineral Resource, historical  exploration work suggests

there is the potential for additional  mineralisation within Black Dragons

landholdings.

FORWARD LOOKING STATEMENTS

This news release contains forward-looking statements that are based on  the

Corporation's current expectations and estimates. Forward-looking  statements

are frequently characterized by words such as "plan",  "expect", "project",

"intend", "believe", "anticipate", "estimate",  "suggest", "indicate" and other

similar words or statements that certain  events or conditions "may" or "will"

occur. Such forward-looking  statements involve known and unknown risks,

uncertainties and other  factors that could cause actual events or results to

differ materially  from estimated or anticipated events or results implied or

expressed in  such forward-looking statements. Such factors include, among

others: the  actual results of current planned exploration activities; changes

in  project parameters as plans to continue to be refined; possible  variations

in recovered material grade or recovery rates; accidents,  labor disputes and

other risks of the mining industry; delays or any  inability in obtaining

governmental approvals or financing; and  fluctuations in metal prices. There

may be other factors that cause  actions, events or results not to be as

anticipated, estimated or  intended. Any forward-looking statement speaks only

as of the date on  which it is made and, except as may be required by

applicable securities  laws, the Corporation disclaims any intent or obligation

to update any  forward-looking statement, whether as a result of new

information,  future events or results or otherwise. Forward-looking statements

are  not guarantees of future performance and accordingly undue reliance

should not be put on such statements due to the inherent uncertainty  therein.

Neither the TSX Venture Exchange, nor its Regulation Services Providers  (as

that term is defined in the policies of the TSX Venture Exchange)  accepts

responsibility for the adequacy or accuracy of this news release.

All figures are rounded to reflect the relative accuracy of the news  release.

[IMAGE]

View source version on businesswire.com:

https://www.businesswire.com/news/home/20190211005527/en/

Contacts

Paul CroninCEO & Managing DirectorP: +44 20  79934077E:

paul.cronin@blackdragongold.com

Source: Black Dragon Gold Corp.

Feb. 11, 2019 16:02 UTC

Black Dragon Gold Corp.: Positive Preliminary Economic Assessment for  Salave

Underground

NEWS RELEASE SUBMITTED BY BUSINESS WIRE

FOR: Black Dragon Gold Corp.

TSX VENTURE SYMBOL: BDG

VANCOUVER, British Columbia--(BUSINESS WIRE)-- Black Dragon Gold Corp.

(ASX/TSX-V: BDG) ("Black Dragon" or the  "Company") is pleased to announce the

positive results of  the Preliminary Economic Assessment ("PEA") completed on

its 100% owned  Salave Gold Project ("Salave" or "Project") located in Asturias

in  northern Spain. The PEA is based on the recently completed Mineral

Resource Estimate completed by CSA Global (See October 25, 2018 News  Release).

All figures are in United States Dollars unless otherwise  stated.

This press release features multimedia. View the full release here:

https://www.businesswire.com/news/home/20190211005527/en/

The PEA demonstrates robust economics for an underground mining  operation with

a 14-year mine life.

Paul Cronin, Managing Director and CEO commented,

"The completion of the PEA is a major milestone on the path to  development of

the Salave Project and the metrics support our belief  that Salave can

potentially generate strong returns for shareholders. It  forms the first step

in our permitting process, presenting a new  optimised process on a zero-

discharge basis that minimises the visual  and surface impact of the project.

The robust results of this PEA underline the potential economic  viability of

the current Salave resource to be mined over an initial 14  year mine life, and

our successful drilling campaign last year indicates  strong potential for

growth in mine life at Salave.

This study supports that Salave can produce over 1.1Moz (560 kt of  concentrate

averaging over 59 g/t Au), providing a number of marketing  options for export

and refining, minimising the need for additional  plant and equipment, and

hence reducing the Projects footprint. The  relatively low upfront capex also

opens alternative financing  opportunities which will ensure that both

shareholders and the local  community benefit from the success of this

Project."

KEY PEA OUTCOMES

Pre-Tax NPV at 5% discount rate: US$ 296.2 million.

After-Tax NPV: US$ 230.0 million

Pre-Tax Internal Rate of Return ("IRR"): 28%

After-Tax Internal Rate of Return ("IRR"): 25%

After-Tax Payback: 3.8 years

Pre-Production Capital Cost, including contingency: US$ 95.3 million

Life of Mine ("LOM") Sustaining Capital Cost: US$19.3 million

Estimated Average LOM Total Cash Cost: $729/ounce (oz) Au

Estimated Average LOM All-In Sustaining Costs ("AISC"): $752/oz Au

CAUTIONARY STATEMENT

The PEA is a preliminary technical and economic study of the potential

viability of the Salave Gold Project. It is preliminary in nature and  includes

Inferred Mineral Resources that are considered too  speculative geologically to

have economic considerations applied to  them that would enable them to be

categorized as Mineral Reserves.  There can be no assurance, and there is no

certainty, that the  preliminary economic assessment contained therein will be

realised.  Further exploration and evaluation work and appropriate studies are

required before Black Dragon will be in a position to estimate any Ore

Reserves or to provide any assurance of an economic development case.

The production target and forecast financial information referred to  in this

PEA are comprised of Measured and Indicated Mineral Resources  (73%) and

Inferred Mineral Resources (27%). The proportion of Inferred  Mineral Resources

is not determinative of the project viability and  does not feature as a

significant proportion early in the mine plan.

Metallurgical recoveries have been based on test work data and costs  have been

estimated by independent consultants generally from budget  quotations,

factored estimates or cost data from similar  operations/projects. Cost

estimate accuracy for the PEA is in the  order of +/-35%.

The PEA is based on the material assumptions outlined herein and in  the

report. These include assumptions about the availability of  funding. While BDG

considers all of the material assumptions to be  based on reasonable grounds,

there is no certainty that they will  prove to be correct or that the range of

outcomes indicated by the PEA  will be achieved. To achieve the range of

outcomes indicated in the  PEA, among other things, funding of in the order of

US$100 million  will likely be required. Investors should note that there is

no  certainty that Black Dragon will be able to raise that amount of  funding

when needed.

It is also likely that such funding may only be available on terms  that may be

dilutive to or otherwise affect the value of BDGs  existing shares. It is also

possible that BDG could pursue other  value realisation strategies such as a

sale, partial sale or joint  venture of the project. If it does, this could

materially reduce BDGs  proportionate ownership of the project. Given the

uncertainties  involved, investors should not make any investment decisions

based  solely on the results of the PEA.

NEXT STEPS

Submission of the Project Description for the Environmental and Social  Impact

Assessment ("ESIA") in February 2019;

Additional geophysics over the entire Investigation Permit at Salave   April

2019;

Issuance of the ESIA Terms of Reference in June 2019;

Soil Geochemistry testing on potential drill targets  June 2019;

Pre-Feasibility Study  October 2019.

PEA KEY ASSUMPTIONS AND INPUTS

Assumed gold price: US$1,250/oz

Exchange Rate of $1.15 /

Life of Mine: 14-years

Main Underground Mining Method: Vertical Retreat & Sub-Level Stoping

Average Diluted Head Grade: 3.87 g/t Au

Total Underground Dilution: 43%

LOM Plant Throughput 9.19 Mt

Access Ramp Gradient of 15% at a 5.0m x 5.5m profile

Mineralised Zone Development at a 4.0m x 4.5m profile

Average Mining and Processing throughput: 2,000 tonnes per day ("tpd")

Flotation Plant Recoveries: 97%

Average Annual Production (LOM): 79,200 oz Au in concentrate at an  average

grade of 59.7 g/t Au

LOM recovered gold in concentrate production: 1,108,420 oz;

Refining and Processing Charges: US$368/t concentrate or US$188/oz Au

Table 1 - PEA Summary Parameters

Input 	 	Unit

Physical Parameters

Total Mineralised Material Tonnes Mined (LOM) 		Mt		9.19

Average Annual Throughput (LOM) 		ktpa		656.3

Head Grade 		Au g/t		3.87

Gold Recovery to Concentrate 		% 		97%

Mine Life 		years		14

Gold Grade of Concentrate 		Au g/t		59.71

Total Concentrate Produced 		kt		560.5

Total Ounces in Concentrate 		koz		1,108.4

Average Annual Production (LOM) 		koz		79.2

Cost Parameters

Mining Costs 		US$/t		40.68

Processing Costs 		US$/t		14.00

General & Administrative 		US$/t		2.71

Total Costs 		US$/t		57.39

Pre-Production Capital Costs

Mine Development & Infrastructure 		US$m		29.7

Mining Equipment 		US$m		11.2

Tailings 		US$m		1.3

Process Plant 		US$m		28.3

Owners Costs & EPCM 		US$m		12.5

Contingency (15%) 		US$m		12.4

Total Pre-Production Capital 		US$m		95.3

Sustaining Capital 		US$m		19.3

LOM Cash Costs 		US$/oz		729.15

LOM AISC 		US$/oz		752.80

MINERAL RESOURCE ESTIMATE

An updated NI 43-101 Mineral Resource Estimate, effective 22 October  2018 is

included in this PEA and has been filed on SEDAR and the ASX  market

announcements platform (See October 25, 2018 News Release).

Tonnes	 																																																																																																																																																																																																																																																																	Au

Category		Mt		g/t	 	koz

Measured 		1.03 		5.59 		185

Indicated 		7.18 		4.43 		1,023

Measured & Indicated 		8.21 		4.58 		1,208

Inferred 		3.12 		3.47 		348

Notes:1. Rounding may cause apparent discrepancies2.  Resource Estimate

conducted by CSA Global of Perth Australia ("CSA")  with an effective date of

October 22, 2018. Classification of the MRE  was completed based on the

guidelines presented by Canadian Institute  for Mining (CIM, May 2014), adopted

for Technical Reports which adhere  to the regulations defined in Canadian NI

43-101. The Mineral Resource  Estimate was also prepared in accordance with the

Australasian Code for  Reporting of Exploration Results, Mineral Resources and

Ore Reserves,  2012 edition ("2012 JORC Code").3. The Mineral Resource

Estimate was first announced on 25 October 2018. Black Dragon confirms  that it

is not aware of any new information or data that materially  affects the

information in the previous announcement and that all  material assumptions and

technical parameters underpinning the Mineral  Resource Estimate continue to

apply and have not materially changed.4.  A cut-off grade of 2 g/t Au has been

applied when reporting the Mineral  Resource Estimate.5. Mineral Resources that

are not Mineral  Reserves do not have demonstrated economic viability but do

have  reasonable prospects for eventual economic extraction.6. The  quantity

and grade of reported Inferred Resources in this estimation are  conceptual in

nature and there has been insufficient exploration to  define these Inferred

Resources as an Indicated or Measured Resource. It  is uncertain if further

exploration will result in upgrading them to an  Indicated or Measured Resource

category, although it is reasonably  expected that the majority of the Inferred

Resources could be upgraded  to Indicated Mineral Resources with further

exploration.7. The  Mineral Resource Estimate underpinning the production

targets in this  announcement was prepared by a Competent Person under the 2012

JORC Code.8.  The title of the report is "Salave Gold Project Mineral Resource

Update  for Black Dragon Gold Corp.", with an effective date of October 22,

2018, and it was authored by Ian Stockton, B.Sc (Geol)., MAusIMM, FAIG,  Dmitry

Pertel, MSc (Geol), MAIF, GAA, and Galen White, B.Sc, FAusIMM,  FGS.

POTENTIALLY EXTRACTABLE PORTION OF  MINERALISATION FOR MINE PLANNING

The mine plan supported by the PEA demonstrates that approximately 81.1%  of

the total 2018 updated Mineral Resource tonnage is amenable to  underground

extraction. For purposes of mine planning, the potentially  extractable portion

of the Mineral Resources are comprised of 9.19  million tonnes at a diluted

grade of 3.87 g/t Au, containing just over  1.1 million ounces of gold. The

mineralised material modelled to be  mined in the PEA contains Mineral

Resources classified in the Inferred  category (28%) that are too speculative

geologically to have economic  considerations applied that would enable them to

be categorized as  Mineral Reserves. These Inferred Resources will require

further  exploration and definition to meet the criteria to be classified as

Indicated or Measured Mineral Resources before being considered for  conversion

to Mineral Reserves at the next level of detailed economic  study.

MINE PLAN

Given environmental and community considerations, the PEA has only  evaluated

underground mining operations. The primary mining method  selected for detailed

analysis in this study was the vertical retreat  mining ("VRM"). Sub-level

stoping was considered as a secondary method  applicable to specific vertical

thin geometries (<15m length). Rock and  paste fill will be used as backfill to

maximize mining recovery.

The mine design was based on basic economic assumptions to create  mineable

stope outlines. A value of 2 g/t was assumed as mine cut-off  grade. Mining

dilution and mineralised material loss factors were also  applied to each

mining shape to reflect the selected mining method.

The mine production rate targets a 0.70 Mtpa of RoM. A conceptual mine  layout

was designed including stopes and development as illustrated in  Figure 1, with

60m levels and 3 x 20m sub-levels. The total mineralised  material from stopes,

drives and sill pillar recovery (50%) will total  9.2Mt at 3.87 g/t Au.

A long term mine schedule was created integrating stopes and development  as

shown in Figures 2 and 3. Mineralised zones were sequenced to  prioritise

higher grades at lower operating costs.

MINERAL PROCESSING

In order to minimise potential social and environmental issues,  processing of

Salave mineralised material has been limited to crushing,  grinding and

flotation, with concentrates exported via local ports. Mine  feed will be

crushed on surface at a rate of 0.7 Mtpa, and then be  processed via

conventional SAG and ball milling followed by sulphide  flotation and

thickening.

The run-of-mine will feed a primary jaw crusher with a capacity of 400  t/h

with a physical availability of 70% with design factor of 20%.

From an intermediate stockpile, the coarse material will feed the mill  circuit

which consists of a conventional SAG and ball mill configuration  working in

close circuit with the cyclones.

The flotation circuit consists of a number of cells of 300-400 m3  capacity

(9-10 cells of 40 m3 each) with two conditioning  tanks for pH stabilisation

and reagents.

The final stage consists of tailings thickening to minimise the fresh  water

consumption and to re-cycle process water. A paste thickener  will achieve a

product of 70-75% of solids.

Based on flotation test work conducted to date, it is assumed that 97%  of the

gold head grade will be recovered in the flotation concentrate  that will be

thickened, filtered and bagged for shipping to customers.

INFRASTRUCTURE AND TAILINGS

Power to the project is available from Tapia, which is linked to the  Asturias

main distribution grid, and an existing network of power lines  enter the

property that are connected to the Spanish national  transmission grid. Water

for both domestic and plant usage can be  sourced from wells, the Porcia River

(2.5km east of the property) or the  reticulated water supply that is currently

in place near the plant  location.

A Tailings Management Facility ("TMF") will be constructed at surface  for

temporary storage of plant tailings. The paste and backfill of the  mine will

minimise the amount of tailings storage at surface, and  various options for

complete tailings disposal are being evaluated. The  TMF design will involve

water recovery in the processing plant and  transportation to geo-membrane

lined facility eliminating any risk for  potential surface and ground water

contamination.

Surface facilities to support the Salave Project will include an

administration and engineering building, security, warehouse, fuel and

explosive storage, fire protection, maintenance shops with a site design  to

accommodate for 50 full time staff.

CAPITAL COSTS AND SENSITIVITIES

Input (US$M) 	 	Pre-Production 	 	Sustaining	 	LOM

Development 		29.7 		15.7 		45.3

Equipment & Infrastructure 		11.2 		3.6 		14.8

Tailings 		1.3 		0.0 		1.3

Process Plant 		28.3 		0.0 		28.3

Owner Costs & EPCM 		12.5 		0.0 		12.5

Contingency (15%) 		12.4 		0.0 		12.4

Total Capex 		95.3		19.3		114.6

Sensitivity Analysis

After-Tax NPV (US$M) 									 																																																																																																																																																																																																																																																																	% relative to the Base Case

Parameter		-20% 	 																																																																																																																																																																																																																																																																	Base Case 			 																																																																																																																																																																																																																																																																	+20% 				-20% 	 																																																																																																																																																																																																																																																																	Base Case 			 	+20%

Gold price 		95.8 																																																																																																																																																																																																																																																																		230.0 																																																																																																																																																																																																																																																																				361.0 				-58% 																																																																																																																																																																																																																																																																		0% 				57%

Processing costs 		244.5 																																																																																																																																																																																																																																																																		230.0 																																																																																																																																																																																																																																																																				163.4 				6% 																																																																																																																																																																																																																																																																		0% 				-29%

Mining costs 		296.1 																																																																																																																																																																																																																																																																		230.0 																																																																																																																																																																																																																																																																				192.3 				29% 																																																																																																																																																																																																																																																																		0% 				-16%

Capex 		245.3 																																																																																																																																																																																																																																																																		230.0 																																																																																																																																																																																																																																																																				214.6 				7% 																																																																																																																																																																																																																																																																		0% 				-7%

Gold Price Sensitivities

Macro Parameters 																																																																																																																																																																																																																																																																		Unit			 																																																																																																																																																																																																																																																																	-20%			 																																																																																																																																																																																																																																																																	Base Case					 																																																																																																																																																																																																																																																																	+20%

Gold Price 																																																																																																																																																																																																																																																																		US$/oz 																																																																																																																																																																																																																																																																				1,000 										1,250 																																																																																																																																																																																																																																																																						1,500

Pre-Tax

NPV5% 																																																																																																																																																																																																																																																																		US$M 																																																																																																																																																																																																																																																																				122.2 																																																																																																																																																																																																																																																																				296.2 																																																																																																																																																																																																																																																																						469.2

IRR 																																																																																																																																																																																																																																																																		% 																																																																																																																																																																																																																																																																				16% 																																																																																																																																																																																																																																																																				28% 																																																																																																																																																																																																																																																																						40%

Post-Tax

NPV5% 																																																																																																																																																																																																																																																																		US$M 																																																																																																																																																																																																																																																																				95.8 																																																																																																																																																																																																																																																																				230.0 																																																																																																																																																																																																																																																																						361.0

IRR 																																																																																																																																																																																																																																																																		% 																																																																																																																																																																																																																																																																				14% 																																																																																																																																																																																																																																																																				25% 																																																																																																																																																																																																																																																																						36%

Payback 																																																																																																																																																																																																																																																																		Years 																																																																																																																																																																																																																																																																				6.3 																																																																																																																																																																																																																																																																				3.8 																																																																																																																																																																																																																																																																						2.6

PROJECT FUNDING

The Board of BDG believes there is a reasonable basis to assume the  necessary

funding for the Salave Gold Project will be obtained for the  following

reasons:

The Company has been able to raise funding for its exploration over  the past

years in order to progress its project. In the last two years  BDG has raised

over $14.5 million via equity placements. These raises  indicate a clear base

of support from new and existing shareholders  and third-party investors. The

Company considers it will be able to  raise funding for the next stage of the

Project, which will advance  the Project to the completion of a detailed

Feasibility Study.

The positive outcomes delivered by the PEA give confidence to the  Board in the

ability of the Company to fund the development capital  through conventional

debt and equity financing. A mix of debt and  equity is the most likely funding

model so 100% of the capital  expenditure will not need to be borrowed. The

Board has a strong  financing track record in funding start up mining

operations, and in  their view, it is reasonably expected that when the project

parameters  in this PEA are met, that funding will be able to be arranged.

Notwithstanding this, the normal risks for the raising of capital will  apply

to the Company, such as the state of equity capital and debt  markets, the

results of the Feasibility Study and the price of gold.

The Company believes that its funding opportunities will be improved  at the

completion of the Feasibility Study as a result of:

(i) confidence in the possibility to increase the Mineral Resource  Estimate

that would serve to improve the mine life of the Project;

(ii) confirmation of earlier metallurgical test work to support,  optimise and

potentially improve concentrate grades; and

(iii) finalisation of further engineering studies to improve the  accuracy of

the assessed capital and operating costs

(iv) offtake contracts for concentrates to improve revenue and  treatment

charge assumptions

The funding models being considered will depend on the outcomes of the

Feasibility Study, but as set out above will likely be conventional  debt and

equity financing, but may include convertible notes, gold  streaming,

prepayment of royalties and other options for projects of a  similar nature.

The raising of equity by the Company may be dilutive to existing  shareholders,

but that will depend on the price at which the then  funding is completed.

Where the market capitalisation of the Company  is low as against the amount of

equity that is required to be raised  at the time, there is a high likelihood

that shareholders will be  substantially diluted. This is to be balanced

against the reasonable  expectation of the Company that as the Project becomes

more advanced,  the value of the Company is more likely to increase, resulting

in the  actual dilution to existing shareholders being less. The reality is

that in this case, although the percentage holding of each shareholder  will be

reduced, the value of that holding will be assessed against a  Company that is

anticipated to have a higher market capitalisation at  the time of the raising.

PEA KEY RECOMMENDATIONS

CRS Ingenieria ("CRS"), in Madrid were the principal authors of the PEA  and

have made the following recommendations for further evaluation that  may

improve the economics of the project:

The mineralisation style indicates that both vertical retreat mining  (VRM) and

room and pillar (RP) are applicable to Salave. For this  study, a combination

of VRM and SLS was selected, configured with  60m-height panels, however, CRS

recommends assessing the benefits of  RP for individual panels.

The production rate of Salave used for the PEA was 0.70 Mtpa. While  this

production capacity is optimal under current assumptions of  mining method and

cut-off grade, CRS suggests the evaluation of  alternative cut-off strategies

that may lead to review the production  rate.

Evaluate mining methods by panel and create integrated layouts.

Develop detailed geotechnical studies to estimate stope and room  dimensions

and modifying factors such as dilution and mineralised  material loss.

Develop a volume balance of waste and paste over the LoM sequence.

Further investigate low grade materials by additional drilling to  verify

geological continuity.

Sill pillars may be recovered if specific technical and economic  studies

demonstrate that economic extraction could reasonably be  justified under

realistic conditions. CRS recommends the completion of  detailed geotechnical

studies to confirm the viability of sill pillar  recovery.

Complete an economic study considering obtain free gold through  panning before

shipment the product.

Develop a detailed market study to identify potential clients for the  Salave

gold concentrates.

QUALIFIED PERSONS AND COMPETENT PERSONS STATEMENT

The information in this announcement that relates to the PEA for the  Salave

Gold Project is based on and fairly represents information and  supporting

documentation prepared by CRS Ingenieria and CSA Global.  Paulo Laymen (P.Eng.,

M.AusIMM., B.Eng., M.Eng.) of CRS Ingenieria  supervised the preparation of the

PEA, is independent of the Company and  a qualified person as defined by

National Instrument 43-101 and has  reviewed and approved the technical

disclosure reported herein. Dmitry  Pertel (P.Geo., MSc (Geol), MAIF, GAA) and

Belinda van Lente (P.Geo.) of  CSA Global were responsible for the Mineral

Resource Estimate and are  independent of the Company and qualified persons as

defined by National  Instrument 43-101 and have reviewed and approved the

technical  disclosure reported herein.

The NI 43-101 Technical Report will be fined on SEDAR within 45 days of  this

release.

BLACK DRAGON GOLD CORP.1000 Cathedral Place925 West Georgia  StreetVancouver,

BC V6C 3L2, Canada,T- +44 20 79934077 F-  +44 20

71128814info@blackdragongold.comwww.blackdragongold.com

ABOUT BLACK DRAGON GOLD

Black Dragon Gold "BDG" is the 100% owner of one of the largest  undeveloped

gold projects in Europe, the Salave project. Salave is  situated in the North

of Spain in the province of Asturias. The Salave  project has an updated

combined Measured and Indicated Mineral Resource  of 8.21 million tonnes

grading 4.58 g/t Au, containing 1.21 million  ounces of gold, plus Inferred

resources totalling 3.12 million tonnes  grading 3.47 g/t Au, containing

348,000 ounces of gold.

A full technical report summarizing the Mineral Resource estimate  completed by

CSA Global is available on the companys web site and  posted on SEDAR. In

addition to the current Mineral Resource, historical  exploration work suggests

there is the potential for additional  mineralisation within Black Dragons

landholdings.

FORWARD LOOKING STATEMENTS

This news release contains forward-looking statements that are based on  the

Corporation's current expectations and estimates. Forward-looking  statements

are frequently characterized by words such as "plan",  "expect", "project",

"intend", "believe", "anticipate", "estimate",  "suggest", "indicate" and other

similar words or statements that certain  events or conditions "may" or "will"

occur. Such forward-looking  statements involve known and unknown risks,

uncertainties and other  factors that could cause actual events or results to

differ materially  from estimated or anticipated events or results implied or

expressed in  such forward-looking statements. Such factors include, among

others: the  actual results of current planned exploration activities; changes

in  project parameters as plans to continue to be refined; possible  variations

in recovered material grade or recovery rates; accidents,  labor disputes and

other risks of the mining industry; delays or any  inability in obtaining

governmental approvals or financing; and  fluctuations in metal prices. There

may be other factors that cause  actions, events or results not to be as

anticipated, estimated or  intended. Any forward-looking statement speaks only

as of the date on  which it is made and, except as may be required by

applicable securities  laws, the Corporation disclaims any intent or obligation

to update any  forward-looking statement, whether as a result of new

information,  future events or results or otherwise. Forward-looking statements

are  not guarantees of future performance and accordingly undue reliance

should not be put on such statements due to the inherent uncertainty  therein.

Neither the TSX Venture Exchange, nor its Regulation Services Providers  (as

that term is defined in the policies of the TSX Venture Exchange)  accepts

responsibility for the adequacy or accuracy of this news release.

All figures are rounded to reflect the relative accuracy of the news  release.

[IMAGE]

View source version on businesswire.com:

https://www.businesswire.com/news/home/20190211005527/en/

Contacts

Paul CroninCEO & Managing DirectorP: +44 20  79934077E:

paul.cronin@blackdragongold.com

Source: Black Dragon Gold Corp.

Powered by Business Wire