Source	MARKET NEWS

Date	08/17/2018

Time	05:15:49 AM

Company	 VVC Exploration Corporation

Title	Drilling

Results Show Additional near Surface Co

CDNX Symbol: VVC

Press Release

VVC EXPLORATIONS CORP ("VVC-V;VVCVF-0")

- Drilling Results Show Additional near Surface Copper

- Mineralization - Samalayuca Property, Chihuahua State, MX

VVC Exploration Corporation ("VVC" or the "Company") announced

assay results from 6 additional diamond drill holes (DDH) from the

ongoing drilling program at its Samalayuca Copper Property (the

"Property") in northern Chihuahua State, Mexico.  A total of eleven

DDH's have now been completed in the Gloria Extension Zone (GEZ).  The

results from five of these holes (SC17-22, SC17-23, SC18-24, SC18-25

and SC18-26) have previously been reported.  All eleven DDH's which

tested the zone intersected significant copper mineralization as

detailed in the table below.

This completes the drill program on the GEZ, with the results

defining and extending the copper mineralization in the near surface

zone.   A total of 1,299.5 meters have been drilled in eleven holes,

two in 2017 and nine in 2018.  The drill holes, which tested a

200-meter section of a  350-meter strike length zone, all intersected

near surface, copper mineralization.  The Company commenced drilling in

early August in the Gloria Extension East Zone (GEEZ), located 20

meters to the east of the GEZ, separated from it by a fault.  The GEEZ

is an "on strike" extension of the GEZ with copper mineralization

traced by artisanal miners over an 850 m strike length.

Results from the eleven (11) drill holes testing the GEZ, are

summarized below, with only values )0.1% Cu shown.

//st

Hole Number      From (m)   To (m)   Length (m)   Ag (ppm)   Cu (%)

SC17_022         14.5       42.25    27.75        7          0.510

SC17_023         10         61       51           5          0.36

SC18_024         8          38       30           6          0.342

Including        9          23       14           10         0.539

SC18_024         53         59.5     6.5          5          0.223

SC18_025 Sec 1   57.2       65       7.8          7          0.394

SC18_025 Sec 2   69         77.9     8.9          4          0.270

SC18_025 Sec 3   84         115      31           6          0.394

SC18_026 Sec 1   34         59       25           7          0.433

SC18_026 Sec 2   126.4      129.2    2.8          3          0.194

SC18_027         52.5       60.5     8            3          0.214

SC18_028         19.7       51.5     31.8         2          0.376

SC18_029 Sec 1   8.5        26.5     18           1          0.310

SC18_029 Sec 2   68         72       4            19         0.967

SC18_030         36         89       53           6          0.357

SC18_031         84.7       93.7     9            4          0.215

SC18_032 Sec 1   3.95       11       7.05         3          0.195

SC18_032 Sec 2   58         64       6            3          0.462

//et

GEZ Drilling

The GEZ is a copper mineralized zone, previously discovered and

mined by artisanal miners (gambusinos) in the 1950's and 1960's, from

two shallow open pits.   VVC started diamond drilling in November 2017

and completed the last of the eleven DDH, SC-17-022, SC-17-023 and

SC-18-024-032 in July.   Results from  the first five DDH have

previously been reported, with the three most recent, SC-18-024-026,

reported in a news release dated August 2, 2018.

VVC's Chairman, Dr. Terrence Martell pointed to the results from

the GEZ as further confirmation of the potential of the Company's

Samalayuca Copper Property and added.  "We are now focused on drilling

the 850-meter strike length of the GEEZ where eight holes have been

drilled to date, all intersecting near surface copper mineralization.

Assays are pending and will be announced when received."

A plan map and sections 425E, 475E, 525E, 575E, show the

dimensions of the copper mineralization in the GEZ.  The maps and

sections along with drill locations and results can be viewed on our

website at:  www.vvcexpl.com.

Exploration Summary - Samalayuca - 2018 Phase III Drilling Program

The Company's Samalayuca Phase III drilling program, planned for

3000 meters, began in May 2018 and is expected to be completed in

September 2018.   The drilling is intended to further evaluate the

potential of the GEZ and GEEZ areas, and to evaluate mineralized zones

in previously mined areas at La Concha and an area which includes the

Zorra, Suerte, Juliana and Trinadad artisanal pits, none of which have

been drilled by VVC.

The Company's goal is to define the thickness and grade of the

mineralized zones and their structural / lithologic settings using

information gained from structural mapping and drilling.  The drilling

shows the potential of the property to host significant stratiform

copper mineralization, which has been traced for over 5 km based on the

shallow artisanal mines and VVC's drilling of the 32 DDH completed to

date.

Geology

The Samalayuca property covers copper mineralisation in the

Samalayuca Sierra, characterised as Stratiform Copper Deposit type, as

recognized by qualified persons, Michel Gauthier and Jocelyn Pelletier

(2012), and Jacques Marchand and Michel Boily (2013).  Fine

disseminated chalcopyrite-bornite and associated oxide copper minerals,

common to these deposits, are noted in a wide chloritized zone along

the Sierra with mineralized zones from 3 to 36 m wide, true thickness,

as defined in the old surface pits.  The stratigraphy strikes NW and

dips gently, approximately 25 degrees to the NE, although it is

variable.  Vertical faults can generate supergene copper enrichment

when they cut the mineralised strata.  The old mining works by the

artisanal miners (gambusinos) targeted the higher grade, surface,

copper zones, exploiting them by shallow (( 20 m deep) open pits and

hand cobbing the ore for direct shipping to the smelter.

Analysis and QA/QC

A total of 1189 core samples were sent to the ALS Chemex

laboratory in Chihuahua city, MX for analysis by their ICP61 technique.

Mineralisation is mainly oxidized copper minerals such as malachite,

azurite, chrysocolla, brochantite, and others with some disseminated

copper sulphides, such as chalcopyrite, bornite, and chalcocite.

ALS Chemex analytical services are accredited by SANAS and are

carried out with a quality assurance protocol in line with ISO

17025:2005.   Samples are stored at the Corporation's field camps, then

put into sealed bags until delivered by company personnel to the

laboratory where the samples are prepared and analyzed.  Samples are

logged in the tracking system, weighed, dried and finely crushed to

better than 70% passing a 2 mm (Tyler 9 mesh, US Std. No.10) screen. A

split of up to 1,000 g is taken and pulverized to better than 85%

passing a 75-micron (Tyler 200 mesh) screen, and a 50-gram split is

analyzed.  Blanks, duplicates and certified reference material

(standards) are inserted to monitor laboratory performance.

This news release has been reviewed for accuracy and compliance

under National Instrument 43-101 by Peter M. Dimmell, BSc., P.Geo. (NL,

ON), a VVC Director and Qualified Person as defined by NI 43-101,

Standards of Disclosure for Mineral Projects.  He has approved the

scientific and technical disclosure in the news release.

About VVC Exploration Corporation

VVC is a Canadian exploration and mining company focused on the

exploration and development of copper and gold deposits in Northern

Mexico, specifically the Samalayuca copper property in Chihuahua State.

VVC has other projects in Mexico and Canada, including gold and silver

prospects, Cumeral and La Tuna, in Sonora and Sinaloa States, Mexico

and a grassroots gold/VMS prospect in the Timmins area of northern

Ontario.

Visit our website at: http://vvcexpl.com.

On behalf of the Board of Directors

Michel J. Lafrance, Secretary-Treasurer

//st

Patrick Fernet - (514) 631-2727         or   Eugene Braigen - (646)

245-3409

E-mail: pfernet@vvcexploration.com           E-mail:

eugene@vvcexpl.com

//et

Neither TSX Venture Exchange nor its Regulation Services Provider

(as that term is defined in policies of the TSX Venture Exchange)

accepts responsibility for the adequacy or accuracy of this release.

____________________________________________________________

(c)2018 Market News Publishing Inc.  All rights reserved.

Toronto:(416)366-8881   Vancouver:(604)689-1101   Fax:(604)689-1106

#1822800009TS11822800340-05143220180817##