Source	MARKET NEWS

Date	09/04/2018

Time	06:47:55 AM

Company	 Rizal Resources Corporation

Title	T'Boli

Gold Silver Project Exploration Update

CDNX Symbol: RZL

Press Release

RIZAL RESOURCES CORP ("RZL-V")

- T'Boli Gold-Silver Project Exploration Update

Rizal Resources Corporation ("Rizal" or the "Company")

Detailed mapping highlights potential controls on the location of high

grade gold at the T'Boli Gold Silver Project ("T'Boli" or T'Boli Project").

Trial mining activities with gold mineralization separation techniques have

resulted in an improved delivered gold grade to the Processing Plant of

over 4g/t for the month of July.

Similar trial mining using the same mining method in the higher-grade South

Veins are planned in late September with the objective of improving the

grade to the Processing Plant.

Processing of bulk samples from trial mining resulted in Gold (601oz) and

Silver (1,059oz) produced and sold during the month of July with revenues

exceeding the operating costs at T'Boli. Total revenue was CAD946,209

(unaudited), while operating costs totaled CAD658,912 (unaudited).

In connection with trial mining and bulk sampling activities, refinements

to the mining method and enhancements to the processing plant are ongoing

with the objective of increasing tonnage throughput and processing

capabilities.

Planned exploration development of drives accessing the South Veins will

intersect two of the high-grade drill hole intercepts with 3.7m (true

width) @ 15g/t (uncut) from TG008 and 8.0m (true width) @ 14.2g/t (uncut)

from TG038, scheduled for completion in 2018.

Rizal has completed a further stage of underground exploration

activities including detailed mapping and assaying, data collation and

interpretation.  There has been a substantial improvement to the

understanding of the high-grade gold locations at T'Boli in recent

months which continues to build on the Company's ability to build up

the information it requires to prepare deliver a current Mineral

Resource estimate, and information required for development of a

current mine plan.

In addition, as part of the ongoing effort to gather important

geological, mining and processing information about the gold

mineralization at T'Boli, a modified approach to the trial mining

commenced in July.  The adjusted mining approach has enabled the

separation of some lower grade and higher-grade material, resulting in

a higher delivered average grade to the processing plant.

Of particular significance for the planned activities in the

coming months will be development of exploration drives providing

access to some of the better drill hole intercepts found within the

drilling database. There are a number of significant high-grade drill

hole intercepts that exist on the defined South Veins underneath the

previous development.  The planned access into the South Vein will

initially be located 10m drill hole TG038 which intersected 8.0m (True

Width) @ 14.2g/t (uncut) and with development to the east planned soon

after to access the South Veins in the vicinity of drill holes, TG079

(1.1m @ 21g/t) and TG080 (2.8m @ 8.0g/t) - see figure 4.

Growing the understanding of the high-grade gold at T'Boli

There is a general observation at the T'Boli Project that

high-grade gold assay results (over 10g/t) exist at various locations

in the drilling database and from the underground sampling data.  It is

considered that the geological understanding of these high-grade gold

locations is the key driver for any assessment of the prospects for

economic extraction of the mineralization at the T'Boli Project.

Therefore, the geological understanding and possible continuity of the

high-grade gold has been a focus of the technical work effort at T'Boli

over the past 6 months.

Earlier observations from mapping available in the North Veins had

shown that the high-grade gold corresponded with intersection points

between structures of various orientations.  It has also been observed

from the earlier underground development and sampling that the

high-grade gold locations in the South Vein were of greater consistency

and continuity than observed in the North Veins.  However, until

recently, accessing the development in the South Veins to validate the

geological structures associated with these high-grade zones was not

possible.

A more detailed review of the South Veins was recently completed

after rehabilitation of the Mofoko underground workings was completed.

This has revealed that there is a relationship between the smaller

scale north dipping veins which link closely to the dominant South

dipping veins and the locations of more consistent high-grade gold

mineralization (Figures 1 and 2).

Trial Mining Activities

Given the complex vein array and linking structures that exist at

the T'Boli mine, a modified and slightly more selective form of trial

mining commenced in July.  This approach looked at allowing sections of

the stope to be collected and sampled prior to delivery to the

processing plant.  Not all stopes could be accessed to identify the

geological location of the material derived from the open stope.

However, selected sections of material gathered from the stope were

sampled and stockpiled separately with the results used to determine

which tonnes were sent to the processing plant and which tonnes were

sent to a low grade or waste stockpile.

The bulk of the tonnes processed during July were from the 515

level access on the North Veins.  The average grade of the bulk sample

results for tonnes delivered to the processing plant are shown in

figure 3 below. The results identified for each location have now been

fully reconciled with the gold produced at the T'Boli processing plant

during the month of July (allowing for estimated gold sent to tails and

adjustments to surface stockpiles).

This approach and attempted selectivity has had an immediate

positive effect on the average grade delivered to the processing plant

in July. Prior to July, the average grade of tonnes processed was fully

reconciled to average less than 2g/t.  In July, the grade delivered to

the Processing Plant has increased to over 4g/t, with an average

reconciled head grade for the month of July of 4.2g/t.

This has also had a significant positive effect on the revenue

raised during the month of July, which has exceeded the total operating

costs at the mine site.  A summary of the total operating costs for the

trial mining activities and the recovered gold and silver for the month

of July is summarized in table 1 (all results are unaudited).

//st

Activity          Tonnes/oz   Total Costs/Revenue (CAD)      Cost per

tonne (CAD)

Total Mining      5,953t                          $319,353   $53.65/t

Costs

Total             4,705t                          $239,651   $50.94/t

Processing

Costs

Other Site        4,705t                          $ 99,909   $21.23/t

Costs &

Royalties

Total Site                                        $658,912   $125.82/t

Costs

(Operating)

Gold Produced     601oz                           $931,479

Silver Produced   1,069oz                         $ 14,730

Total Revenue                                     $946,209

//et

Note: All revenue and operating costs reported are management

prepared and unaudited

Table 1: Summary of total operating costs and Gold/Silver produced

for the month of July, 2018. Gold and Silver Revenue is based on a

realized gold price of CAD1,549.88/oz and CAD13.77/oz respectively.

All costs estimates are calculated using a CAD:PHP exchange rate of

40.63.

Future Work Program

Work at the T'Boli mine is continuing with the trial mining on the

North Veins and development is well underway to access the interpreted

higher-grade material located on the South Veins.  The initial level

access into the South Veins is planned to be at 10 to 15m vertically

beneath and in-between drill holes TG038 (Figure 4).  A second deeper

access location on the South Vein will be also developed in the coming

months which is situated 80m to the east of, and 10 to 15m beneath the

significant drill hole intersection identified in TD008 (Figure 4).

Important note regarding Mining and Processing Activities at

T'Boli

Note that the T'Boli Project does not have a current Mineral

Resource estimate or Mineral Reserves and that the absence of a

feasibility study of mineral reserves demonstrating economic and

technical viability is associated with an increased risk of failure of

the mining operation due to increased uncertainty of results.  Mining

and processing activity at the T'Boli Project is ongoing on a

small-scale basis associated with the Company's underground exploration

activities and this should not be construed as Rizal having made a

commercial production decision.

Quality Control

Channel sampling undertaken within the exploration development

drives are consistently taken on a horizontal line as rock chip samples

between 1m to 1.5m above the floor of the development which is

approximately perpendicular to the dominant dip of the mineralized

structures.  The dominant geological domains and mineralized structures

dip to the South at between 60 and 90 degrees.  Sample intervals vary

from between 0.2m and 1.0m in width with the sample boundaries obeying

the observed geological contacts.

Drill cores from the prior drilling programs were HQ, NQ and BQ in

size.   For drill core that was HQ or NQ size, selected sampling

intervals were sub-sampled using different techniques depending on the

nature of the drill core.  Competent drill core was cut longitudinally

in half with a core saw; Clay dominant or soft core was divided into

two halves using a spatula; Broken core or rubble was physically

separated into 2 separate and equal halves.  For drill core that was BQ

in size, the whole sample interval was taken for analysis.  Sample

intervals taken were predominantly at 1m or less, with sample interval

boundaries selected to ensure that the sampling interval did not cross

over a defined geological contact.

Of particular note with regard to the quality control of the prior

drill core samples is that for many of the sampling locations the core

recovery was reported to be poor.  Based on the observations of the

mineralized structures from the underground development this is

interpreted to be a result of some clay rich sections that occur in

close proximity to the mineralized structures.  The underground channel

sampling has also identified that the clay rich locations often contain

significant gold mineralization.  The above observations indicate that

the core loss recorded in the prior drilling is due to the poor capture

of the mineralized clay rich material surrounding parts of the larger

structures at T'Boli.  The loss of mineralized material may result in

an underestimation of the gold content within some of the reported

drilling intercepts.

Prior drilling samples were analysed by Interteck, which is an

independent external laboratory, with the processing laboratory

situated in Manila, Philippines.

Sampling preparation and analysis for the channel samples are

undertaken at the Company's internal laboratory situated at T'Boli.

For both the prior drilling and the current channel samples from

the exploration development, the Company has maintained quality control

systems which includes the insertion of blanks, standards and pulp

duplicates.  The control samples from the prior drilling information

have been reviewed for any possible lab bias.  The review has concluded

that there are no material biases or inaccuracies in the assay

database, with additional check data required to confirm an observation

that there may be a slight bias of results on the low side resulting in

a slight understatement in the reported gold assay information.

Qualified Persons

Steven Olsen, member of AIG, has reviewed the exploration data and

prepared the scientific and technical information regarding exploration

results contained within this news release.  Steven Olsen, an

Independent Consultant, is a "Qualified Person" as such term is defined

in National Instrument 43-101 and he has verified and approved the

contents of this news release.

About Rizal

Rizal is an exploration and development Company with a focus on

its flagship T'Boli Project situated in the Philippines.  The Company

is using underground development to explore and define the extensive

gold mineralization that exists at T'Boli.

Peter Main President and CEO pmain@rizalresources.com

For further information please visit our website at

www.rizalresources.com, Email us at info@rizalresources.com, Phone:

778-370-1372 Fax: 604-608-5442

Neither TSX Venture Exchange nor its Regulation Services Provider

(as that term is defined in the policies of the TSX Venture Exchange)

accepts responsibility for the adequacy or accuracy of this release.

Photos accompanying this announcement are available at

http://www.globenewswire.com/NewsRoom/AttachmentNg/91dec7f4-2125-49b8-8120-

1991de1eb065

http://www.globenewswire.com/NewsRoom/AttachmentNg/0e34aeb2-6eee-4156-9f9c-

eba7cbe50860

http://www.globenewswire.com/NewsRoom/AttachmentNg/ce729d30-9192-42fc-a289-

b322e9166375

http://www.globenewswire.com/NewsRoom/AttachmentNg/54c45c37-6ce2-481a-a06e-

883db2d634c9

____________________________________________________________

(c)2018 Market News Publishing Inc.  All rights reserved.

Toronto:(416)366-8881   Vancouver:(604)689-1101   Fax:(604)689-1106

#1824600034TS11824601175-06474220180904##