Source	Marketwire

Date	06/22/2017

Time	05:30:36 AM

Company	 MIDLAND EXPLORATION INC.

Title	Midland and Osisko Identify Several Prospective Areas in the James Bay

Region Near Eleonore and Resume Exploration Work

CDNX Symbol: MD

Press Release

NEWS RELEASE TRANSMITTED BY MARKETWIRED

FOR: Midland Exploration Inc.

TSX VENTURE SYMBOL: MD

June 22, 2017

Midland and Osisko Identify Several Prospective Areas in the James Bay Region

Near Eleonore and Resume Exploration Work

MONTREAL, QUEBEC--(Marketwired - June 22, 2017) - Midland Exploration Inc.

("Midland") (TSX VENTURE:MD) is pleased to announce that several new

prospective areas were identified during the first phase of a joint exploration

program carried out 50/50 with Osisko Mining ("Osisko"). This strategic

alliance formed in 2016 covers a regional area of more than 100 kilometres in

length, deemed highly favourable and respectively located to the northwest of

the Eleonore gold mine and to the southeast of the new gold discovery at

Cheechoo.

The first regional exploration program carried out in 2016 included till

surveys (717 samples), lake sediment surveys (102 samples) as well as

prospecting work (240 rock samples). As a result of this work, several

prospective areas were delineated, including eleven (11) high-priority areas

that will be the focus of a ground follow-up during the next phase of

exploration.

These new exploration targets, identified during the 2016 program, may be

divided into two broad areas, located on either side of the Eleonore mine: the

Northwest Area and the Southeast Area. Anomaly thresholds for tills and lake

sediments were characterized by Osisko using their regional database for the

James Bay region.

Northwest Area

/T/

--  Target 1: Characterized by a cluster of till samples with anomalous gold

values above the 95th percentile ("PCT"), over more than 2 kilometres,

in heavy mineral concentrates and in the fine fraction.

--  Target 2: Presence of an anomaly in gold grain counts in till samples

and of arsenic values greater than 95 PCT in the fine fraction, in

addition to a 2-metre-wide iron formation that graded 0.17 g/t Au in the

prospecting campaign.

--  Target 3: A follow-up on an anomaly in gold grain counts in till samples

yielded a copper anomaly greater than 95 PCT in the fine fraction.

--  Target 4: Anomaly covering more than 6 square kilometres, characterized

by four (4) lakes with anomalous gold values greater than 99 PCT and

one lake with anomalous arsenic greater than 99.5 PCT.

/T/

Southeast Area

/T/

--  Target 5: Cluster of gold and copper anomalies in the fine fraction of

till samples greater than 95 PCT.

--  Target 6: Area characterized by several gold anomalies identified by

prospecting, with values of 0.48 g/t Au, 0.28 g/t Au, 0.21 g/t Au and

0.17 g/t Au in grab samples collected in mafic volcanic rocks and

paragneisses with pyrrhotite and pyrite mineralization. This area is

located at the contact between the La Grande and Opinaca subprovinces.

(Note that values from grab samples reported in this press release are

selective by nature and may not be representative of the mineralized

zones).The contact between these two geological subprovinces is the most

important gold metallotect in the region.

--  Target 7: Three copper anomalies in the fine fraction of till samples

greater than 95 PCT, combined with a zinc anomaly greater than 99 PCT;

a float boulder also yielded a grade of 6.85% Cu.

--  Target 8: Target characterized by favourable geology identified during

prospecting, with the presence of a silicified paragneiss with calc-

silicate alteration and 2-10% pyrite and 2-3% pyrrhotite mineralization,

which yielded values up to 0.31 g/t Au in grab sample. This target is

also located directly along the prolific contact between the La Grande

and Opinaca subprovinces.

--  Target 9: A gold anomaly greater than 95 PCT in the fine fraction,

where cursory prospecting work was carried out in 2016.

--  Target 10: Cluster of lake sediment anomalies over more than 5

kilometres, characterized by two gold anomalies greater than 99.5 PCT,

one gold anomaly greater than 95 PCT, and one antimony anomaly greater

than  98 PCT.

--  Target 11: Several lakes with anomalous antimony values ranging from 0.3

to 0.9 ppm Sb.

/T/

The upcoming exploration program, with a total budget of at least $200,000,

will mainly include prospecting and till surveys to follow up on these

high-priority targets identified in 2016.

This new joint venture with Osisko encompasses several properties for a total

of 1,827 mining claims covering a surface area of approximately 952.9 square

kilometres. These new properties are located about 12 kilometres to the

southeast and northwest of the Eleonore deposit held by Goldcorp Inc. This

world-class gold deposit hosts proven reserves estimated, as at June 30, 2016,

at 3.09 million tonnes at a grade of 6.72 g/t Au (0.67 Moz Au) and probable

reserves of 20.35 million tonnes grading 5.97 g/t Au (3.90 Moz Au). The ore

deposit also hosts measured resources of 3.21 million tonnes grading 7.27 g/t

Au (0.75 Moz Au), indicated resources of 1.92 million tonnes grading 2.97 g/t

Au (0.18 Moz Au) and inferred resources of 5.14 million tonnes at a grade of

5.66 g/t Au (0.93 Moz Au). (Note that the resource is not necessarily

indicative of the mineralization on the property, and there is no certainty

that a resource will be identified on the Company's property).

Maps showing the location of the new prospective areas and properties held by

the strategic alliance may be consulted using the following link:

http://media3.marketwire.com/docs/BJ_Osisko_JV_June2017.pdf

Quality Control

Rock samples from the EJV property are assayed by standard 30 gram

fire-assaying with AA or gravimetric finish at AlS laboratories in Val d'Or,

Quebec or Sudbury, Ontario. All samples are also analysed for multi-elements,

using four-acid -ICP -AES method at AlS laboratories. Exploration program

design, Quality Assurance/Quality interpretation of results is performed by

qualified persons employing a Quality Assurance/Quality Control program

consistent with the industry best practices. Standards and blanks are included

with every 20 samples for Quality Assurance/Quality Control purposes by the

Corporation as well as the lab.

Till samples of 15 kg were shipped to Overburden Drilling Management in Ottawa,

Ontario for gold grain counts using a shaking table and then hand panning for

gold. A non-magnetic heavy mineral fraction extracted from the 15 kg tills was

shipped to Actlabs in Ancaster, Ontario. The heavy mineral fraction was assayed

for a multi-element analysis which consists in an atomic emission spectroscopy

analysis (ICP-ES) with an aqua-regia digestion.

Till samples of 1 kg were sieved using a 63 micrometer mesh to get a fine

fraction at Actlabs in Ancaster, Ontario. The fine fraction was analyzed using

ICP-ES following a four-acid digestion and a neutron activation analysis

(INAA). The data was reviewed by Mario Masson, VP Exploration for Midland,

certified geologist and Qualified Person as defined by NI 43-101.

About Midland

Midland targets the excellent mineral potential of Quebec to make the discovery

of new world-class deposits of gold, platinum group elements, base metals and

rare earth elements. Midland is proud to count on reputable partners such as

Osisko Mining Inc., Altius Resources Inc., Agnico Eagle Mines Limited, Teck

Resources Limited, IAMGOLD Corporation, SOQUEM INC., Japan Oil Gas and Metals

National Corporation, and Abcourt Mines Inc. Midland prefers to work in

partnership and intends to quickly conclude additional agreements in regard to

newly acquired properties. Management is currently reviewing other

opportunities and projects to build up the Company portfolio and generate

shareholder value.

This press release was prepared by Mario Masson, VP Exploration for Midland,

certified geologist and Qualified Person as defined by NI 43-101. For further

information, please consult Midland's website: www.explorationmidland.com

Neither TSX Venture Exchange nor its Regulation Services Provider (as that term

is defined in the policies of the TSX Venture Exchange) accepts responsibility

for the adequacy or accuracy of this release.

This press release may contain forward-looking statements that are subject to

known and unknown risks and uncertainties that could cause actual results to

vary materially from targeted results. Such risks and uncertainties include

those described in Midland's periodic reports including the annual report or in

the filings made by Midland from time to time with securities regulatory

authorities.

- 30 -

CONTACT INFO:

Gino Roger

President and Chief Executive Officer

450 420-5977

Fax: 450 420-5978

info@midlandexploration.com

www.explorationmidland.com

INDUSTRY: Manufacturing and Production - Mining and Metals

SUBJECT: MEX - MINING EXPLORATION UPDATE

----------------------------------------------

Suite 900, 25 York Street, Toronto, ON M5J 2V5

Toll Free:888-299-0338

Phone: 416-362-0885

info@marketwired.com

----------------------------------------------

If you no longer want to receive

announcements from us, please do not reply to this e-mail.

Instead simply click

http://marketwire.com/mw/fmu.jsp?id=595545678