Source	MARKET NEWS

Date	05/30/2016

Time	06:14:55 AM

Company	 Divergent Energy Services Corp.

Title	Performance Results From Its Second generation L

CDNX Symbol: DVG

Press Release

Performance Results From Its Second generation Linear#Electromagnetic

Submersible Pump# --->@NEWS RELEASE#

DIVERGENT ENERGY SERVICES CORP ("DVG-V;ILISF-0")

- Performance Results From Its Second-generation Linear

- Electromagnetic Submersible Pump

(Not for dissemination in the United States of America)

DIVERGENT Energy Services Corp. ("Divergent" or the "Corporation")

is pleased to announce that a second-generation Linear Electromagnetic

Submersible Pump (the "Pump") was installed in an oil well in Southeast

Saskatchewan on May 17, 2016. The Pump started without issues and is

operating at expected pump efficiency and with a reduction in average

power draw as compared to the previous installation.

The Pump has been producing 19.7 m3/d (124 bbl/d) from a depth of

1,360m (4,462'). Pumping rates will be incrementally increased towards

the anticipated upper limit of 27 m3/d (170 bbl/d).

As compared to the Pump installed on January 22, 2016, the current

Pump is using an average of 9% less electricity, which is attributed to

reduced friction within the motor and results from an improved

lubricant.

The Corporation is pursuing additional installations with clients

that have expressed interest in the Pump's potential to reduce

operating costs from both rod wear and electricity usage.

ABOUT THE PUMP

The Linear Electromagnetic Submersible Motor creates the

reciprocating motion that is conventionally created at surface by

pumpjacks and transferred to the pump by sucker rods. By creating

motion at the bottom of the well, the mechanical efficiency is

increased with the motor coupled directly to the downhole,

reciprocating positive-displacement pump. The Pump is powered and

controlled by a variable frequency drive at surface via an industry

standard electric cable that is attached to the outside of the

production tubing. The elimination of surface lifting equipment and

sucker rods directly reduces operating costs in environments such as

deviated and horizontal wells.

ABOUT DIVERGENT ENERGY SERVICES CORP.

Headquartered in Calgary, Alberta, DIVERGENT Energy Services Corp.

provides an array of artificial lift products and services that are

used in the oil and gas industry. Products include Linear

Electromagnetic Submersible Pumps, Electric Submersible Pumps and

Electric Submersible Progressing Cavity Pumps.

FORWARD LOOKING STATEMENTS

This press release contains forward-looking statements, including,

without limitation, statements pertaining to the installation of

additional hardware and the starting of the Pump. All statements

included herein, other than statements of historical fact, are

forward-looking information and such information involves various risks

and uncertainties. There can be no assurance that such information will

prove to be accurate, and actual results and future events could differ

materially from those anticipated in such information. A description of

assumptions used to develop such forward-looking information and a

description of risk factors that may cause actual results to differ

materially from forward-looking information can be found in the

Corporation's disclosure documents on the SEDAR website at

www.sedar.com. Forward-looking statements are based on estimates and

opinions of management of the Corporation at the time the information

is presented. The Corporation may, as considered necessary in the

circumstances, update or revise such forward-looking statements,

whether as a result of new information, future events or otherwise, but

the Corporation undertakes no obligation to update or revise any

forward-looking statements, except as required by applicable securities

laws.

Neither the TSX Venture Exchange nor its Regulation Services

Provider (as that term is defined in the policies of the TSX Venture

Exchange) accepts responsibility for the adequacy or accuracy of this

release.

ENQUIRIES:

DIVERGENT Energy Services Corp.

Ken Berg

President and Chief Executive Officer

(403) 543-0060

(403) 543-0069 (FAX)

DIVERGENT Energy Services Corp.

Scott Hamilton

Chief Financial Officer

(403) 543-0060

(403) 543-0069 (FAX)

www.divergentenergyservices.com

______________________________

_______________________________________________

TSX Venture closing price for DVG-V

Date: 2016/05/27

Closing Price: 0.24

____________________________________________________________

(c)2016 Market News Publishing Inc.  All rights reserved.

Toronto:(416)366-8881   Vancouver:(604)689-1101   Fax:(604)689-1106

#1615000019TS11615001086-06120120160530##