Source	MARKET NEWS

Date	05/03/2018

Time	05:15:03 AM

Company	 Otis Gold Corp.

Title	2017 Oakley

Project Exploration Results

CDNX Symbol: OOO

Press Release

OTIS GOLD CORP ("OOO-V;OGLDF-0")

- 2017 Oakley Project Exploration Results

Otis Gold Corp. ("Otis" or the "Company") is pleased to announce

the results of its 2017 exploration program at the Oakley Project,

Cassia County, Idaho. Surface geochemical sampling has revealed

significant gold- and silver-in-soil anomalies over 2 kilometres (or

"km"); stream sediment sampling over 3 km of drainage indicates

increasing gold and silver values toward the potential source, and rock

sampling of outcrops over a large area of epithermal black matrix

breccia reveal gold values up to 3.0 g/t gold (or "Au") and silver

values up to 306 g/t silver (or "Ag").

The Oakley Project ("Oakley") is a gold-silver exploration

prospect located 21 km south of Oakley, Idaho, and 3 km north of the

Idaho-Utah state line along the western flank of the Albion Mountains,

part of the Albion-Raft River-Grouse Creek Metamorphic Core Complex.

Oakley contains three primary targets located approximately 6 km apart,

comprising the Blue Hill Creek and Matrix Creek targets located at the

southern end of the project (Regional Geology map), and Cold Creek

target to the north. All work reported herein was performed at the

Matrix Creek target, where black matrix breccia (or "BMB") forms the

dominant host rock and is associated with regional scale detachment

structures.

Highlights of the 2017 Matrix Creek Target Work

*  A total of 1,334 soil samples were analyzed for gold and

silver. The results define a coherent 2-km-long gold- and

silver-in-soil anomaly that is coincident with a detachment structure

and associated mineralized BMB body. The anomalies extend from Matrix

Creek northwest toward the Blue Hill Creek deposit along what is now

known as "Discovery Ridge". The anomalous zone is defined by gold

values up to 0.988 g/t Au and silver values up to 5.4 g/t Ag.

*  58 rock-chip samples of BMB collected on Discovery Ridge along

the brecciated detachment zone contained up to 3.0 g/t Au and up to 306

g/t Ag along 350-metres (or "m") of outcrop; this target that has not

been drill-tested (Regional Sampling Au Results and Regional Sampling

Ag Results maps).

*  49 stream-sediment samples collected along 3 km of the Matrix

Creek drainage, and into the area of its headwaters, exhibit Ag values

ranging from 0.2 to 2.3 g/t and increase toward Discovery Ridge.

*  34 one and half metre chip-channel rock samples of BMB

collected in, and along, the Discovery Ridge target area contained

intervals that include 18.0 m averaging 0.34 g/t Au and 97.7 g/t Ag,

7.9 m averaging 0.30 g/t Au and 83.3 g/t Ag, 6.7 m averaging 0.34 g/t

Au and 61.4 g/t Ag, and 2.1 m averaging 1.37 g/t Au and 132 g/t Ag.

These values indicate the potential for additional gold-silver targets

where the detachment extends down-dip below the surface.

In 2018, Otis plans to expand its land position as well as conduct

further exploration, including drilling, to follow-up on the previous

programs.

Details of the 2017 Exploration

The 2017 exploration program followed up on a 2015 program from

which assays of 14 rock samples of BMB returned an average of 0.55 g/t

Au and 70.4 g/t Ag; they were collected from a ridge in the Matrix

Creek target area, now referred to as "Discovery Ridge". Exploration in

late Spring 2017 consisted of structural mapping and the collection of

1,334 soil samples, 58 rock-chip samples, 49 stream-sediment samples,

and 34 one and a half metre long chip-channel samples of the BMB.

Chip-channel samples were collected using a generator-powered electric

chipping hammer. ALS Minerals of Reno, Nevada, analyzed all soil, rock

and stream-sediment samples. Soil samples were dry screened to 180

microns (80 mesh); rocks samples were crushed and a 250g split

pulverized to 85% passing 75 microns (200 mesh). Gold was analyzed by

fire assay and Au-ICP22; 48 elements including silver were analyzed by

ICP MS-61; Ag-OG62 was used for silver over-limit assay greater than

100 ppm.

About Matrix Creek and the Oakley Project

The Oakley Project hosts gold-silver, epithermal hot spring-type

mineralization at two targets: Blue Hill Creek and Cold Creek, and

detachment-related gold-silver mineralization at Matrix Creek. Blue

Hill Creek (Technical Report dated August 8, 2016) contains an Inferred

Resource of 163,000 ounces gold in 9.97 million tonnes at a grade of

0.51 g/t Au. The Blue Hill Creek resource is hosted in rocks of the

Tertiary Salt Lake Formation within a northwest-trending Tertiary

graben that is part of a north trending, five-mile-long by

one-mile-wide zone of low-sulphidation, hot spring-type gold

occurrences along the western margin of the Albion Mountains.

Matrix Creek is located 1.5 km southeast of the Blue Hill Creek

resource. The BMB mineralization at Matrix Creek is distinctive from

the epithermal mineralization at Blue Hill Creek in terms of style,

host rock lithology and structural controls, however, both occur within

the Tertiary graben setting. The BMB from outcrop section observations

is at least 12 m thick and consists of quartzite fragments in a black

to dark-grey matrix of fine-grained quartz and pyrite. The host rocks

are part of a lower Paleozoic package of quartzite and limestone that

occurs throughout eastern Nevada, western Utah, and southern Idaho.

In 2014, an Otis-Radius Gold joint venture at Blue Hill Creek

intercepted gold mineralization in 3 out of 5 core holes drilled. These

gold intercepts (Otis News Release dated February 5, 2015) were: 123 m

averaging 0.691 g/t Au in drill hole BHC 14-01, 41.1 m averaging 0.705

g/t Au in drill hole BHC 14-03, and 113.9 m averaging 0.405 g/t Au in

drill hole BHC 14-04. All three holes also contained silver intercepts

that were not reported and these intercepts included: 148 m averaging

5.54 g/t Ag, 61.9 m averaging 6.03 g/t Ag, and 101.7 m averaging 4.18

g/t Ag, respectively. At Otis request, ALS Minerals performed 48-hour

bottle-roll tests on the BMB with the results showing gold recoveries

up to 82%, and silver recoveries up to 93%.

Otis President and CEO, Craig Lindsay stated: "We are pleased with

the results generated at Oakley, and we see significant potential to

aggressively expand the project. It is uncommon to see the type of

surface mineralization discovered at Matrix Creek, in terms of both

grade and strike length. We now have three well-defined exploration

targets at Oakley in Blue Hill Creek, Cold Creek and Matrix Creek."

The qualified person under National Instrument (NI 43-101)

Standards of Disclosure for this News Release is Alan Roberts, CPG, and

Vice President of Exploration, who has reviewed and approved its

technical contents.

About the Company Otis is a resource company focused on the

acquisition, exploration, and development of precious metal deposits in

Idaho, USA. Otis is currently developing its flagship property, the

Kilgore Project, located in Clark County, Idaho and the Oakley Project,

located in Cassia County, Idaho.

ON BEHALF OF THE BOARD

"Craig T. Lindsay"

President & CEO

For additional information, please

Contact Information:

contact:

Mr. Tony Perri - Corporate Development

Tel:  (604) 424-8100  Email: tony@otisgold.com

Neither TSX Venture Exchange nor its Regulation Services Provider

(as defined in the policies of the TSX Venture Exchange) accepts

responsibility for the adequacy or accuracy of this release.

The forward-looking information contained in this press release is

made as of the date hereof and Otis undertakes no obligation to update

publicly or revise any forward-looking information, whether as a result

of new information, future events or otherwise, unless so required by

applicable securities laws.

This News Release does not constitute an offer to sell or a

solicitation of an offer to sell any securities in the United States.

The securities have not been and will not be registered under the

United States Securities Act of 1933, as amended (the "US Securities

Act") or any State securities laws, and may not be offered or sold

within the United States or to US Persons unless registered under the

US Securities Act and applicable State securities laws, or an exemption

from such registration is available.

Photos accompanying this announcement are available at

ttp://resource.globenewswire.com/Resource/Download/27816346-fa4d-48e2-9964-

01b9759f3faa

http://resource.globenewswire.com/Resource/Download/0995ff23-2d97-4dda-aee2

-d114dd8a7969

http://resource.globenewswire.com/Resource/Download/471fbba7-15d8-40e7-ac63

-c381084d0e90

____________________________________________________________

(c)2018 Market News Publishing Inc.  All rights reserved.

Toronto:(416)366-8881   Vancouver:(604)689-1101   Fax:(604)689-1106

#1812200002TS11812200802-05135520180503##