Source	CANADA NEWSWIRE

Date	12/02/2019

Time	04:02:57 AM

Company	 Jervois Mining Limited

Title	Jervois drilling results from Idaho Cobalt Operations, United States

CDNX Symbol: JRV

Press Release

" BC-Jervois-IdahoCobalt   12-02 6657

News release via Canada NewsWire, Toronto 416-863-9350 -ME-

Attention co.402640:

^Jervois drilling results from Idaho Cobalt Operations, United States@

ASX / TSX-V: JRVOTC: JRVMF / FRA: IHS

HIGHLIGHTS

<<

--  Successful infill and metallurgical drilling in 2H 2019 at Idaho Cobalt

Operations ("ICO") has intercepted high grade cobalt ("Co") intervals up

to 2.83% Co in the main RAM resource zone and hanging wall zones.

--  Selected composite intervals include 4.6 metres (at) 1.14% Co, 2.75% Cu,

1.01g/t Au; 3.5 metres (at) 1.46% Co, 2.75% Cu, 1.83g/t Au, 5.4 metres (at)

0.60% Co, 1.36% Cu, 0.74g/t Au and 3.7 metres (at) 1.07% Co, 0.13% Cu,

0.69g/t Au.

--  Drilling programme core provided ore composites for metallurgical

testwork to both support locked cycle tests associated with the

production of separated cobalt and copper concentrates, and generate

physical product samples. Cobalt concentrate has been dispatched to

customers with further shipments expected across December and January.

--  An updated resource is currently underway. CSA Global have been engaged

to undertake an independent audit ahead of finalization of the

geological model for BFS and the selection of senior lender Independent

Engineers for project financing. The updated resource model will be

released once the audit is complete.

--  Confidence in the economic potential of the resource is growing as more

information is generated for the updated BFS at ICO. This includes the

potential to operate at higher production rates than currently

formalised under existing feasibility studies.

<>>

MELBOURNE, Dec. 2, 2019 /CNW/ - Jervois Mining Limited (the "Company" or

"Jervois") (ASX: JRV) (TSX-V: JRV) (OTC: JRVMF) (FRA: IHS) is pleased to

provide results of the 2H 2019 drilling results at its Idaho Cobalt Operations

("ICO").

Since taking ownership of Idaho Cobalt Operations ("ICO") in July,

Jervois aggressively drilled during 2H 2019 to optimize and de-risk the

bankable feasibility study ("BFS") underway, and to begin to assess potential

mineral resource upside. The Company completed 3,125 metres of diamond core

drill drilling over 19 holes to support the bankable feasibility study ("BFS")

currently in progress. All holes targeting the currently modelled Ram resource

intersected mineralization, representing an increase of over 20 percent of all

resource grade holes undertaken on the Ram deposit. In addition, two holes

were extended to explore the footwall of the RAM deposit, successful results

for both step out exploration holes were reported 15 October 2019. The Ram

deposit resource remains open at depth and along strike offering opportunities

for expansion. The initial footwall exploration results confirm an extension

of the Blackbird mine horizons resulting in the opportunity to further explore

the Blacktail North extension zone on strike and dip. An extensive drill

programme is planned during the US summer of 2020.

Fifteen core holes have intercepted Co-Cu mineralization re-confirming

the Ram zone mineralization and extending hanging wall zones. Ore grade

intervals typically include a high grade component and medium grade zones

providing a composite mineable width. Both whole and half PQ size core has

been used to provide samples for ongoing metallurgical testwork and

concentrate production. Significant intercepts are summarized in Table 2

below. Table 1 lists the drillhole collar details and Figures 1 is a plan view

and Figures 2, 3 and 4 are oblique cross sections of the completed drilling.

Table 1: Drillhole Collars

Coordinates

BHID

UTM_E_NAD83

UTM_N_NAD83

Elevation

Plan

Plan

Final Depth

Z11N                    Z11N                     (m)

Dip            Azi            (m)

---

R19-02                707546.8                5002220.8

2196.9            -75            245               431.2

---

R19-03                707547.1                5002223.1

2196.9            -67            220               164.0

---

R19-04                707579.2                5002233.8

2198.2            -70            245               186.8

---

R19-05                707579.8                5002234.2

2198.3            -75            245               196.0

---

R19-06                707580.5                5002234.5

2198.2            -80            245               232.3

---

R19-07                707579.4                5002236.2

2198.2            -65            225               220.4

---

R19-08                707569.3                5002273.8

2169.3            -64            252               165.2

---

R19-09                707570.6                5002273.9

2169.3            -70            248               177.4

---

R19-10                707571.7                5002274.1

2169.1            -76            242               184.1

---

R19-11                707540.7                5002261.1

2171.9            -60            250               118.6

---

R19-12                707540.7                5002261.1

2171.9            -70            250               127.7

---

R19-13                707542.3                5002262.0

2171.9            -62            228               122.8

---

R19-14                707514.8                5002291.6

2154.4            -75            227               116.7

---

R19-15                707514.4                5002291.9

2154.4            -65            238               116.7

---

R19-16                707514.0                5002293.2

2154.3            -68            270               124.4

---

R19-17                707417.4                5002240.3

2158.7            -90              0                20.7

---

R19-18                707417.4                5002240.3

2158.7            -75            245                16.5

---

R19-19                707416.5                5002241.9

2158.7            -55            205                15.0

---

Table 2: Significant Drilling Results for Holes R19-02 to R19-19.

Co>0.15%

BHID

Depth

Depth

Intercept     Co %    Cu %      Au (g/t)

Composite

Zone

From          To (m)          (m)

(m)

---

R19-02         122.8           123.6               0.8    0.34

0.05          0.54

Ram

R19-03         119.8           120.5               0.8    1.00

1.47          0.64

2.4m (at) 0.45% Co,

Ram

120.5           121.3               0.8    0.21      2.52          0.19

1.43% Cu, 1.1g/t Au

Ram

121.3           122.2               0.9    0.20      0.48          2.23

Ram

123.1           123.9               0.8    0.49      0.10          0.63

Ram

128.0           128.9               0.9    1.17      0.11          0.52

Ram

R19-04          73.1            73.8               0.7    0.19

0.03          0.10

HW

150.3           154.4               4.1

Whole core Met

Ram

154.4           155.4               1.1    0.18      0.35          0.21

1.7m (at) 0.22% Co,

Ram

155.4           156.1               0.6    0.27      0.08          0.14

0.25% Cu, 0.18g/t Au

Ram

R19-05         117.3           118.0               0.6    0.28

1.15          0.12

HW

156.5           157.2               0.7    1.03      2.48           0.7

4.6m (at) 1.14% Co,

Ram

157.2           157.9               0.7    0.75      2.92          0.55

2.75% Cu, 1.01g/t Au

Ram

157.9           158.5               0.6    0.47      1.99          0.33

Ram

158.5           159.1               0.6    0.99      2.85           1.6

Ram

159.1           159.8               0.7    2.18      4.55          1.96

Ram

159.8           160.3               0.5    1.49      2.66          1.11

Ram

160.3           161.1               0.8    1.09      1.86          0.91

Ram

R19-06         167.3           171.3               4.0

Whole core Met

Ram

171.3           171.9               0.6    0.81      6.06          4.47

3.5m (at) 1.46% Co, Au

Ram

171.9           172.5               0.6     1.1      3.93          1.73

2.75% Cu, 1.83g/t

Ram

172.5           173.1               0.6    1.23      1.63          0.77

Ram

173.1           174.0               0.9    1.18      2.58          0.73

Ram

174.0           174.8               0.8    2.76      0.20          1.93

Ram

R19-07          69.4            70.0               0.6    0.28

0.03          0.05

HW

75.0            75.6               0.6    2.24      0.01          0.63

1.2m (at) 1.32% Co,

HW

75.6            76.2               0.6    0.41      0.02          0.24

0.02% Cu, 0.44g/t Au

HW

154.0           154.8               0.9    1.35      3.17           2.8

5.4m (at) 0.6% Co,

Ram

154.8           155.5               0.6    0.35      0.91          0.29

1.36% Cu, 0.74g/t Au

Ram

155.5           156.1               0.6    0.97      2.88          0.56

Ram

156.1           157.1               0.9    0.29      1.26          0.38

Ram

157.1           157.9               0.8    0.51      0.83          0.32

Ram

157.9           158.8               0.9    0.42      0.38          0.23

Ram

158.8           159.4               0.6    0.28      0.07          0.36

Ram

165.0           165.6               0.6    0.74      0.03          0.28

FW

R19-08         126.6           127.4               0.9    0.49

0.59          0.23

2.4m (at) 0.64% Co,

Ram

127.4           128.1               0.7    0.37      0.59          0.21

0.62% Cu, 0.37g/t Au

Ram

128.1           128.9               0.8    1.06      0.70          0.68

Ram

R19-09          51.5            52.0               0.5    0.51

0.02          0.21

HW

135.7           136.4               0.7    1.91      0.95          1.36

1.3m (at) 1.33% Co,

Ram

136.4           137.0               0.6    0.67      0.20          0.34

0.6% Cu, 0.89g/t Au

Ram

137.3           138.1               0.7    0.90      0.36          0.42

Ram

R19-10          55.1            55.7               0.6    0.77

0.00          0.38

HW

99.1            99.7               0.6    0.15      0.12          0.07

HW

131.4           132.4               1.1    0.18      0.13          0.03

HW

144.8           145.4               0.6    0.22      0.27          0.20

Ram

146.0           146.5               0.5    0.49      0.64          0.28

2.3m (at) 0.51% Co,

Ram

146.5           147.0               0.6    0.79      1.40          0.50

0.64% Cu,0.30g/t Au

Ram

147.0           147.5               0.5    0.50      0.14          0.41

Ram

147.5           148.3               0.8    0.31      0.40          0.08

Ram

R19-11          32.1            32.6               0.5    0.21

0.15          0.54

HW

99.0            99.7               0.7    0.27      0.08          0.42

3.7m (at) 1.07% Co,

Ram

99.7           100.6               0.9    0.27      0.05          0.22

0.13% Cu, 0.69g/t Au

Ram

100.6           101.5               0.9    0.42      0.25          0.52

Ram

101.5           102.1               0.6    2.83      0.21          1.47

Ram

102.1           102.7               0.6    2.35      0.05          1.15

Ram

R19-12         105.2           105.8               0.6    0.31

0.43          0.26

2.4m (at) 0.57%Co,

Ram

105.8           106.4               0.6    1.13      0.14          1.05

0.26%Cu, 0.47g/tAu

Ram

106.4           107.0               0.6    0.67      0.35          0.39

Ram

107.0           107.6               0.6    0.16      0.15          0.18

Ram

R19-13          64.0            64.9               0.9    0.20

0.02          0.10

HW

103.9           104.5               0.6    0.48      0.09          0.37

Ram

104.5           105.2               0.6    2.41      0.15          1.46

1.8m (at) 1.06% Co,

Ram

105.2           105.8               0.6    0.28      0.08          0.23

0.1% Cu, 0.69g/t Au

Ram

106.4           107.0               0.6    0.15      0.23          0.48

Ram

R19-14           0.0             0.0               0.0    0.00

0          0.00

No Significant Assays

R19-15          64.6            65.2               0.6    0.45

0.04          0.47

Ram

R19-16          23.5            24.4               0.9    0.56

0.02          0.21

HW

81.7            82.4               0.7    0.23      0.10          0.35

Ram

R19-17           8.8             9.4               0.6    0.21

0.04          2.18

Ram

R19-18

No Significant Assays

R19-19

No Significant Assays

---

Notes:                  1. Core holes selectively

sampled.

2. Whole core Met - Whole core

ore composites consumed by

metallurgical testwork, no

assays.

3. Ram (equal sign) Main Ram Zone, HW (equal sign)

Hanging wall Zone, FW (equal sign)

Footwall Zone.

4. Previous results for holes

R19-01 and R19-02 reported 15

October 2019.

Holes R19-18 and R19-19 did not record significant cobalt as these holes

were specifically targeted for oxidized material for metallurgical tests as

part of the BFS, at very shallow depths - less than 20 metres below surface.

At these near surface depths, the ore zones are variably leached of cobalt and

copper but typically with gold remaining in the zones. These near surface

areas will form the future crown pillar of the mine and are not considered for

mining or inclusion in resources nor reserves. Hole R19-14 encountered a shear

at the expected Ram intercept resulting in oxidation and leaching of cobalt

and copper.

Overall the drilling results are in line with expectations from

historical drilling and the previous modelled resource. The 2H 2019 drilling

by Jervois confirms the ICO as a high-grade primary cobalt deposit with

significant copper and gold credits. Information from this drilling program

will not only be used to update the resource model but also provides

information for the development of the integrated resource model with

structural and lithological controls which will aid with realistic resource

modelling and mine design.

Metallurgical samples were taken from all Ram intercepts and are

currently undergoing testwork at SGS Lakefield (Canada). Results from these

tests will be fed into the BFS update and also provide concentrate samples for

marketing under the split Cobalt and Copper concentrates regime currently in

place.

Confidence in the economic potential of the resource is growing as more

information is generated for the updated BFS on the ICO. This includes the

potential to ultimately operate at higher production rates than currently

formalised under existing feasibility studies in an environmentally

responsible manner. A domestic cobalt refinery within the United States will

also be positively impacted by a mining operation delivering higher volumes of

cobalt concentrate for a longer period.

Quality AssuranceCore samples or geochemical analysis are sent to ALS

Elko Nevada, an independent and fully accredited laboratory in the USA for

analysis for gold & multi-element Induction Coupled Plasma Spectroscopy. Core

samples from the main Ram zone are sent to SGS Lakefield, Canada for analysis

as per ALS Elko and for further metallurgical testing. Jervois also has a

regimented Quality Assurance, Quality Control program where at least 10%

duplicates and blanks are inserted into each sample shipment.

On behalf of the Board of Directors of Jervois,Bryce Crocker, CEO and

Dir

d

holes.

Documentation of primary data, data entry procedures, data

verification, data storage (physical and electronic)

protocols.

Discuss any adjustment to assay

data.

Significant intersections are alternatively verified by the CP and QP of the

company.

No holes have been twinned in this drill programme.

Data is collected using a customized template on MS Excel.  The data is backed

up

systematically on and off site as well as on the cloud. As well, data is

recorded

using a master Microsoft Office Excel spreadsheet.

All data below detection limit have been entered as zero.

Samples received damaged at the laboratory, or with insufficient sample weight

for

analysis had the interval or location left blank.

---

Location of data                                Accuracy and quality of surveys

used to locate drill

holes                                                        All collars were

surveyed by licensed surveyors. Down-hole surveys were routinely

points                                          (collar and down-hole surveys),

trenches, mine

workings                                                           carried out

on all holes using a Reflex EZ-Trac at 50 foot intervals downhole.

and other locations used in Mineral Resource

estimation.                                                          Holes were

setup on collar using a Reflex TN14 Gyro.

Specification of the grid system used.

Quality and adequacy of

topographic control.

All datum is collected and recorded in UTM_NAD83 Z11N

The 3D location of the individual samples is considered to be adequately

established,

consistent with accepted industry standards.

Data spacing and                                Data spacing for reporting of

Exploration

Results.                                                                To

date, due to the exploratory nature of the footwall drilling, the spacing is

wide.

distribution

Whether the                                                            Assay

results for only one hole and for only the footwall intercept have been

data spacing and distribution is sufficient to establish

received. Further intervals have been sampled but the assays are not yet

available.

the degree of geological and grade continuity appropriate

for the Mineral Resource and Ore Reserve estimation

procedure(s) and classifications applied.

Whether sample

compositing has been applied.

The intervals reported for these 2 holes is exploratory and data spacing would

not be

considered sufficient to establish a Mineral Resource or Ore Reserve Estimation

for

this particular mineralisation intercept.

Data is reported both composited and uncomposited.

---

Orientation of                                  Whether the orientation of

sampling achieves

unbiased                                                             Drilling

sections are orientated perpendicular to the strike of the host rocks.

data in relation                                sampling of possible structures

and the extent to

which                                                           Drill holes

were inclined between 55 and 90 to optimize intercepts of

to geological                                   this is known, considering the

deposit

type.

mineralisation with respect to thickness and distribution.

structure

If the relationship

between the drilling orientation and the orientation of keymineralised

structures is considered to have introduced a

sampling bias, this should be assessed and reported if

material.

Drilling with angled holes in most instances provides a representative sample

across

the stratigraphy.

Sample security                                 The measures taken to ensure

sample

security.

All individual samples are bagged and sealed with a zip tie.  Then individual

samples

are bagged in poly woven sacks and sealed with a zip tie. The samples are sent

by

courier to the lab and tracked. To date, no sample shipments have had reported

problems and/or a breach in security.

---

Audits or reviews                               The results of any audits or

reviews of

sampling

Jervois protocols consist of a regimented internal QA/QC which match or exceed

techniques and

data.

global industry standards.  This drilling program has not

been subjected to audits

or reviews so far, however this will be completed as part of the current

feasibility

study of which this drilling is a part.

---

Section 2 Reporting of Exploration Results

Criteria

JORC Code explanation

Commentary

Mineral tenement and                                 Type, reference

name/number, location and ownership

including                                                          Idaho Cobalt

Operations consists of 243

land tenure status                                   agreements or material

issues with third parties such as

joint                                                         unpatented

mineral claims totalling 5990

ventures, partnerships, overriding royalties, native

title                                                             acres.  The

claims are 100% owned by Jervois

interests, historical sites, wilderness or national park

and                                                           subsidiary

Formation Capital LLC and are in

environmental

settings.

good standing.

The security of the tenure held at the time of

reporting along with any known impediments to obtaining a

licence to operate in the area.

Unpatented Mineral Claims:

Ownership of unpatented mining claims in the

U.S. is in the name of the holder, with

ownership of the minerals belonging to the

United States of America, under the

administration of the U.S. Bureau of Land

Management. Under the Mining Law of 1872,

which governs the location of unpatented

mining claims on federal lands, the locator

has the right to explore, develop and mine

minerals on unpatented mining claims without

payments of production royalties to the

federal government.  Annual claim maintenance

and filing fees paid before September 1st each

year are the only federal encumbrances to

unpatented mining claims. Exploration plans

are permitted and administered by the Unites

States Forestry Service.

The United States Department of Agriculture

Salmon Challis National Forest (the Forest

Service) issued a revised Record of Decision

(the ROD) for the ICO in January 2009. The ROD

described the decision to approve a Mine Plan

of Operations (MPO) for mining, milling and

concentrating mineralized material from the

ICO. The ROD was subsequently affirmed by the

Forest Service in April 2009. The Plan of

Operations at the ICO mine and mill remained

unchanged and the ROD remains in place. In

December 2009, the Forest Service approved the

MPO allowing for the commencement of ICO

construction.

There are no known encumbrances.

---

Exploration done by                                  Acknowledgment and

appraisal of exploration by other

parties.                                                          The ICO came

under Jervois management following

other

parties

the merger with eCobalt in 2019. Prior to this

merger, the area has a long history of copper

and cobalt exploration and mining. Copper

mineralization in the Blackbird Creek area was

discovered in 1892, and the area was soon

explored as both a copper and gold prospect.

The area was first mined by Union Carbide at

the Haynes-Stellite Mine located south of the

present ICO claim block, during World War I.

Union Carbide mined approximately 4,000 tons

of cobalt-bearing ore before ceasing

operations. From 1938 to 1941, the Uncle Sam

Mining and Milling Company operated a mine at

the south end of the present Blackbird mine

and reportedly mined about 3,600 tons of ore.

Calera Mining Company, a division of Howe Sound

Company, developed and mined the Blackbird

deposit between 1943 and 1959 under a contract

to supply cobalt to the U.S. government.

Calera stopped mining when the government

contract was terminated in 1960.

Machinery Center Inc. mined from the district

between 1963 and 1966, when Idaho Mining

Company (owned by Hanna Mining Company)

purchased the property. Noranda optioned the

property from Hanna in 1977 and carried out

extensive exploration, mine rehabilitation andmetallurgical testing. In 1979

Noranda and

Hanna formed the Blackbird Mining Company

(BMC) to develop the property. BMC completed

an internal feasibility study of their

property at the time, including material from

the Sunshine deposit in 1982. BMC allowed

perimeter claims to lapse in 1994, and eCobalt

restaked much of that ground. From 1995 to the

present, eCobalt completed surface geochemical

sampling and drilled 158 diamond drill holes

on the ICO ground.

Geology                                              Deposit type, geological

setting and style of mineralisation.

Deposit Types:

Whilst the deposits in the Idaho Cobalt Belt

have been studied over many years the deposit

types are still a subject of debate. Prior to

2005 the overriding opinion was that the

deposits are sedimentary exhalative and are

referred to as the Blackbird Sediment Hosted

Cu-Co. And have been described as stratabound

iron-, cobalt-, copper-, and arsenic-rich

sulphide mineral accumulations in nearly

carbonate-free argillite/siltite couplets

and quartzites

Post 2005 the discovery of high concentrations

of rare earth elements (REE) lead to the

postulation that the deposits are not

volcanogenic massive sulphide or sedimentary

exhalative deposits but instead are iron

oxide-copper-gold (IOCG) deposits

Geological Setting:

The ICO is situated in the Idaho Cobalt Belt, a

30- to 35-mile long metallogenic district

characterized by stratiform/tabular copper-

cobalt deposits. The deposits are hosted by a

thick, dominantly clastic sequence of Middle

Proterozoic age sandwiched between late

Proterozoic quartz monzonitic intrusions. The

clastic sediments were deposited in a large

fault-bounded basin, probably as large

submarine fan complexes and/or deltaic aprons

that were frequently "drowned" by continuing

subsidence within the basin. All significant

copper-cobalt deposits and occurrences are

found in the Proterozoic Apple Creek

Formation, which constitutes the base of this

sequence. This formation was originally

correlated with Pritchard Formation

metasediments of the Belt supergroup to the

north, its age being constrained by dates of

1.37 Ga for adamellites intruding the sequence

and 1.7 Ga from mafic dykes and sills emplaced

along the basin margin faults.

The structure of the Apple Creek Formation is

dominated by the regional rift structure.

Cobalt-copper-gold mineralization occurs

along a northwest-southeast trending

structure parallel to and west of the central

axis of the rift.

There is a series of northerly trending faults

that are considered to represent initial

growth faults, reactivated by Laramide and

younger events. The district has also been

affected by north-easterly structures of the

Trans-Challis Fault Zone.

The ICO is hosted in Proterozoic age meta-

sediments found on the east side of the

central Idaho Batholith comprising granitic-

to-granodioritic rocks. The Idaho Cobalt Belt

represents a distinct district dominated bystratabound cobalt + copper +/- gold

mineralization, with a remobilized

constituent. The district is underlain by

strata of the middle Proterozoic-age Apple

Creek Formation, which is an upward-

thickening, upward-coarsening clastic

sequence at least 49,000 feet thick that

represents a major basin-filling episode and

was formerly considered part of the Yellow

Jacket Formation.

The Apple Creek can be divided into three

units. The lower unit of the Apple Creek

Formation is over 15,000 feet thick and

consists mainly of argillite and siltite, with

lesser occurrences of fine-grained quartzite

and carbonates. Graded bedding and planar to

wavy laminae are common in the lower unit,

which is locally metamorphosed to phyllite.

The middle unit of the Apple Creek Formation

is up to 3,600 feet thick and comprises

several upward-coarsening sequences of

argillite, siltite, and quartzite, with

distinctive biotite-rich interbeds that

generally have a direct correlation to

mineralization. The middle unit hosts the

majority of the known cobalt, copper and gold

occurrences in the Idaho Cobalt Belt. The

upper unit exceeds 9,800 feet in thickness and

is predominantly composed of thin- to thick

bedded, very fine- to fine-grained

quartzite.

Mafic tuffs within the Apple Creek Formation

are the oldest igneous rocks exposed in the

Sunshine-Blackpine district. They are

accompanied by felsic tuffs and carbonatitic

tuffs. Some mafic dikes and sills intrude the

Apple Creek Formation and may be comagmatic

with the mafic tuff beds. Several small

lamproitic diatremes may also be coeval with

mafic volcanism.

The Apple Creek Formation has undergone varying

degrees of regional metamorphism, ranging from

greenschist facies in the southern part of the

district to amphibolite grade facies in the

northern part of the district. Several types

of mafic dikes and sills, ranging from 3 ft.

to 100 ft. thick, intrude the Apple Creek

Formation and are interpreted as feeders to

the exhalative mafic tuffs, which are most

abundant in areas of intrusive actvity

Style of Mineralization:

Mineralization at the ICO is characterized as

syngenetic, stratiform/tabular exhalative

deposits within, or closely associated with,

the mafic sequences of the Apple Creek

Formation. This mineralization is dominantly

bedding concordant and the deposits range from

nearly massive to disseminated. Some

crosscutting mineralization is present that

may be in feeder zones to the stratiform

mineralization or may be due to remobilization

locally into fracture quartz veins and/or

crosscutting structures.

Dominant minerals include cobaltite (CoAsS) and

chalcopyrite (CuFeS2), with lesser, variable

occurrences of gold. Other minerals present in

small quantities are pyrite (FeS2), pyrrhotite

(FeS), arsenopyrite (FeAsS), linnaeite ((CoNi)3S4), loellingite (FeAs2),

safflorite

(CoFeAs2), enargite (Cu3AsS4) and marcasite

(FeS2).

Recently, rare-earth minerals have been

identified in samples from the deposit as

monazite, xenotime and allanite. At this time,

these minerals have not been considered for

potential recovery as by-products of the Co-

(Cu-Au).

The Ram is the largest and best-known deposit

in the ICO area. It consists of a Hanging-

wall Zone with 3 primary and 4 minor horizons,

a Main Zone comprising 3 horizons, and a

Footwall Zone with 3 horizons. These sub-

parallel horizons generally strike N15oW and

dip 50o - 60o to the northeast. Most of the

significant Co mineralization is associated

with exhalative lithologies i.e. biotitic

tuffaceous exhalate (BTE), siliceous

tuffaceous exhalate (STE),

" BC-Jervois-IdahoCobalt   12-02 6657

News release via Canada NewsWire, Toronto 416-863-9350 -ME-

Attention co.402640:

^Jervois drilling results from Idaho Cobalt Operations, United States@

ASX / TSX-V: JRVOTC: JRVMF / FRA: IHS

HIGHLIGHTS

<<

--  Successful infill and metallurgical drilling in 2H 2019 at Idaho Cobalt

Operations ("ICO") has intercepted high grade cobalt ("Co") intervals up

to 2.83% Co in the main RAM resource zone and hanging wall zones.

--  Selected composite intervals include 4.6 metres (at) 1.14% Co, 2.75% Cu,

1.01g/t Au; 3.5 metres (at) 1.46% Co, 2.75% Cu, 1.83g/t Au, 5.4 metres (at)

0.60% Co, 1.36% Cu, 0.74g/t Au and 3.7 metres (at) 1.07% Co, 0.13% Cu,

0.69g/t Au.

--  Drilling programme core provided ore composites for metallurgical

testwork to both support locked cycle tests associated with the

production of separated cobalt and copper concentrates, and generate

physical product samples. Cobalt concentrate has been dispatched to

customers with further shipments expected across December and January.

--  An updated resource is currently underway. CSA Global have been engaged

to undertake an independent audit ahead of finalization of the

geological model for BFS and the selection of senior lender Independent

Engineers for project financing. The updated resource model will be

released once the audit is complete.

--  Confidence in the economic potential of the resource is growing as more

information is generated for the updated BFS at ICO. This includes the

potential to operate at higher production rates than currently

formalised under existing feasibility studies.

<>>

MELBOURNE, Dec. 2, 2019 /CNW/ - Jervois Mining Limited (the "Company" or

"Jervois") (ASX: JRV) (TSX-V: JRV) (OTC: JRVMF) (FRA: IHS) is pleased to

provide results of the 2H 2019 drilling results at its Idaho Cobalt Operations

("ICO").

Since taking ownership of Idaho Cobalt Operations ("ICO") in July,

Jervois aggressively drilled during 2H 2019 to optimize and de-risk the

bankable feasibility study ("BFS") underway, and to begin to assess potential

mineral resource upside. The Company completed 3,125 metres of diamond core

drill drilling over 19 holes to support the bankable feasibility study ("BFS")

currently in progress. All holes targeting the currently modelled Ram resource

intersected mineralization, representing an increase of over 20 percent of all

resource grade holes undertaken on the Ram deposit. In addition, two holes

were extended to explore the footwall of the RAM deposit, successful results

for both step out exploration holes were reported 15 October 2019. The Ram

deposit resource remains open at depth and along strike offering opportunities

for expansion. The initial footwall exploration results confirm an extension

of the Blackbird mine horizons resulting in the opportunity to further explore

the Blacktail North extension zone on strike and dip. An extensive drill

programme is planned during the US summer of 2020.

Fifteen core holes have intercepted Co-Cu mineralization re-confirming

the Ram zone mineralization and extending hanging wall zones. Ore grade

intervals typically include a high grade component and medium grade zones

providing a composite mineable width. Both whole and half PQ size core has

been used to provide samples for ongoing metallurgical testwork and

concentrate production. Significant intercepts are summarized in Table 2

below. Table 1 lists the drillhole collar details and Figures 1 is a plan view

and Figures 2, 3 and 4 are oblique cross sections of the completed drilling.

Table 1: Drillhole Collars

Coordinates

BHID

UTM_E_NAD83

UTM_N_NAD83

Elevation

Plan

Plan

Final Depth

Z11N                    Z11N                     (m)

Dip            Azi            (m)

---

R19-02                707546.8                5002220.8

2196.9            -75            245               431.2

---

R19-03                707547.1                5002223.1

2196.9            -67            220               164.0

---

R19-04                707579.2                5002233.8

2198.2            -70            245               186.8

---

R19-05                707579.8                5002234.2

2198.3            -75            245               196.0

---

R19-06                707580.5                5002234.5

2198.2            -80            245               232.3

---

R19-07                707579.4                5002236.2

2198.2            -65            225               220.4

---

R19-08                707569.3                5002273.8

2169.3            -64            252               165.2

---

R19-09                707570.6                5002273.9

2169.3            -70            248               177.4

---

R19-10                707571.7                5002274.1

2169.1            -76            242               184.1

---

R19-11                707540.7                5002261.1

2171.9            -60            250               118.6

---

R19-12                707540.7                5002261.1

2171.9            -70            250               127.7

---

R19-13                707542.3                5002262.0

2171.9            -62            228               122.8

---

R19-14                707514.8                5002291.6

2154.4            -75            227               116.7

---

R19-15                707514.4                5002291.9

2154.4            -65            238               116.7

---

R19-16                707514.0                5002293.2

2154.3            -68            270               124.4

---

R19-17                707417.4                5002240.3

2158.7            -90              0                20.7

---

R19-18                707417.4                5002240.3

2158.7            -75            245                16.5

---

R19-19                707416.5                5002241.9

2158.7            -55            205                15.0

---

Table 2: Significant Drilling Results for Holes R19-02 to R19-19.

Co>0.15%

BHID

Depth

Depth

Intercept     Co %    Cu %      Au (g/t)

Composite

Zone

From          To (m)          (m)

(m)

---

R19-02         122.8           123.6               0.8    0.34

0.05          0.54

Ram

R19-03         119.8           120.5               0.8    1.00

1.47          0.64

2.4m (at) 0.45% Co,

Ram

120.5           121.3               0.8    0.21      2.52          0.19

1.43% Cu, 1.1g/t Au

Ram

121.3           122.2               0.9    0.20      0.48          2.23

Ram

123.1           123.9               0.8    0.49      0.10          0.63

Ram

128.0           128.9               0.9    1.17      0.11          0.52

Ram

R19-04          73.1            73.8               0.7    0.19

0.03          0.10

HW

150.3           154.4               4.1

Whole core Met

Ram

154.4           155.4               1.1    0.18      0.35          0.21

1.7m (at) 0.22% Co,

Ram

155.4           156.1               0.6    0.27      0.08          0.14

0.25% Cu, 0.18g/t Au

Ram

R19-05         117.3           118.0               0.6    0.28

1.15          0.12

HW

156.5           157.2               0.7    1.03      2.48           0.7

4.6m (at) 1.14% Co,

Ram

157.2           157.9               0.7    0.75      2.92          0.55

2.75% Cu, 1.01g/t Au

Ram

157.9           158.5               0.6    0.47      1.99          0.33

Ram

158.5           159.1               0.6    0.99      2.85           1.6

Ram

159.1           159.8               0.7    2.18      4.55          1.96

Ram

159.8           160.3               0.5    1.49      2.66          1.11

Ram

160.3           161.1               0.8    1.09      1.86          0.91

Ram

R19-06         167.3           171.3               4.0

Whole core Met

Ram

171.3           171.9               0.6    0.81      6.06          4.47

3.5m (at) 1.46% Co, Au

Ram

171.9           172.5               0.6     1.1      3.93          1.73

2.75% Cu, 1.83g/t

Ram

172.5           173.1               0.6    1.23      1.63          0.77

Ram

173.1           174.0               0.9    1.18      2.58          0.73

Ram

174.0           174.8               0.8    2.76      0.20          1.93

Ram

R19-07          69.4            70.0               0.6    0.28

0.03          0.05

HW

75.0            75.6               0.6    2.24      0.01          0.63

1.2m (at) 1.32% Co,

HW

75.6            76.2               0.6    0.41      0.02          0.24

0.02% Cu, 0.44g/t Au

HW

154.0           154.8               0.9    1.35      3.17           2.8

5.4m (at) 0.6% Co,

Ram

154.8           155.5               0.6    0.35      0.91          0.29

1.36% Cu, 0.74g/t Au

Ram

155.5           156.1               0.6    0.97      2.88          0.56

Ram

156.1           157.1               0.9    0.29      1.26          0.38

Ram

157.1           157.9               0.8    0.51      0.83          0.32

Ram

157.9           158.8               0.9    0.42      0.38          0.23

Ram

158.8           159.4               0.6    0.28      0.07          0.36

Ram

165.0           165.6               0.6    0.74      0.03          0.28

FW

R19-08         126.6           127.4               0.9    0.49

0.59          0.23

2.4m (at) 0.64% Co,

Ram

127.4           128.1               0.7    0.37      0.59          0.21

0.62% Cu, 0.37g/t Au

Ram

128.1           128.9               0.8    1.06      0.70          0.68

Ram

R19-09          51.5            52.0               0.5    0.51

0.02          0.21

HW

135.7           136.4               0.7    1.91      0.95          1.36

1.3m (at) 1.33% Co,

Ram

136.4           137.0               0.6    0.67      0.20          0.34

0.6% Cu, 0.89g/t Au

Ram

137.3           138.1               0.7    0.90      0.36          0.42

Ram

R19-10          55.1            55.7               0.6    0.77

0.00          0.38

HW

99.1            99.7               0.6    0.15      0.12          0.07

HW

131.4           132.4               1.1    0.18      0.13          0.03

HW

144.8           145.4               0.6    0.22      0.27          0.20

Ram

146.0           146.5               0.5    0.49      0.64          0.28

2.3m (at) 0.51% Co,

Ram

146.5           147.0               0.6    0.79      1.40          0.50

0.64% Cu,0.30g/t Au

Ram

147.0           147.5               0.5    0.50      0.14          0.41

Ram

147.5           148.3               0.8    0.31      0.40          0.08

Ram

R19-11          32.1            32.6               0.5    0.21

0.15          0.54

HW

99.0            99.7               0.7    0.27      0.08          0.42

3.7m (at) 1.07% Co,

Ram

99.7           100.6               0.9    0.27      0.05          0.22

0.13% Cu, 0.69g/t Au

Ram

100.6           101.5               0.9    0.42      0.25          0.52

Ram

101.5           102.1               0.6    2.83      0.21          1.47

Ram

102.1           102.7               0.6    2.35      0.05          1.15

Ram

R19-12         105.2           105.8               0.6    0.31

0.43          0.26

2.4m (at) 0.57%Co,

Ram

105.8           106.4               0.6    1.13      0.14          1.05

0.26%Cu, 0.47g/tAu

Ram

106.4           107.0               0.6    0.67      0.35          0.39

Ram

107.0           107.6               0.6    0.16      0.15          0.18

Ram

R19-13          64.0            64.9               0.9    0.20

0.02          0.10

HW

103.9           104.5               0.6    0.48      0.09          0.37

Ram

104.5           105.2               0.6    2.41      0.15          1.46

1.8m (at) 1.06% Co,

Ram

105.2           105.8               0.6    0.28      0.08          0.23

0.1% Cu, 0.69g/t Au

Ram

106.4           107.0               0.6    0.15      0.23          0.48

Ram

R19-14           0.0             0.0               0.0    0.00

0          0.00

No Significant Assays

R19-15          64.6            65.2               0.6    0.45

0.04          0.47

Ram

R19-16          23.5            24.4               0.9    0.56

0.02          0.21

HW

81.7            82.4               0.7    0.23      0.10          0.35

Ram

R19-17           8.8             9.4               0.6    0.21

0.04          2.18

Ram

R19-18

No Significant Assays

R19-19

No Significant Assays

---

Notes:                  1. Core holes selectively

sampled.

2. Whole core Met - Whole core

ore composites consumed by

metallurgical testwork, no

assays.

3. Ram (equal sign) Main Ram Zone, HW (equal sign)

Hanging wall Zone, FW (equal sign)

Footwall Zone.

4. Previous results for holes

R19-01 and R19-02 reported 15

October 2019.

Holes R19-18 and R19-19 did not record significant cobalt as these holes

were specifically targeted for oxidized material for metallurgical tests as

part of the BFS, at very shallow depths - less than 20 metres below surface.

At these near surface depths, the ore zones are variably leached of cobalt and

copper but typically with gold remaining in the zones. These near surface

areas will form the future crown pillar of the mine and are not considered for

mining or inclusion in resources nor reserves. Hole R19-14 encountered a shear

at the expected Ram intercept resulting in oxidation and leaching of cobalt

and copper.

Overall the drilling results are in line with expectations from

historical drilling and the previous modelled resource. The 2H 2019 drilling

by Jervois confirms the ICO as a high-grade primary cobalt deposit with

significant copper and gold credits. Information from this drilling program

will not only be used to update the resource model but also provides

information for the development of the integrated resource model with

structural and lithological controls which will aid with realistic resource

modelling and mine design.

Metallurgical samples were taken from all Ram intercepts and are

currently undergoing testwork at SGS Lakefield (Canada). Results from these

tests will be fed into the BFS update and also provide concentrate samples for

marketing under the split Cobalt and Copper concentrates regime currently in

place.

Confidence in the economic potential of the resource is growing as more

information is generated for the updated BFS on the ICO. This includes the

potential to ultimately operate at higher production rates than currently

formalised under existing feasibility studies in an environmentally

responsible manner. A domestic cobalt refinery within the United States will

also be positively impacted by a mining operation delivering higher volumes of

cobalt concentrate for a longer period.

Quality AssuranceCore samples or geochemical analysis are sent to ALS

Elko Nevada, an independent and fully accredited laboratory in the USA for

analysis for gold & multi-element Induction Coupled Plasma Spectroscopy. Core

samples from the main Ram zone are sent to SGS Lakefield, Canada for analysis

as per ALS Elko and for further metallurgical testing. Jervois also has a

regimented Quality Assurance, Quality Control program where at least 10%

duplicates and blanks are inserted into each sample shipment.

On behalf of the Board of Directors of Jervois,Bryce Crocker, CEO and

Director

Competent Person's StatementThe information in this release that relates

to Mineral Exploration is based on information compiled by David Selfe who is

full time employee of the company and a Fellow of the Australasian Institute

of Mining and Metallurgy and Dean Besserer, P.Geol. who is a full time

employee of the company and a member of The Association of Professional

Engineers and Geoscientists of Alberta. Both David Selfe and Dean Besserer

have sufficient experience which is relevant to the style of mineralization

and type of deposit under consideration and to the activity which they are

undertaking to qualify as a Competent Person as defined in the 2012 Edition of

the 'Australasian Code for Reporting of Exploration Results, Mineral Resources

and Ore Reserves'. David Selfe and Dean Besserer consent to the inclusion in

the release of the matters based on their information in the form and context

in which it appears.

Disclosure required for TSX-V Regulations

Qualified Person's Statement The technical content of this news release

has been reviewed and approved by Dean Besserer, P.Geol., General Manager -

Exploration for the Company and a Qualified Person as defined by National

Instrument 43-101

Neither TSX Venture Exchange nor its Regulation Services Provider (as

that term is defined in policies of the TSX Venture Exchange) accepts

responsibility for the adequacy or accuracy of this release.

This news release may contain certain "Forward-Looking Statements" within

the meaning of the United States Private Securities Litigation Reform Act of

1995 and applicable Canadian securities laws. When used in this news release,

the words "anticipate", "believe", "estimate", "expect", "target, "plan",

"forecast", "may", "schedule" and other similar words or expressions identify

forward-looking statements or information. These forward-looking statements or

information may relate to drilling and feasibility study outcomes undertaken

on the ICO, the reliability of third party information, and certain other

factors or information. Such statements represent the Company's current views

with respect to future events and are necessarily based upon a number of

assumptions and estimates that, while considered reasonable by the Company,

are inherently subject to significant business, economic, competitive,

political and social risks, contingencies and uncertainties. Many factors,

both known and unknown, could cause results, performance or achievements to be

materially different from the results, performance or achievements that are or

may be expressed or implied by such forward-looking statements. The Company

does not intend, and does not assume any obligation, to update these

forward-looking statements or information to reflect changes in assumptions or

changes in circumstances or any other events affections such statements and

information other than as required by applicable laws, rules and regulations.

JORC Code, 2012 Edition - Table 1

Section 1 Sampling Techniques and Data

Criteria

JORC Code explanation

Commentary

Sampling                                        Nature and quality of sampling

(eg cut channels, random

techniques                                      chips, or specific specialised

industry standard

measurement tools appropriate to the minerals under

investigation, such as down hole gamma sondes, or handheld

XRF instruments, etc). These examples should not be taken

as limiting the broad meaning of sampling.

Include reference to

measures taken to ensure sample representivity and the

appropriate calibration of any measurement tools or

systems used.

Aspects of the determination of mineralisation that

are Material to the Public Report.

In cases where

'industry standard' work has been done this would be

relatively simple (eg 'reverse circulation drilling was

used to obtain 1 m samples from which 3 kg was pulverised

to produce a 30 g charge for fire assay'). In other cases

more explanation may be required, such as where there is

coarse gold that has inherent sampling problems. Unusual

commodities or mineralisation types (eg submarine nodules)

may warrant disclosure of detailed information.

Sampling is by diamond drill coring.

All drill core was sampled contingent on geology and core recovery:

Core was collected directly from the core barrel into core boxes, and Core

samples

were split in half, with the top half of the core analysed and other half

retained

as reference core in the tray. Core trays were clearly labelled with the hole

number, tray number and metre intervals marked.

Samples were cut along the orientation line before being correctly placed back

into

the tray. The half-core was sampled, ensuring that the same side is consistently

sampled, and placed into sample bags labelled with the assigned sample number.

Downhole measurements are recorded using a Reflex EZ-Trac at 50 foot intervals

downeach hole and at the end of every hole.

Field sampling followed Jervois protocols including industry standard quality

control

procedures.

Samples were sent to one of two laboratories: ALS Elko Geochemistry, an

independent

and fully accredited laboratory in Nevada, USA ("ALS") for analysis for gold and

multi-element Induction Coupled Plasma Spectroscopy ("ICP") or to SGS

Lakefield, an

independent and fully accredited laboratory in Ontario, Canada ("SGS") for

analysis

for gold and multi-element Induction Coupled Plasma Spectroscopy ("ICP").

Jervois

also has a regimented Quality Assurance, Quality Control ("QA/QC") program

where at

least 10% standards and blanks are inserted into each sample shipment.

Sample representivity is ensured by:

Diamond Core: For all drilling core was halved for sub?sampling with a diamond

saw.

For PQ, core was then quartered for sampling. Sample intervals range from 2

feet to

5 feet in length, with majority of samples assayed over 3 feet intervals.

Handheld XRF instruments were used to spot check drill core for mineralization,

however those results were not relied on.  All sample results reported on are

from

ALS or SGS.

All of the drilling was diamond drill core (PQ/HQ).  Typically, drill core was

sampled on nominal 3 foot half core samples for HQ and quarter core for PQ.

All sample analyses were completed at ALS or SGS.  ALS and SGS are global

independent

laboratories which are ISO accredited.

Samples are received at the laboratory:  Bar codes are scanned and logged;

samples

are weighed and dried; samples are crushed to 70% less than 2mm, riffle split

off

250g, pulverize split to better than 85% passing 75 microns; all samples are

analyzed for 35 elements using ICP-AES and gold using 30 gram Fire assay for

core,

both with an AA finish.  Any samples with over-limits specific to base metals or

gold are re-analyzed.

---

Drilling                                        Drill type (eg core, reverse

circulation,

open-hole                                                               PQ

coring was completed up to the limit of PQ penetration and then reduced to HQ to

techniques                                      hammer, rotary air blast,

auger, Bangka, sonic, etc)

and                                                          the end of hole

where appropriate.

details (eg core diameter, triple or standard tube, depth

of diamond tails, face-sampling bit or other type,

whether core is oriented and if so, by what method, etc).

Holes were generally angled from -55 to -90 degrees at varying azimuths.

Drill sample                                    Method of recording and

assessing core and chip

sample                                                            All holes are

teched and all intervals are measured for recovery and RQD's are

recovery                                        recoveries and results

assessed.

calculated.  Recovery % recorded in the geotechnical records as

equivalent to the

Measures taken to

maximise                                                            length of

core recovered, as a percentage of the drill run.

sample recovery and ensure representative nature of the

samples.

Whether a relationship exists between sample recovery

and grade and whether sample bias may have occurred due

to preferential loss/gain of fine/coarse material.

Excellent recoveries were obtained from Diamond drilling.

There is no bias noted between sample recovery and grade. Excellent recoveries

were

obtained from Diamond drilling other than in faulted zones which were not

sampled.

---

Logging                                         Whether core and chip samples

have been geologically and

geotechnically logged to a level of detail to support

appropriate Mineral Resource estimation, mining studies and

metallurgical studies.

Whether logging is qualitative or

quantitative in nature. Core (or costean, channel, etc)

photography.

The total length and percentage of the relevant

intersections logged.

Diamond drilling:

Drill core is photographed and logged prior to sampling;

Core has been geologically and geotechnically logged to a level of detail

appropriate

to support mineral resource estimation and mining studies.

Logging has been conducted both qualitatively and quantitatively; full

description of

lithologies, alteration and comments are noted, as well as percentage estimates

on

veining and sulphides.

The total length of all holes drilled in 2019 was 3126m. All depths are listed

in the

table in the body of the text. All drill holes are logged in their entirety.

Sub-sampling                                    If core, whether cut or sawn

and whether quarter, half

or                                                         All core was

half-cut lengthwise using a diamond saw.  The HQ core half-core was

techniques and                                  all core

taken.

sampled. For PQ core half was cut again and quarter

core was sampled. For holes R19-

sample                                                  If non-core, whether

riffled, tube sampled,

rotary                                                        04 and R19-06,

whole core for select intervals was crushed and used for

preparation                                             split, etc and whether

sampled wet or

dry.

Metallurgical testwork. These intervals are not reported as the method requires

Head

For all                                                           grade

calculation.

sample types, the nature, quality and appropriateness of

the sample preparation technique.

Quality control procedures

adopted for all sub-sampling stages to maximise

representivity of samples.

Measures taken to ensure that the

sampling is representative of the in situ material

collected, including for instance results for fieldduplicate/second-half

sampling.

Whether sample sizes

are appropriate to the grain size of the material being

sampled.

Samples are received at the laboratory:  Bar codes are scanned and logged;

samples

are weighed and dried; samples are crushed to 70% less than 2mm, riffle split

off

250g, pulverize split to better than 85% passing 75 microns; all samples are

analyzed for 35 elements using ICP-AES and gold using 30 gram Fire assay for

core,

both with an AA finish.  Any samples with over-limits specific to base metals or

gold are re-analyzed.

For core sampling the same side is consistently sampled, half-core is retained

in

the tray for HQ for PQ quarter core is retained and half core is reserved for

metallurgical test sample. The assay sub- sample is placed into sample bags

labelled with the assigned sample number.

One in 20 samples is duplicated where the core is quartered and a quarter cut

sample

is analysed as a duplicate.  The remaining quarter samples is retained in the

tray.

Sample sizes of 2-3 kg are appropriate for the grain size of material. The

sample

preparation technique and sample sizes are considered appropriate to the

material

being sampled.

---

Quality of assay                                The nature, quality and

appropriateness of the assaying and

data and                                        laboratory procedures used and

whether the technique is

laboratory tests                                considered partial or total.

For geophysical tools, spectrometers,

handheld XRF instruments, etc, the parameters used in

determining the analysis including instrument make and

model, reading times, calibrations factors applied and

their derivation, etc.

Nature of quality control

procedures adopted (eg standards, blanks, duplicates,

external laboratory checks) and whether acceptable levels

of accuracy (ie lack of bias) and precision have been

established.

The ICP-AES and Fire Assay (30 gram) are

considered total and are high quality.

Jervois has a regimented Quality Control protocol which has consisted of

systematic

submission of blanks, standards and duplicates in addition to those conducted

at the

laboratory.

Precision levels for all blanks, standards and duplicate samples fell within

acceptable ranges.

Verification of                                 The verification of significant

intersections by either

sampling and                                    independent or alternative

company personnel.

assaying

The use of twinne

d

holes.

Documentation of primary data, data entry procedures, data

verification, data storage (physical and electronic)

protocols.

Discuss any adjustment to assay

data.

Significant intersections are alternatively verified by the CP and QP of the

company.

No holes have been twinned in this drill programme.

Data is collected using a customized template on MS Excel.  The data is backed

up

systematically on and off site as well as on the cloud. As well, data is

recorded

using a master Microsoft Office Excel spreadsheet.

All data below detection limit have been entered as zero.

Samples received damaged at the laboratory, or with insufficient sample weight

for

analysis had the interval or location left blank.

---

Location of data                                Accuracy and quality of surveys

used to locate drill

holes                                                        All collars were

surveyed by licensed surveyors. Down-hole surveys were routinely

points                                          (collar and down-hole surveys),

trenches, mine

workings                                                           carried out

on all holes using a Reflex EZ-Trac at 50 foot intervals downhole.

and other locations used in Mineral Resource

estimation.                                                          Holes were

setup on collar using a Reflex TN14 Gyro.

Specification of the grid system used.

Quality and adequacy of

topographic control.

All datum is collected and recorded in UTM_NAD83 Z11N

The 3D location of the individual samples is considered to be adequately

established,

consistent with accepted industry standards.

Data spacing and                                Data spacing for reporting of

Exploration

Results.                                                                To

date, due to the exploratory nature of the footwall drilling, the spacing is

wide.

distribution

Whether the                                                            Assay

results for only one hole and for only the footwall intercept have been

data spacing and distribution is sufficient to establish

received. Further intervals have been sampled but the assays are not yet

available.

the degree of geological and grade continuity appropriate

for the Mineral Resource and Ore Reserve estimation

procedure(s) and classifications applied.

Whether sample

compositing has been applied.

The intervals reported for these 2 holes is exploratory and data spacing would

not be

considered sufficient to establish a Mineral Resource or Ore Reserve Estimation

for

this particular mineralisation intercept.

Data is reported both composited and uncomposited.

---

Orientation of                                  Whether the orientation of

sampling achieves

unbiased                                                             Drilling

sections are orientated perpendicular to the strike of the host rocks.

data in relation                                sampling of possible structures

and the extent to

which                                                           Drill holes

were inclined between 55 and 90 to optimize intercepts of

to geological                                   this is known, considering the

deposit

type.

mineralisation with respect to thickness and distribution.

structure

If the relationship

between the drilling orientation and the orientation of keymineralised

structures is considered to have introduced a

sampling bias, this should be assessed and reported if

material.

Drilling with angled holes in most instances provides a representative sample

across

the stratigraphy.

Sample security                                 The measures taken to ensure

sample

security.

All individual samples are bagged and sealed with a zip tie.  Then individual

samples

are bagged in poly woven sacks and sealed with a zip tie. The samples are sent

by

courier to the lab and tracked. To date, no sample shipments have had reported

problems and/or a breach in security.

---

Audits or reviews                               The results of any audits or

reviews of

sampling

Jervois protocols consist of a regimented internal QA/QC which match or exceed

techniques and

data.

global industry standards.  This drilling program has not

been subjected to audits

or reviews so far, however this will be completed as part of the current

feasibility

study of which this drilling is a part.

---

Section 2 Reporting of Exploration Results

Criteria

JORC Code explanation

Commentary

Mineral tenement and                                 Type, reference

name/number, location and ownership

including                                                          Idaho Cobalt

Operations consists of 243

land tenure status                                   agreements or material

issues with third parties such as

joint                                                         unpatented

mineral claims totalling 5990

ventures, partnerships, overriding royalties, native

title                                                             acres.  The

claims are 100% owned by Jervois

interests, historical sites, wilderness or national park

and                                                           subsidiary

Formation Capital LLC and are in

environmental

settings.

good standing.

The security of the tenure held at the time of

reporting along with any known impediments to obtaining a

licence to operate in the area.

Unpatented Mineral Claims:

Ownership of unpatented mining claims in the

U.S. is in the name of the holder, with

ownership of the minerals belonging to the

United States of America, under the

administration of the U.S. Bureau of Land

Management. Under the Mining Law of 1872,

which governs the location of unpatented

mining claims on federal lands, the locator

has the right to explore, develop and mine

minerals on unpatented mining claims without

payments of production royalties to the

federal government.  Annual claim maintenance

and filing fees paid before September 1st each

year are the only federal encumbrances to

unpatented mining claims. Exploration plans

are permitted and administered by the Unites

States Forestry Service.

The United States Department of Agriculture

Salmon Challis National Forest (the Forest

Service) issued a revised Record of Decision

(the ROD) for the ICO in January 2009. The ROD

described the decision to approve a Mine Plan

of Operations (MPO) for mining, milling and

concentrating mineralized material from the

ICO. The ROD was subsequently affirmed by the

Forest Service in April 2009. The Plan of

Operations at the ICO mine and mill remained

unchanged and the ROD remains in place. In

December 2009, the Forest Service approved the

MPO allowing for the commencement of ICO

construction.

There are no known encumbrances.

---

Exploration done by                                  Acknowledgment and

appraisal of exploration by other

parties.                                                          The ICO came

under Jervois management following

other

parties

the merger with eCobalt in 2019. Prior to this

merger, the area has a long history of copper

and cobalt exploration and mining. Copper

mineralization in the Blackbird Creek area was

discovered in 1892, and the area was soon

explored as both a copper and gold prospect.

The area was first mined by Union Carbide at

the Haynes-Stellite Mine located south of the

present ICO claim block, during World War I.

Union Carbide mined approximately 4,000 tons

of cobalt-bearing ore before ceasing

operations. From 1938 to 1941, the Uncle Sam

Mining and Milling Company operated a mine at

the south end of the present Blackbird mine

and reportedly mined about 3,600 tons of ore.

Calera Mining Company, a division of Howe Sound

Company, developed and mined the Blackbird

deposit between 1943 and 1959 under a contract

to supply cobalt to the U.S. government.

Calera stopped mining when the government

contract was terminated in 1960.

Machinery Center Inc. mined from the district

between 1963 and 1966, when Idaho Mining

Company (owned by Hanna Mining Company)

purchased the property. Noranda optioned the

property from Hanna in 1977 and carried out

extensive exploration, mine rehabilitation andmetallurgical testing. In 1979

Noranda and

Hanna formed the Blackbird Mining Company

(BMC) to develop the property. BMC completed

an internal feasibility study of their

property at the time, including material from

the Sunshine deposit in 1982. BMC allowed

perimeter claims to lapse in 1994, and eCobalt

restaked much of that ground. From 1995 to the

present, eCobalt completed surface geochemical

sampling and drilled 158 diamond drill holes

on the ICO ground.

Geology                                              Deposit type, geological

setting and style of mineralisation.

Deposit Types:

Whilst the deposits in the Idaho Cobalt Belt

have been studied over many years the deposit

types are still a subject of debate. Prior to

2005 the overriding opinion was that the

deposits are sedimentary exhalative and are

referred to as the Blackbird Sediment Hosted

Cu-Co. And have been described as stratabound

iron-, cobalt-, copper-, and arsenic-rich

sulphide mineral accumulations in nearly

carbonate-free argillite/siltite couplets

and quartzites

Post 2005 the discovery of high concentrations

of rare earth elements (REE) lead to the

postulation that the deposits are not

volcanogenic massive sulphide or sedimentary

exhalative deposits but instead are iron

oxide-copper-gold (IOCG) deposits

Geological Setting:

The ICO is situated in the Idaho Cobalt Belt, a

30- to 35-mile long metallogenic district

characterized by stratiform/tabular copper-

cobalt deposits. The deposits are hosted by a

thick, dominantly clastic sequence of Middle

Proterozoic age sandwiched between late

Proterozoic quartz monzonitic intrusions. The

clastic sediments were deposited in a large

fault-bounded basin, probably as large

submarine fan complexes and/or deltaic aprons

that were frequently "drowned" by continuing

subsidence within the basin. All significant

copper-cobalt deposits and occurrences are

found in the Proterozoic Apple Creek

Formation, which constitutes the base of this

sequence. This formation was originally

correlated with Pritchard Formation

metasediments of the Belt supergroup to the

north, its age being constrained by dates of

1.37 Ga for adamellites intruding the sequence

and 1.7 Ga from mafic dykes and sills emplaced

along the basin margin faults.

The structure of the Apple Creek Formation is

dominated by the regional rift structure.

Cobalt-copper-gold mineralization occurs

along a northwest-southeast trending

structure parallel to and west of the central

axis of the rift.

There is a series of northerly trending faults

that are considered to represent initial

growth faults, reactivated by Laramide and

younger events. The district has also been

affected by north-easterly structures of the

Trans-Challis Fault Zone.

The ICO is hosted in Proterozoic age meta-

sediments found on the east side of the

central Idaho Batholith comprising granitic-

to-granodioritic rocks. The Idaho Cobalt Belt

represents a distinct district dominated bystratabound cobalt + copper +/- gold

mineralization, with a remobilized

constituent. The district is underlain by

strata of the middle Proterozoic-age Apple

Creek Formation, which is an upward-

thickening, upward-coarsening clastic

sequence at least 49,000 feet thick that

represents a major basin-filling episode and

was formerly considered part of the Yellow

Jacket Formation.

The Apple Creek can be divided into three

units. The lower unit of the Apple Creek

Formation is over 15,000 feet thick and

consists mainly of argillite and siltite, with

lesser occurrences of fine-grained quartzite

and carbonates. Graded bedding and planar to

wavy laminae are common in the lower unit,

which is locally metamorphosed to phyllite.

The middle unit of the Apple Creek Formation

is up to 3,600 feet thick and comprises

several upward-coarsening sequences of

argillite, siltite, and quartzite, with

distinctive biotite-rich interbeds that

generally have a direct correlation to

mineralization. The middle unit hosts the

majority of the known cobalt, copper and gold

occurrences in the Idaho Cobalt Belt. The

upper unit exceeds 9,800 feet in thickness and

is predominantly composed of thin- to thick

bedded, very fine- to fine-grained

quartzite.

Mafic tuffs within the Apple Creek Formation

are the oldest igneous rocks exposed in the

Sunshine-Blackpine district. They are

accompanied by felsic tuffs and carbonatitic

tuffs. Some mafic dikes and sills intrude the

Apple Creek Formation and may be comagmatic

with the mafic tuff beds. Several small

lamproitic diatremes may also be coeval with

mafic volcanism.

The Apple Creek Formation has undergone varying

degrees of regional metamorphism, ranging from

greenschist facies in the southern part of the

district to amphibolite grade facies in the

northern part of the district. Several types

of mafic dikes and sills, ranging from 3 ft.

to 100 ft. thick, intrude the Apple Creek

Formation and are interpreted as feeders to

the exhalative mafic tuffs, which are most

abundant in areas of intrusive actvity

Style of Mineralization:

Mineralization at the ICO is characterized as

syngenetic, stratiform/tabular exhalative

deposits within, or closely associated with,

the mafic sequences of the Apple Creek

Formation. This mineralization is dominantly

bedding concordant and the deposits range from

nearly massive to disseminated. Some

crosscutting mineralization is present that

may be in feeder zones to the stratiform

mineralization or may be due to remobilization

locally into fracture quartz veins and/or

crosscutting structures.

Dominant minerals include cobaltite (CoAsS) and

chalcopyrite (CuFeS2), with lesser, variable

occurrences of gold. Other minerals present in

small quantities are pyrite (FeS2), pyrrhotite

(FeS), arsenopyrite (FeAsS), linnaeite ((CoNi)3S4), loellingite (FeAs2),

safflorite

(CoFeAs2), enargite (Cu3AsS4) and marcasite

(FeS2).

Recently, rare-earth minerals have been

identified in samples from the deposit as

monazite, xenotime and allanite. At this time,

these minerals have not been considered for

potential recovery as by-products of the Co-

(Cu-Au).

The Ram is the largest and best-known deposit

in the ICO area. It consists of a Hanging-

wall Zone with 3 primary and 4 minor horizons,

a Main Zone comprising 3 horizons, and a

Footwall Zone with 3 horizons. These sub-

parallel horizons generally strike N15oW and

dip 50o - 60o to the northeast. Most of the

significant Co mineralization is associated

with exhalative lithologies i.e. biotitic

tuffaceous exhalate (BTE), siliceous

tuffaceous exhalate (STE), and quartzite with

impregnations of biotitic tuffaceous exhalate

(QTZ/BTE) or siliceous tuffaceous exhalate

(QTZ/STE).

---

Drill hole Information                               A summary of all

information material to the understanding

of                                                          All Material drill

holes relating to this

the exploration results including a tabulation of the

following                                                        report are

detailed in the tables included in

information for all Material drill

holes:

the body text of the report.

easting and northing of the drill hole collar

elevation or RL (Reduced Level - elevation above sea level in

metres) of the drill hole collar

dip and azimuth of

the hole

down hole length and interception depth

hole length.

If the exclusion of this

information is justified on the basis that the information is

not Material and this exclusion does not detract from the

understanding of the report, the Competent Person should

clearly explain why this is the case.

Data aggregation                                     In reporting Exploration

Results, weighting

averaging                                                                  For

this report weighted averaging has been

methods                                              techniques, maximum and/or

minimum grade truncations

(eg                                                               used in

reported composite intervals and

cutting of high grades) and cut-off grades are

usually

individual results are also listed, no grade

Material and should be

stated.

truncations etc. has been used.

Where aggregate intercepts incorporate short

lengths of high grade results and longer lengths of low grade

results, the procedure used for such aggregation should be

stated and some typical examples of such aggregations should be

shown in detail

The assumptions used for any reporting of metal equivalent

values should be clearly stated.

Aggregate intercepts are reported using a grade

metre calculation.  For example: ((assay x

meter interval sampled) + (assay x meter

interval sampled) + (assay x meter interval

sampled) /divided by total number of meters

in the interval). Individual sample intercepts

are also shown.

No metal equivalent values have been reported.

---

Relationship between                                 These relationships are

particularly important in the

reporting                                                        As the exact

orientation of the Blacktail North

mineralisation widths                                of Exploration

Results.

mineralisation is not known, the geometry of

and intercept lengths                                             If the

geometry of the mineralisation

with                                                                the

mineralisation with respect to the drill

respect to the drill hole angle is known, its nature should

be                                            hole angle is not precisely known

but is

reported.

assumed to be similar to, if not

parallel to

If it is not known and only the down hole lengths

are                                                                  that of

the known Ram deposit. Ram deposit and

reported, there should be a clear statement to this effect

(eg                                                         Hanging-wall zones

have a well-defined

'down hole length, true width not

known').

geometry from past drilling. Drilling

orientation is targeted to intercept

perpendicular to the mineralized zones and is

therefore approximates true width.

Notwithstanding that, true width is not

reported and only down hole length is

reported.

Diagrams                                             Appropriate maps and

sections (with scales) and tabulations

of                                                         Plan and cross

sections included in the body of

intercepts should be included for any significant

discovery                                                            the report.

being reported These should include, but not be limited to a

plan view of drill hole collar locations and appropriate

sectional views.

---

Balanced reporting                                   Where comprehensive

reporting of all Exploration Results is

not                                                        Assay returns are

complete. The assay summarypracticable, representative reporting of both low

and

high                                                             table lists

all significant assays above 0.2%

grades and/or widths should be practiced to avoid

misleading                                                           Co which

is at the bottom end of low grade

reporting of Exploration

Results.

material as defined by previous economic

studies. Where returned grades for an entire

hole were below 0.2% Co, "No Significant

Assays" is stated.

Other substantive                                    Other exploration data, if

meaningful and material, should

be                                                          Table 1a summarizes

other exploration data.  It

exploration data                                     reported including (but

not limited to):

geological

includes: ground geophysics (magnetics, VLF

observations; geophysical survey results; geochemical

survey                                                           TEM);

geochemical surveys (soil sampling);

results; bulk samples - size and method of

treatment;

rock grab sampling; and trenching.

metallurgical test results; bulk density, groundwater,

geotechnical and rock characteristics; potential deleterious

or contaminating substances.

---

Further work                                         The nature and scale of

planned further work (eg tests

for                                                             Planning for

further surface exploration

lateral extensions or depth extensions or large-scale

step-                                                            drilling will

commence in early 2020. A

out

drilling).

digital geological and structural model

isDiagrams clearly highlighting the areas of

possible                                                                 under

construction incorporating the 2019

extensions, including the main geological interpretations

and                                                       drilling data. An

update to the Resource model

future drilling areas, provided this information is

not                                                                will be

completed incorporating the 2019

commercially

sensitive.

drilling data and geological-structural model

to support an ongoing feasibility study.

---

TABLE 1a - OTHER EXPLORATION DATA

Data Type                     Summary of Results

---

Airborne Geophysics

Not Applicable

---

Ground Geophysics             A VLF-EM and Ground Magnetics survey

was completed in 1996 over the ICO

claims. The results were weak and

inconclusive. No follow-up work was

recommended.

---

Geochemical Surveys           In 1995, soil sampling of selected

areas was conducted on lines spaced

200 ft. and 400 ft. apart, with

samples collected at intervals of

100 ft. along the lines.

In 1996, the soil grid was extended

north and soil samples were

collected on lines spaced 200 ft.

apart with samples collected at 25

ft. intervals along the lines.

Some infill samples were collected

from the 1995 soil grid. Other parts

of the grid were also extended and

sampled on 25 ft. intervals where it

was deemed warranted.

A total of 8,427 soil samples were

collected during the 1995/1996

campaign. A further 95 soil samples

were taken in 1997.

Other exploration activities

conducted during 1995/1996 include

surface geological mapping at a

scale of 1 in. to 100 ft.

---

Trenching                     In 1995/1996 mapping of old trenches

and prospect pits, and collection of

979 surface rock samples including

those from trenches. In 1997 three

trenches, 623 ft. long in aggregate,

were excavated within the "prism" of

the road; the trenches were mapped

and 83 rock samples were collected.

---

Geochemical Survey Results    The 1995 soil sampling program lead

to the discovery of the southern end

of the Ram target.

---

View original content to download

multimedia:http://www.prnewswire.com/news-releases/jervois-drilling-results-fr

om-idaho-cobalt-operations-united-states-300967217.html

SOURCE Jervois Mining Limited

View original content to download multimedia:

http://www.newswire.ca/en/releases/archive/December2019/02/c0192.html

-0-                           12/02/2019

/For further information: Investors and analysts:  Simon Clarke, EGM

Corporate Affairs, sclarke(at)jervoismining.com, Cell: +1 604 551 9665; Media:

Nathan Ryan, NWR Communications, nathan.ryan(at)nwrcommunications.com.au, Cell:

+61 420 582 887/

/Web Site: https://jervoismining.com.au/ /

(JRVMF JRV. JRV IHS)

CO:  Jervois Mining Limited

ST:  Australia

IN:  MNG OTC