Source	CANADA NEWSWIRE

Date	12/09/2019

Time	04:04:01 AM

Company	 Noble Mineral Exploration Inc.

Title	Noble Announces Canada Nickel Completes Inaugural Drill Program - All 9

holes Intersect Nickel-Cobalt-PGM Mineralization >330 metres Across 1.4

Kilometre Strike Length

CDNX Symbol: NOB

Press Release

" BC-NOB-CanadaNickelDrill   12-09 2125

#News release via Canada NewsWire, Toronto 416-863-9350 -ME-

Attention co.173022:

^Noble Announces Canada Nickel Completes Inaugural Drill Program - All 9

holes Intersect Nickel-Cobalt-PGM Mineralization >330 metres Across 1.4

Kilometre Strike Length@

TSX.V: NOB FWB: NB7 OTC.PK: NLPXF

TORONTO, Dec. 9, 2019 /CNW/ - Noble Mineral Exploration Inc. (TSX-V: NOB)

("Noble") is pleased to announce that it has been advised by Canada Nickel

Company Inc. ("Canada Nickel" or the "Company") of the assay results from the

first nine diamond drill holes (5,267m) from the Company's 2019 exploration

campaign at the Crawford nickel-cobalt project located just north of Timmins,

Ontario.

<<

--  All 9 holes drilled as part of its inaugural 2019 drill campaign have

intersected nickel-cobalt-PGM mineralization in excess of 330 metres

o 4 holes are continuously mineralized in excess of 500 metres and 4 of 9

holes ended in mineralization

--  Mineralization now defined over 1.4 kilometres by 350 to 600 metres wide

to a depth of 500 metres - Remains open along strike, at depth and to

the north

--  Higher grade area (>0.3% nickel) defined on northern side of structure

across a 1 kilometre strike length with intersections of 126 to 260

metres containing an incrementally higher-grade zone (>0.35% nickel)

with intersections of 35 to 134 metres. Remains open to the north, west,

and at depth.

o Hole CR19-11 intersected 390 m of 0.271% nickel, 0.014% cobalt, including

134 m of 0.357% nickel, 0.015% cobalt, and 0.09 g/t Pd+Pt

--  Second drill mobilized to accelerate drilling for initial resource and

begin drilling additional targets. Mineralogy and metallurgical testing

to be accelerated.

<>>

Mark Selby, Chairman and CEO of Canada Nickel, stated, "This year's

drilling highlights the potential of the Crawford nickel-cobalt project as a

new, large scale nickel-cobalt sulphide discovery with mineralization now

defined over an area 1.4 kilometres long by 350 to 600 metres wide and up to

500 metres deep and remaining open in multiple directions. Unlike some other

projects of its type, this drilling has also highlighted at surface a wide,

higher grade (>0.35%) area over a kilometre of strike length. With its

potential scale, the project's excellent location and infrastructure, our

team's significant experience and insights from advancing other large scale

ultramafic projects, and a robust nickel market increasingly driven by demand

from the electric vehicle segment, we are well positioned to unlock Crawford's

potential as a strategic, geopolitically low risk source of nickel and

cobalt."

Vance White, President and CEO stated "I am very pleased with the results

of Canada Nickel's inaugural drilling campaign. Subject to shareholder and

regulatory approval, investors who acquire Noble shares will be entitled to

receive their pro rata share of 10,000,000 shares (~18% of Canada Nickel) to

be distributed to Noble shareholders under a plan of arrangement that is

proposed for approval at a special meeting of Noble shareholders to be held on

December 27, 2019."

Based on Noble's current issued and outstanding share count and assuming

certain share issuances relating to the plan of arrangement are completed, for

each Noble share held, a shareholder of Noble would be eligible to receive

approximately 0.07108 of a Canada Nickel share. Subject to shareholder and

regulatory approval, it is expected that the consolidation of the Crawford

Nickel-Sulphide Project will be completed in January 2020, and Canada Nickel

has applied to trade on the TSX Venture Exchange ("TSXV") under the symbol

"CNC".

Crawford Nickel-Cobalt Sulphide Project - 2019 Drilling

Drilling in 2019 began in September to follow up on the initial

four-hole, 1,818-metres drilling campaign reported in early 2019 which

intersected multi-hundred metre intervals of serpentinized dunite with two of

the four holes ending in mineralization. (See Noble release dated March 4,

2019). The focus of this initial phase of drilling in 2019 was to extend the

mineralization along strike and begin testing the northern and southern extent

of the mineralization. The 9 holes which have been completed to date have

successfully extended the structure to 1.4 kilometre strike length by 350 to

600 metres wide by up to 500 metres deep. (See Figure 1 below.) The

mineralization remains open along strike, both north and south, and at depth.

The nearly kilometre deep anomaly is only partially tested at depth (see

Figure 2 below). The results of this drilling are summarized in Tables 1a and

1b below.

Figure 1 - Crawford Nickel-Cobalt Project - Plan View of 2018 and 2019

Drilling

Total magnetic intensity colored with a linear color transform from low

magnetic field (blue) to high (red)

Figure 2 - Crawford Nickel Cobalt Project - Section View of CR18-04 and

CR19-12

Total magnetic intensity colored with a linear color transform from low

magnetic field (blue) to high (red)

Higher Grade Zone

The 2019 drilling has also highlighted a higher grade area (>0.3% nickel)

on the northern side of the structure across a 1 kilometre strike length with

intersections of 126 to 260 metres containing an incrementally higher grade

zone (>0.35% nickel) with intersections of 35 to 134 metres. This higher-grade

zone remains open to the north, west and at depth. (see Figure 3)

Figure 3 - Crawford Nickel-Cobalt Project - Plan View showing Higher

Grade Zone

Total magnetic intensity colored with a linear color transform from low

magnetic field (blue) to high (red)

Next Steps

A second drill has been mobilized to accelerate the drilling required for

the initial resource and allow additional targets to be tested. Samples to

begin comprehensive mineralogical and metallurgical work are being selected

and initial results are expected during Q1-2020.

Table 1a - 2019 Crawford Nickel-Cobalt Project (Drilling from North Side

from East to West)

Intersections from North Wall

---

DDH ID

From

To

Length

Ni

Co

Pd

Pt

S

---

(m)

(m)

(m)

(%)

(%)

(ppb)

(ppb)

(%)

CR19-11                48.0        438.0             390.0

0.271          0.014          28.1         11.4        0.19

---

includes        69.0        292.5             223.5

0.324          0.015          43.8         16.8        0.27

---

includes       144.0        277.5             133.5

0.357          0.015          61.9         23.4        0.32

---

CR19-10                55.5        388.5             333.0

0.277          0.013          25.5         10.3        0.34

---

includes        61.5        271.5             210.0

0.320          0.013          30.5         11.1        0.49

---

includes       208.5        243.0              34.5

0.355          0.015          37.0         13.2        1.18

---

CR19-09                55.5        513.0             457.5

0.254          0.013          19.7         10.0        0.08

---

includes        70.5        309.0             238.5

0.310          0.013          26.9         13.2        0.11

---

includes       192.0        265.5              73.5

0.365          0.014          47.0         10.5        0.17

---

CR19-08                36.0        592.5             556.5

0.251          0.013          20.2         11.0        0.06

---

includes        70.5        468.0             397.5

0.271          0.013          18.2         11.2        0.06

---

includes       165.0        348.0             183.0

0.319          0.012          33.4         14.6        0.11

---

includes       183.0        223.5              40.5

0.351          0.013          26.3         12.8        0.18

---

CR19-13               102.0        609.0             507.0

0.237          0.013          10.1          8.1        0.03

---

includes       300.0        528.0             228.0

0.276          0.013          19.4         13.0        0.04

---

includes       300.0        405.0             105.0

0.325          0.012          37.3         13.7        0.07

---

includes       304.5        343.5              39.0

0.351          0.012          26.3          9.5        0.10

---

Note that the intersections are referenced to depth downhole and do not

necessarily represent a true width. Canada Nickel has insufficient information

to determine the attitude, either of the ultramafic body or of mineralized

zones within it at this point in the exploration program. True widths will be

less than the core lengths by unknown factors but based on the orientation of

the drillholes are estimated to be 65% of the length of the mineral

intersections.

Table 1b - 2019 Crawford Nickel-Cobalt Project (Drilling from South Side

from East to West)

Intersections from South Wall

---

DDH ID

From

To

Length

Ni

Co

Pd

Pt

S

---

(m)

(m)

(m)

(%)

(%)

(ppb)

(ppb)

(%)

CR19-06            207.0       576.0            369.0

0.229          0.011          5.3          3.5        0.04

---

includes      297.0       463.5            166.5

0.272          0.012          2.9          1.2        0.03

---

CR19-05             51.6       582.0            530.4

0.227          0.013         18.0          7.9        0.13

---

includes      432.0       481.5             49.5

0.310          0.015         36.7         10.4        0.53

---

includes      445.5       472.5             27.0

0.359          0.018         50.8         14.8        0.78

---

CR19-12             57.0       571.5            514.5

0.210          0.013         17.1         17.3        0.06

---

includes       57.0       279.0            222.0

0.293          0.012         13.4          3.8        0.09

---

includes       61.5       157.5             96.0

0.310          0.013         17.0          4.8        0.16

---

includes       72.0        88.5             16.5

0.365          0.014         18.7          4.1        0.24

---

CR19-07            204.0       619.5            415.5

0.221          0.013          7.2          7.5        0.01

---

includes      591.0       619.5             28.5

0.265          0.013         91.8         34.2        0.04

---

Note that the intersections are referenced to depth downhole and do not

necessarily represent a true width. Canada Nickel has insufficient information

to determine the attitude, either of the ultramafic body or of mineralized

zones within it at this point in the exploration program. True widths will be

less than the core lengths by unknown factors but based on the orientation of

the drillholes are estimated to be 65% of the length of the mineral

intersections.

Quality Assurance (QA) and Quality Control (QC)

William E. MacRae, MSc, P.Geo., a Qualified Person as defined by NI

43-101, is responsible for the on-going drilling and sampling program,

including quality assurance (QA) and quality control (QC). The core is

collected from the drill in sealed core trays and transported to the core

logging facility. The core is marked and sampled at 1.5 metre lengths and cut

with a diamond blade saw. Samples are bagged with QA/QC samples inserted in

batches of 35 samples per lot. Samples are transported in secure bags directly

from the Canada Nickel core shack Actlabs Timmins, an ISO/IEC 17025 accredited

lab. Analysis for precious metals (gold, platinum and palladium) are completed

by Fire Assay while analysis for nickel, cobalt, sulphur and 17 other elements

are performed using a peroxide fusion and ICP-OES analysis. Certified

standards and blanks are inserted at a rate of one QA/QC sample per 32 core

samples making a batch of 35 samples that are submitted for analysis.

Qualified Person

Stephen J. Balch P.Geo. (ON), VP Exploration of Canada Nickel and a

"qualified person" as such term is defined by National Instrument 43-101, has

verified the data disclosed in this news release, and has otherwise reviewed

and approved the technical information in this news release on behalf of

Canada Nickel Company Inc.

About Canada Nickel Company

Canada Nickel Company Inc. is a Canadian-based company focused on the

acquisition, exploration, development and eventual production of Nickel

assets. Canada Nickel is currently exploring its 100% owned flagship Crawford

Nickel-Cobalt project in the heart of the prolific Timmins-Cochrane mining

camp. The Crawford Project comprises 650 hectares with approximately 17,700

meters drilled to date, with identified Nickel and Cobalt bearing minerals

similar to that at the Dumont Nickel Deposit (RNC Minerals property). Canada

Nickel is focused on proving up the large bulk tonnage opportunity atCrawford,

which provides investors with pure leverage to Nickel, unencumbered

by earn-in agreements, in a geopolitically stable jurisdiction.

Cautionary Statement Concerning Forward-Looking Statements

This press release contains certain information that may constitute

"forward-looking information" under applicable Canadian securities

legislation. Forward looking information includes, but is not limited to,

timing for completion of the proposed transaction, the amount and uses of

proceeds, strategic plans, including future exploration and development

results, and corporate and technical objectives. Forward-looking information

is necessarily based upon a number of assumptions that, while considered

reasonable, are subject to known and unknown risks, uncertainties, and other

factors which may cause the actual results and future events to differ

materially from those expressed or implied by such forward-looking

information. Factors that could affect the outcome include, among others:

future prices and the supply of metals, the results of drilling, inability to

raise the money necessary to incur the expenditures required to retain and

advance the property, environmental liabilities (known and unknown), general

business, economic, competitive, political and social uncertainties, results

of exploration programs, risks of the mining industry, delays in obtaining

governmental approvals, and failure to obtain regulatory or shareholder

approvals. There can be no assurance that such information will prove to be

accurate, as actual results and future events could differ materially from

those anticipated in such information. Accordingly, readers should not place

undue reliance on forward-looking information. All forward-looking information

contained in this press release is given as of the date hereof and is based

upon the opinions and estimates of management and information available to

management as at the date hereof. Canada Nickel disclaims any intention or

obligation to update or revise any forward-looking information, whether as a

result of new information, future events or otherwise, except as required by

law.

Contacts (Canada Nickel)

Russell Starr, DirectorPhone: 647-669-9801email:

RussellStarr(at)canadanickel.com

Jessie Liu-Ernsting, VP Corporate Development and Investor

RelationsPhone: 416-809-9296 email: jessieliuernsting(at)canadanickel.com

About Noble Mineral Exploration Inc.

Noble Mineral Exploration Inc. is a Canadian-based junior exploration

company whic

" BC-NOB-CanadaNickelDrill   12-09 2125

#News release via Canada NewsWire, Toronto 416-863-9350 -ME-

Attention co.173022:

^Noble Announces Canada Nickel Completes Inaugural Drill Program - All 9

holes Intersect Nickel-Cobalt-PGM Mineralization >330 metres Across 1.4

Kilometre Strike Length@

TSX.V: NOB FWB: NB7 OTC.PK: NLPXF

TORONTO, Dec. 9, 2019 /CNW/ - Noble Mineral Exploration Inc. (TSX-V: NOB)

("Noble") is pleased to announce that it has been advised by Canada Nickel

Company Inc. ("Canada Nickel" or the "Company") of the assay results from the

first nine diamond drill holes (5,267m) from the Company's 2019 exploration

campaign at the Crawford nickel-cobalt project located just north of Timmins,

Ontario.

<<

--  All 9 holes drilled as part of its inaugural 2019 drill campaign have

intersected nickel-cobalt-PGM mineralization in excess of 330 metres

o 4 holes are continuously mineralized in excess of 500 metres and 4 of 9

holes ended in mineralization

--  Mineralization now defined over 1.4 kilometres by 350 to 600 metres wide

to a depth of 500 metres - Remains open along strike, at depth and to

the north

--  Higher grade area (>0.3% nickel) defined on northern side of structure

across a 1 kilometre strike length with intersections of 126 to 260

metres containing an incrementally higher-grade zone (>0.35% nickel)

with intersections of 35 to 134 metres. Remains open to the north, west,

and at depth.

o Hole CR19-11 intersected 390 m of 0.271% nickel, 0.014% cobalt, including

134 m of 0.357% nickel, 0.015% cobalt, and 0.09 g/t Pd+Pt

--  Second drill mobilized to accelerate drilling for initial resource and

begin drilling additional targets. Mineralogy and metallurgical testing

to be accelerated.

<>>

Mark Selby, Chairman and CEO of Canada Nickel, stated, "This year's

drilling highlights the potential of the Crawford nickel-cobalt project as a

new, large scale nickel-cobalt sulphide discovery with mineralization now

defined over an area 1.4 kilometres long by 350 to 600 metres wide and up to

500 metres deep and remaining open in multiple directions. Unlike some other

projects of its type, this drilling has also highlighted at surface a wide,

higher grade (>0.35%) area over a kilometre of strike length. With its

potential scale, the project's excellent location and infrastructure, our

team's significant experience and insights from advancing other large scale

ultramafic projects, and a robust nickel market increasingly driven by demand

from the electric vehicle segment, we are well positioned to unlock Crawford's

potential as a strategic, geopolitically low risk source of nickel and

cobalt."

Vance White, President and CEO stated "I am very pleased with the results

of Canada Nickel's inaugural drilling campaign. Subject to shareholder and

regulatory approval, investors who acquire Noble shares will be entitled to

receive their pro rata share of 10,000,000 shares (~18% of Canada Nickel) to

be distributed to Noble shareholders under a plan of arrangement that is

proposed for approval at a special meeting of Noble shareholders to be held on

December 27, 2019."

Based on Noble's current issued and outstanding share count and assuming

certain share issuances relating to the plan of arrangement are completed, for

each Noble share held, a shareholder of Noble would be eligible to receive

approximately 0.07108 of a Canada Nickel share. Subject to shareholder and

regulatory approval, it is expected that the consolidation of the Crawford

Nickel-Sulphide Project will be completed in January 2020, and Canada Nickel

has applied to trade on the TSX Venture Exchange ("TSXV") under the symbol

"CNC".

Crawford Nickel-Cobalt Sulphide Project - 2019 Drilling

Drilling in 2019 began in September to follow up on the initial

four-hole, 1,818-metres drilling campaign reported in early 2019 which

intersected multi-hundred metre intervals of serpentinized dunite with two of

the four holes ending in mineralization. (See Noble release dated March 4,

2019). The focus of this initial phase of drilling in 2019 was to extend the

mineralization along strike and begin testing the northern and southern extent

of the mineralization. The 9 holes which have been completed to date have

successfully extended the structure to 1.4 kilometre strike length by 350 to

600 metres wide by up to 500 metres deep. (See Figure 1 below.) The

mineralization remains open along strike, both north and south, and at depth.

The nearly kilometre deep anomaly is only partially tested at depth (see

Figure 2 below). The results of this drilling are summarized in Tables 1a and

1b below.

Figure 1 - Crawford Nickel-Cobalt Project - Plan View of 2018 and 2019

Drilling

Total magnetic intensity colored with a linear color transform from low

magnetic field (blue) to high (red)

Figure 2 - Crawford Nickel Cobalt Project - Section View of CR18-04 and

CR19-12

Total magnetic intensity colored with a linear color transform from low

magnetic field (blue) to high (red)

Higher Grade Zone

The 2019 drilling has also highlighted a higher grade area (>0.3% nickel)

on the northern side of the structure across a 1 kilometre strike length with

intersections of 126 to 260 metres containing an incrementally higher grade

zone (>0.35% nickel) with intersections of 35 to 134 metres. This higher-grade

zone remains open to the north, west and at depth. (see Figure 3)

Figure 3 - Crawford Nickel-Cobalt Project - Plan View showing Higher

Grade Zone

Total magnetic intensity colored with a linear color transform from low

magnetic field (blue) to high (red)

Next Steps

A second drill has been mobilized to accelerate the drilling required for

the initial resource and allow additional targets to be tested. Samples to

begin comprehensive mineralogical and metallurgical work are being selected

and initial results are expected during Q1-2020.

Table 1a - 2019 Crawford Nickel-Cobalt Project (Drilling from North Side

from East to West)

Intersections from North Wall

---

DDH ID

From

To

Length

Ni

Co

Pd

Pt

S

---

(m)

(m)

(m)

(%)

(%)

(ppb)

(ppb)

(%)

CR19-11                48.0        438.0             390.0

0.271          0.014          28.1         11.4        0.19

---

includes        69.0        292.5             223.5

0.324          0.015          43.8         16.8        0.27

---

includes       144.0        277.5             133.5

0.357          0.015          61.9         23.4        0.32

---

CR19-10                55.5        388.5             333.0

0.277          0.013          25.5         10.3        0.34

---

includes        61.5        271.5             210.0

0.320          0.013          30.5         11.1        0.49

---

includes       208.5        243.0              34.5

0.355          0.015          37.0         13.2        1.18

---

CR19-09                55.5        513.0             457.5

0.254          0.013          19.7         10.0        0.08

---

includes        70.5        309.0             238.5

0.310          0.013          26.9         13.2        0.11

---

includes       192.0        265.5              73.5

0.365          0.014          47.0         10.5        0.17

---

CR19-08                36.0        592.5             556.5

0.251          0.013          20.2         11.0        0.06

---

includes        70.5        468.0             397.5

0.271          0.013          18.2         11.2        0.06

---

includes       165.0        348.0             183.0

0.319          0.012          33.4         14.6        0.11

---

includes       183.0        223.5              40.5

0.351          0.013          26.3         12.8        0.18

---

CR19-13               102.0        609.0             507.0

0.237          0.013          10.1          8.1        0.03

---

includes       300.0        528.0             228.0

0.276          0.013          19.4         13.0        0.04

---

includes       300.0        405.0             105.0

0.325          0.012          37.3         13.7        0.07

---

includes       304.5        343.5              39.0

0.351          0.012          26.3          9.5        0.10

---

Note that the intersections are referenced to depth downhole and do not

necessarily represent a true width. Canada Nickel has insufficient information

to determine the attitude, either of the ultramafic body or of mineralized

zones within it at this point in the exploration program. True widths will be

less than the core lengths by unknown factors but based on the orientation of

the drillholes are estimated to be 65% of the length of the mineral

intersections.

Table 1b - 2019 Crawford Nickel-Cobalt Project (Drilling from South Side

from East to West)

Intersections from South Wall

---

DDH ID

From

To

Length

Ni

Co

Pd

Pt

S

---

(m)

(m)

(m)

(%)

(%)

(ppb)

(ppb)

(%)

CR19-06            207.0       576.0            369.0

0.229          0.011          5.3          3.5        0.04

---

includes      297.0       463.5            166.5

0.272          0.012          2.9          1.2        0.03

---

CR19-05             51.6       582.0            530.4

0.227          0.013         18.0          7.9        0.13

---

includes      432.0       481.5             49.5

0.310          0.015         36.7         10.4        0.53

---

includes      445.5       472.5             27.0

0.359          0.018         50.8         14.8        0.78

---

CR19-12             57.0       571.5            514.5

0.210          0.013         17.1         17.3        0.06

---

includes       57.0       279.0            222.0

0.293          0.012         13.4          3.8        0.09

---

includes       61.5       157.5             96.0

0.310          0.013         17.0          4.8        0.16

---

includes       72.0        88.5             16.5

0.365          0.014         18.7          4.1        0.24

---

CR19-07            204.0       619.5            415.5

0.221          0.013          7.2          7.5        0.01

---

includes      591.0       619.5             28.5

0.265          0.013         91.8         34.2        0.04

---

Note that the intersections are referenced to depth downhole and do not

necessarily represent a true width. Canada Nickel has insufficient information

to determine the attitude, either of the ultramafic body or of mineralized

zones within it at this point in the exploration program. True widths will be

less than the core lengths by unknown factors but based on the orientation of

the drillholes are estimated to be 65% of the length of the mineral

intersections.

Quality Assurance (QA) and Quality Control (QC)

William E. MacRae, MSc, P.Geo., a Qualified Person as defined by NI

43-101, is responsible for the on-going drilling and sampling program,

including quality assurance (QA) and quality control (QC). The core is

collected from the drill in sealed core trays and transported to the core

logging facility. The core is marked and sampled at 1.5 metre lengths and cut

with a diamond blade saw. Samples are bagged with QA/QC samples inserted in

batches of 35 samples per lot. Samples are transported in secure bags directly

from the Canada Nickel core shack Actlabs Timmins, an ISO/IEC 17025 accredited

lab. Analysis for precious metals (gold, platinum and palladium) are completed

by Fire Assay while analysis for nickel, cobalt, sulphur and 17 other elements

are performed using a peroxide fusion and ICP-OES analysis. Certified

standards and blanks are inserted at a rate of one QA/QC sample per 32 core

samples making a batch of 35 samples that are submitted for analysis.

Qualified Person

Stephen J. Balch P.Geo. (ON), VP Exploration of Canada Nickel and a

"qualified person" as such term is defined by National Instrument 43-101, has

verified the data disclosed in this news release, and has otherwise reviewed

and approved the technical information in this news release on behalf of

Canada Nickel Company Inc.

About Canada Nickel Company

Canada Nickel Company Inc. is a Canadian-based company focused on the

acquisition, exploration, development and eventual production of Nickel

assets. Canada Nickel is currently exploring its 100% owned flagship Crawford

Nickel-Cobalt project in the heart of the prolific Timmins-Cochrane mining

camp. The Crawford Project comprises 650 hectares with approximately 17,700

meters drilled to date, with identified Nickel and Cobalt bearing minerals

similar to that at the Dumont Nickel Deposit (RNC Minerals property). Canada

Nickel is focused on proving up the large bulk tonnage opportunity atCrawford,

which provides investors with pure leverage to Nickel, unencumbered

by earn-in agreements, in a geopolitically stable jurisdiction.

Cautionary Statement Concerning Forward-Looking Statements

This press release contains certain information that may constitute

"forward-looking information" under applicable Canadian securities

legislation. Forward looking information includes, but is not limited to,

timing for completion of the proposed transaction, the amount and uses of

proceeds, strategic plans, including future exploration and development

results, and corporate and technical objectives. Forward-looking information

is necessarily based upon a number of assumptions that, while considered

reasonable, are subject to known and unknown risks, uncertainties, and other

factors which may cause the actual results and future events to differ

materially from those expressed or implied by such forward-looking

information. Factors that could affect the outcome include, among others:

future prices and the supply of metals, the results of drilling, inability to

raise the money necessary to incur the expenditures required to retain and

advance the property, environmental liabilities (known and unknown), general

business, economic, competitive, political and social uncertainties, results

of exploration programs, risks of the mining industry, delays in obtaining

governmental approvals, and failure to obtain regulatory or shareholder

approvals. There can be no assurance that such information will prove to be

accurate, as actual results and future events could differ materially from

those anticipated in such information. Accordingly, readers should not place

undue reliance on forward-looking information. All forward-looking information

contained in this press release is given as of the date hereof and is based

upon the opinions and estimates of management and information available to

management as at the date hereof. Canada Nickel disclaims any intention or

obligation to update or revise any forward-looking information, whether as a

result of new information, future events or otherwise, except as required by

law.

Contacts (Canada Nickel)

Russell Starr, DirectorPhone: 647-669-9801email:

RussellStarr(at)canadanickel.com

Jessie Liu-Ernsting, VP Corporate Development and Investor

RelationsPhone: 416-809-9296 email: jessieliuernsting(at)canadanickel.com

About Noble Mineral Exploration Inc.

Noble Mineral Exploration Inc. is a Canadian-based junior exploration

company which, in addition to its shareholdings in Spruce Ridge Resources Ltd.

and MacDonald Mines Exploration Ltd., and its interest in the Holdsworth gold

exploration property in the area of Wawa, Ontario, 78,500 hectares (~194,000

acres) of mineral rights in the Timmins-Cochrane areas of Northern Ontario

known as Project 81. Project 81 hosts diversified drill-ready gold,

nickel-cobalt and base metal exploration targets at various stages of

exploration. More detailed information is available on the website at

www.noblemineralexploration.com.

Noble's common shares trade on the TSX Venture Exchange under the symbol

"NOB".

View original content to download

multimedia:http://www.prnewswire.com/news-releases/noble-announces-canada-nick

el-completes-inaugural-drill-program--all-9-holes-intersect-nickel-cobalt-pgm-

mineralization-330-metres-across-1-4-kilometre-strike-length-300971137.html

SOURCE Noble Mineral Exploration Inc.

View original content to download multimedia:

http://www.newswire.ca/en/releases/archive/December2019/09/c4496.html

-0-                           12/09/2019

/For further information: Contacts (Noble): H. Vance White, President,

Phone: 416-214-2250, Fax: 416-367-1954 Email:

info(at)noblemineralexploration.com; Investor Relations Email:

ir(at)noblemineralexploration.com/

/Web Site: http://www.noblemineralexploration.com /

(NLPXF NOB. NB7)

CO:  Noble Mineral Exploration Inc.

ST:  Ontario

IN:  MNG OTC

SU:

-30-

#