Source	CANADA NEWSWIRE

Date	03/25/2021

Time	06:05:15 AM

Company	 Cantex Mine Development Corp.

Title	Cantex Continues to Intersect High Grade Massive Sulphides at North

Rackla, Yukon Including a 7.3m Intersection of 29.86% Lead-Zinc and

104g/t Silver

CDNX Symbol: CD

Press Release

" BC-Cantex-North-Rackla   03-25 1604

#News release via Canada NewsWire, Toronto 416-863-9350 -ME-

Attention co.102687:

^Cantex Continues to Intersect High Grade Massive Sulphides at North

Rackla, Yukon Including a 7.3m Intersection of 29.86% Lead-Zinc and

104g/t Silver@

KELOWNA, BC, March 25, 2021 /CNW/ - Cantex Mine Development Corp. (TSXV:

CD) (the "Company") is pleased to release an update on the work program at its

14,077 hectare North Rackla claim block where drill results continue to define

a lead-zinc-silver mineralized system with Broken Hill Type (BHT) affinities.

Dr. Chuck Fipke reports

RESULTS

GZ Zone

Hole YKDD20-177 was drilled vertically into the GZ zone located 550

metres southeast of the high-grade Extension Sector and intersected 7.3 metres

of 24.45% zinc, 5.41% lead and 104 g/t silver between 46.1 and 53.4 metres

depth. The hole was collared 64 metres southwest of the previously reported

hole YKDD20-174 (see news release from February 1, 2021) which contained 4.05m

of 32.14% zinc, 7.25% lead and 130 g/t silver within 8.95m of 22.77% lead-zinc

and 76g/t silver (see Figures 1 and 2). As the whole GZ Zone is covered in

overburden, structural geologist Chris Buchanan is focusing on acquiring more

data to define the trend of these extremely rich massive sulphides. Additional

drilling and/or excavation of overburden are needed to establish how the GZ

Zone massive sulphides relate to the high-grade Main Zone massive sulphides.

By way of comparison, the Mount Isa Mine is one of the largest zinc mines

in the world and hosts Proven and Probable reserves containing 7.4% zinc, 3.6%

lead and 66g/t silver.

Main Zone

Central Sector

Drilling from pad MZ35 at the Central Sector intersected high grade

intersection of 4.7 metres of 21.05% lead-zinc with 66 g/t silver including

2.8 metres of 26.33% zinc, 7.22% lead and 99 g/t silver from 594.9 metres

depth in the -73 degree inclined hole YKDD20-170. This new mineralization

occurs in dolomite approximately 100m outward from the dolomite-argillite

contact (see Figures 1 and 3). Thin (1 to 1.3m wide) intercepts of massive

sulphide were also found in dolomite outward from the contact in holes

YKDD20-166 and YKDD20-168 also drilled from pad MZ35 at -55 degree and -65

degree inclinations respectively. The massive sulphides are increasing in

thickness and grade with depth (33.55% lead-zinc with 99 g/t silver) in these

holes (YKDD20-170). An additional deep hole drilled at -80 degrees inclination

is needed to determine whether this newly discovered mineralization continues

to increase in thickness and grade with depth.

Discovery Sector

A -45 degree inclined hole (YKDD20-178) drilled from pad MZ51 intersected

5.23% lead, 4.73% zinc and 31 g/t silver at the dolomite-argillite contact

located 250m northeast of the Discovery Sector. Unfortunately, this

intersection (that was spotted by Buchanan) has been gouged and faulted

subsequent to lead-zinc-silver mineralization deposition. Thus, mineralization

may have been displaced at the drill intercept by the faulting. The end of the

drill program prevented the completion of -55 and -65 degree inclined holes.

These will be drilled this season.

Drill results from the final holes of the 2020 season are presented in

Table 1.

Table 1: Drill results

Pad Dip

Hole             From                 To    Length

True Width     Silver

Lead + Zinc        Lead        Zinc           Copper

Manganese

---

---

(m)                  (m)     (m)

(m)          g/t

%              %           %

%

%

GZ02C     -89

YKDD20-177                      37.00                38.00      1.00

n/a                0.10                     1.04        0.02

1.02              0.00                       0.21

---

46.10                53.40      7.30

n/a              103.68                    29.86        5.41

24.45              0.01                       0.27

56.00                57.00      1.00

n/a                1.08                     0.21        0.05

0.15              0.01                       0.34

GZ02B     -45

YKDD20-181

Did not reach target depth

---

-90

YKDD20-179

No significant results

GZ01      -45

YKDD20-180

No significant results

---

MZ0X      -68

YKDD20-167

No significant results

---

MZ07X     -64

YKDD20-169

No significant results

---

-66

YKDD20-173                       3.00                 4.00

1.00                     0.61        0.40                     1.70

0.01         1.69              0.01                       0.25

335.70               336.70      1.00                     0.61

7.80                     6.18        0.20         5.98

0.02                       2.84

341.40               341.90      0.50                     0.30

24.10                    11.19        0.24        10.95

0.02                       1.13

-70

YKDD20-171                     352.35               362.70

10.35                     5.86       27.50                     7.19

1.62         5.57              0.05                       3.15

Including           355.80               359.10      3.30

1.87       30.90                    13.35        1.89        11.46

0.02                       3.81

364.50               365.00      0.50                     0.28

15.95                     5.00        4.91         0.09

0.00                       1.88

MZ33      -85

YKDD20-182

Did not reach target depth

---

MZ35      -55

YKDD20-166                     132.15               132.65

0.50                     0.36        3.00                     1.82

0.14         1.68              0.01                       0.03

---

283.50               294.30     10.80                     7.77

9.55                     4.46        2.32         2.14

0.04                       4.09

Including           286.40               292.00      5.60

4.03       13.82                     6.89        3.69         3.20

0.02                       4.57

479.70               481.00      1.30                     1.19

21.84                     7.84        3.81         4.03

0.06                       2.69

-65

YKDD20-168                     211.85               212.50

0.65                     0.41        0.50                     2.07

0.00         2.07              0.01                       0.08

373.85               374.35      0.50                     0.32

7.41                     5.74        0.75         4.99

0.08                       0.26

411.60               412.20      0.60                     0.38

15.10                     8.70        1.93         6.77

0.01                       0.18

553.00               554.00      1.00                     0.81

4.00                     3.76        0.19         3.57

0.01                       0.69

-73

YKDD20-170                     439.20               439.75

0.55                     0.31       45.80                     1.48

0.61         0.87              0.89                       2.54

594.90               599.60      4.70                     3.55

66.30                    21.05        5.21        15.84

0.04                       3.04

Including           596.35               599.15      2.80

2.11       99.40                    33.55        7.22        26.33

0.07                       1.48

MZ51      -45

YKDD20-178                         68                 68.5      0.50

n/a               27.00                    10.52        4.60

5.92              0.02                       1.80

---

70                   79      9.00

n/a               30.50                     9.96        5.23

4.73              0.09                       2.21

159                159.5      0.50

n/a                3.24                     1.35        0.03

1.32              0.01                       0.70

168.5                169.4      0.90

n/a                0.40                     1.21        0.01

1.20              0.00                       3.72

SUMMARY OF STRUCTURAL ANALYSIS

Our structural geologist Chris Buchanan has completed a detailed

structural analysis, including plan maps and cross sections. He has explained

that a major east-west trending strike-slip fault displaced the Extension

Sector from the Central Sector by a distance of about 200m, explaining why

drill intersections were unsuccessful in this 200m gap. He has also identified

northeast trending faults that could displace the massive sulphides by up to

30m. The mineralization along the argillite-dolomite contact is very prolific

and the Cantex geologists all agree that favourable intersections are likely

along the northeast trending contact mapped by Buchanan from the Central

Sector all the way to pad MZ51 and beyond. Drilling of this area is planned to

commence in the spring.

SAMPLE PREPARATION

The drill holes reported in this press release were drilled using HQ

(63.5mm) diamond drill bits. The core was logged, marked up for sampling and

then divided into equal halves using a diamond saw on site. One half of the

core was left in the original core box. The other half was sampled and placed

into sealed bags which were in turn placed into larger bags closed with

security seals prior to being transported to CF Mineral Research Ltd. in

Kelowna, BC.

At CF Minerals the drill core was dried prior to crushing to -10 mesh.

The samples, which averaged over 3kg, were then mixed prior to splitting off

800g. The 800g splits were pulverized to -200 mesh and a 250g split was sent

for assay. Quality control procedures included running a barren sand sample

through both the crusher and pulveriser between each sample to ensure no

inter-sample contamination occurred. Silica blanks were inserted along with

certified reference samples. These quality control samples were each inserted

approximately every 20 samples.

ALS Chemex in Vancouver assayed the samples using a four-acid digestion

with an ICP-MS finish. The 48 element ME-MS61 technique was used to provide a

geochemical signature of the mineralization. Where lead, zinc or copper values

exceeded one percent the Pb-OG62, Zn-OG62 or Cu-OG62 techniques were used.

These have upper limits of 20% lead, 30% zinc and 50% copper, respectively.

Samples with lead and zinc values over these limits were then analyzed by

titration methods Pb-VOL70 and Zn-VOL50. Where silver samples exceeded 100 g/t

the Ag-OG62 technique was used which has an upper limit of 1,500 g/t. The over

limit analyses contributed to delays in receiving final assay results.

FUTURE PLANS

The gold and base metal results for 312 rock samples and 2,800 soil-talus

samples collected during 2020 are forthcoming and will be released when

received.

Cantex looks forward to the coming drill season where building the known

tonnage of the Main Zone will be the main focus. Further drill testing of the

GZ Zone and additional targets within the North Rackla claim block is also

planned.

The technical information and results reported here have been reviewed by

Mr. Chad Ulansky P.Geol., a Qualified Person under National Instrument 43-101,

who is responsible for the technical content of this release.

Signed,

Charles Fipke

Charles FipkeChairman

Neither TSX Venture Exchange nor its Regulation Services Provider (as

that term is defined in the policies of the TSX Venture Exchange) accepts

responsibility for the adequacy or accuracy of this release. Information set

forth in this news release includes forward-looking statements under

applicable securities laws. Forward-looking statements are statements that

relate to future, not past, events. In this context, forward-looking

statements often address expected future business and financial performance,

and often contain words such as "anticipate", "believe", "plan", "estimate",

"expect", and "intend", statements that an action or event "may", "might",

"could", "should", or "will" be taken or occur, or other similar expressions.

All statements, other than statements of historical fact, included herein

including, without limitation; statements about the terms and completion of

the proposed sale transaction are forward-looking statements. By their nature,

forward-looking statements involve known and unknown risks, uncertainties and

other factors which may cause the actual results, performance or achievements,

or other future events, to be materially different from any future results,

performance or achievements expressed or implied by such

forward-lookingstatements. Such factors include, among others, risks identified

in the

management discussion and analysis section of the Company's interim and most

recent annual financial statements or other reports and filings with Canadian

securities regulators. Forward looking statements are made based on

management's beliefs, estimates and opinions on the date that statements are

made and the respective companies undertake no obligation to update

forward-looking statements if these beliefs, estimates and opinions or other

circumstances should change, except as required by applicable securities laws.

Investors are cautioned against attributing undue certainty to forward-looking

statements.

SOURCE Cantex Mine Development Corp.

View original content to download multimedia:

http://www.newswire.ca/en/releases/archive/March2021/25/c0088.html

-0-                           03/25/2021

/For further information: Cantex Mine Development Corp, Tel:

+1-250-860-8582; Email: info(at)cantex.ca/

/Web Site: http://www.cantex.ca /

(CD.)

CO:  Cantex Mine Development Corp.

ST:  British Columbia

IN:  MNG

SU:

-30-

#