Source	MARKET NEWS

Date	09/26/2017

Time	05:19:58 AM

Company	 Kilo Goldmines Ltd.

Title	Stream

Sediment Sampling Defines New Gold Explor

CDNX Symbol: KGL

Press Release

KILO GOLDMINES LTD ("KGL-V")

- Stream Sediment Sampling Defines New Gold Exploration Targets

- Within The Ngayu Greenstone Belt

Kilo Goldmines Ltd. ("Kilo" or "KGL" or the "Company") is pleased

to provide the results of a BLEG (Bulk Leach Extractable Gold) stream

sediment sampling program on five licences within the Ngayu greenstone

belt in the north-eastern Democratic Republic of Congo ("DRC").

Photos accompanying this announcement are available at

http://www.globenewswire.com/NewsRoom/AttachmentNg/3d457c4a-e559-4ed0-9a3c-

8dd5ef03cf5b

http://www.globenewswire.com/NewsRoom/AttachmentNg/ec39f1ba-6e5e-4e3c-ba4a-

57bcf867c826

http://www.globenewswire.com/NewsRoom/AttachmentNg/5e1819bb-4ab0-4178-b3cc-

68b90fb606ae

http://www.globenewswire.com/NewsRoom/AttachmentNg/cb438ecc-8c2b-483e-89e3-

2defbd6114a8

HIGHLIGHTS

Three well-defined BLEG anomalies have been located as follows:

*  In the northern part of PE9692, where anomalous values of 21 -

41 ppb Au (compared to background values of (3 ppb) indicate the

potential for mineralization over a strike of about 2 km.  Follow-up

mapping and rock chip sampling have shown the presence of mineralized

BIF (Banded Iron Formation) forming an anticlinal fold closure, with

values up to 5.70 g/t Au.

*  In the southwestern part of PE9692, a stream catchment with

an anomalous value of 65 ppb Au indicates a potential extension of

mineralization from the adjacent Loncor Itali prospect.

*  In the southern part of PE9695, a strong anomaly with values

up to 372 ppb, indicates the possible presence of mineralization

parallel to Loncor's Yindi prospect located 4 km to the southwest.

BLEG SAMPLING PROGRAM

The BLEG sampling program was carried out on five of the Company's

exploration licences in the Ngayu greenstone belt (PE137, PE138, PE140,

PE9692 and PE9695), the locations of which are shown in Figure 1.

Drainage catchments within these licences were defined using

hydrography vector maps derived from 50 cm stereo satellite photos by

Photosat, Canada. A total of 310 catchments were sampled over an area

of 239 km2, resulting in an average catchment size of 0.77 km2.

At each sampling site, 3 kg of the finest-grained sediment (mud)

was collected from the top of the steam bed.  A duplicate sample was

taken at every fifth site for quality control purposes.  The samples

were thoroughly dried at the Company's base camp at Adumbi, and 1.1 kg

of each sample was sent to ALS Minerals in Ireland for analysis.  Gold

was analysed by cyanide leach bottle roll on 1 kg of sample.  Arsenic,

and a suite of 51 other elements, were determined by aqua regia

digestion of 0.5 g of sample followed by ICP-MS and ICP-AES.

QUALITY CONTROL

Analytical quality was assessed by means of randomly-inserted

standard reference materials and blanks.   All standards passed the

Company's QC criteria and 15 of the 16 blank samples returned values (1

ppb Au; one blank assayed 2.3 ppb.  Sample representivity was assessed

by comparing analyses of the original samples with 46 field duplicates;

good correlations were obtained (see examples in Figure 2) indicating

that the sampling technique produced reliable, representative samples

with a low nugget effect.

ANALYTICAL RESULTS

PE9692.  As shown in Figure 3, anomalous gold values of 21 - 41

ppb Au were returned for catchments over an area of approximately 3 km

x 2 km in the northern part of the licence, with background values of

up to 3 ppb Au dominating in the southeast.  The northern anomaly

coincides with an area of artisanal alluvial mining activity, and

following the BLEG sampling, was further investigated by mapping and

rock chip sampling.  As shown in Figure 4, the area is underlain by BIF

interlayered with chlorite schist, carbonaceous schist and

quartz-sericite schist, which is interpreted to form the closure of a

northerly-plunging anticline and to represent a very favourable

litho-structural setting for gold mineralization.  Sulphidised BIF was

found in outcrop and float at a number of localities, and as shown in

Figure 4, values of up to 5.70 g/t were returned from rock chip

samples. Auriferous quartz veins were also sampled in several

localities, and are being exploited by artisanal miners in a bedrock

working in the northwest of the mapped block.

An anomalous BLEG result of 65 ppb Au was returned for a catchment

in the southwest of PE9692.  This area lies immediately to the east of

Loncor Resources' Itali prospect, and probably indicates an extension

of the Itali mineralization onto the Company's licence.

Arsenic values show a general correlation with gold in PE9695,

although the arsenic anomalies are less well-defined, probably due to

greater dispersion in both the primary mineralization and tropical

weathering environment.  A similar relationship was also found in the

other licence areas.

PE9695.  A strong BLEG anomaly is present in the south of PE9695,

with values of 40 - 372 ppb Au occurring in catchments covering over an

area of about 4 km2 (Figure 4).  The source of the anomalism appears to

be located approximately 4 km to the northeast of Loncor's Yindi

Prospect, and may represent a parallel (i.e. northwest-trending)

mineralized zone.

PE137 and PE140.  All BLEG samples from PE137 returned background

values of (2 ppb.  Although two isolated samples from PE140 returned

elevated values of 5.5 ppb Au and 7.0 ppb Au, the mineralization

potential of both these licences is considered to be low.

PE138.  This licence is located in flat-lying terrain immediately

to the north of the Ngayu River with four streams flowing southwards

through the property. However, three of these streams traverse an area

of intense artisanal activity 1 - 2 km north of the licence boundary,

and the anomalous values of 21 - 217 ppb Au therefore probably

represent contamination from this area.  A stream in the east, which

originates within PE138, returned low Au values. Although the geometry

of the drainage system has not enabled the whole area to be assessed by

the BLEG survey, further work will be difficult due to extensive

alluvial cover from the Ngayu River flood plain, and the property is

considered to be a low exploration priority.

FURTHER EXPLORATION

Additional work is warranted in the three areas of BLEG anomalism

on PE9692 and PE9695 described above. A programme would entail soil

sampling over the anomalous catchments, initially at a line spacing of

160 m, with infill at 80 m depending on results.  Soil anomalies would

be further investigated by trenching and/or mechanical augering, with

the objective of defining mineralized targets for drill testing.

Qualified Person

Howard Fall, B.Sc., PhD, MAusIMM, QP (Geo) is the 'Qualified

Person' (as such term is defined under National Instrument 43-101) of

Kilo and has reviewed the scientific and technical information

contained in this release.

About Kilo

Kilo Goldmines Ltd. is a Canadian gold exploration company that is

listed on the TSX Venture Exchange under the symbol 'KGL' and on the

Frankfurt Exchange under the symbol '02K'.  The Company holds about

2,417 km of prospective Archaean Kibalian greenstone in the Kilo-Moto

area in the Democratic Republic of the Congo.

Incorporated within these licences is:

- the Somituri project (71.25% owned by KGL), comprising six

contiguous licences (361km) held by KGL-Somituri SARL

- the KGL Isiro SARL Joint Venture (JV) with Randgold Resources

Ltd (2,056 km), for gold and associated minerals only. The JV is

managed by Randgold and financed by it to a pre-feasibility (PFS) for a

51% participation interest. Upon completion of the PFS, KGL can

participate in funding or Randgold will increase its participation to

65% by completing a Feasibility Study. Areas which may be deemed of no

interest to Randgold will be returned to KGL.

KGL has retained the rights to explore for and develop iron ore

resources and other minerals associated with the licences held by KGL

Isiro SARL.

Disclaimer

This news release may contain forward looking statements

concerning future operations of Kilo Goldmines Ltd. All forward looking

statements concerning Kilo's future plans and operations, including

management's assessment, project expectations or beliefs may be subject

to certain assumptions, risks and uncertainties beyond Kilo's control.

Investors are cautioned that any such statements are not guarantees of

future performance and that actual performance and exploration and

financial results may differ materially from any estimates or

projections.

Neither TSX Venture Exchange nor its Regulation Services Provider

(as that term is defined in the policies of the TSX Venture Exchange)

accepts responsibility for the adequacy or accuracy of this release.

TSX Venture closing price for KGL-V

Date: 2017/09/25

Closing Price: 0.04

TEL:  +1 416 360 3406  Philip Gibbs, Interim CEO

EMAIL:  philip.gibbs@kilogoldmines.com

INTERNET:  www.kilogoldmines.com

____________________________________________________________

(c)2017 Market News Publishing Inc.  All rights reserved.

Toronto:(416)366-8881   Vancouver:(604)689-1101   Fax:(604)689-1106

#1726800007TS11726800706-05172520170926##