Source	CANADA NEWSWIRE

Date	10/17/2019

Time	09:03:44 PM

Company	 Rockwealth Resources Corp.

Title	Rockwealth Announces Realgold's Results from Drill and Surface Trench

Program at Uluktau and Alai-Karabiy Gold Project

CDNX Symbol: RWR

Press Release

" BC-RockwealthDrillResult   10-18 3234

#News release via Canada NewsWire, Vancouver 604-669-7764 -ME-

Attention co.331768:

^Rockwealth Announces Realgold's Results from Drill and Surface Trench

Program at Uluktau and Alai-Karabiy Gold Project@

Initial exploration program successfully targets Carlin type gold

mineralization at three areas in the Kyrgyz Republic

KELOWNA, BC, Oct. 18, 2019 /CNW/ - Rockwealth Resources Corp.

("Rockwealth" or the "Company") (TSXV: RWR) is pleased to announce that

Realgold Resources Corp. ("Realgold") has updated results for its ongoing 2019

exploration on its Carlin-type gold portfolio project located in the southwest

Kyrgyz Republic (the "Uluktau and Alai-Karabiy Gold Project"). This update

provides gold assay results received to date from core drilling and channel

sampling of surface trenches on three areas on Realgold's high priority

Uluktaui and Alai-Karabiy (North Karabiy and Slantsevyi Klin) project. A total

of twelve drill holes have been successfully completed from surface and all

assays received; five holes at North Karabiy and seven holes at Slantsevyi

Klin. Of particular note is hole DH-SK-03_19 at Slantsevyi Klin, which hit

multiple intervals including 15m (at) 3.2 g/t Au including 4m (at) 6.1 g/t Au.

Assays have also been received from channel sampling on nine trenches: five at

North Karabiy and four trenches at Uluktau. Of particular note is trench NK-4

at North Karabiy, which returned an interval of 14m (at) 7.8 g/t Au including 3m

(at) 27.1 g/t Au and trench UK-54 at Uluktau, which returned an interval of 5.2m

(at) 15.3 g/t Au.

Program Highlights

North Karabiy Area

<<

--  Five holes completed in 512.1m with two holes restarted due to technical

problems with drilling

--  All holes intersected mineralized intervals in widths ranging from 1.0

to 5.0m

--  Results include hole DH-NK-5bis: 3m (at) 5.3 g/t Au (149.0m to 152.0m),

including 1m (at) 10.3 g/t Au (149.0m to 150.0m)

--  Sampling at five surface trenches completed

--  All trenches intersected gold mineralization

--  Results include trench NK-4 with intervals such as 14.0m (at)7.8 g/t Au

including 3m (at) 27.1 g/t Au

<>>

Slantsevyi Klin Area

<<

--  Seven holes completed in 284.9m including one hole restarted due to

technical problems with drilling

--  Four holes intersected gold mineralized intervals in widths ranging from

1.0 to 15.0m

--  Results include hole DH-SK-03_19 which hit multiple intervals such as

15m (at) 3.2 g/t Au (9.0m to 24.0m) including 4m (at) 6.1 g/t Au (11.0m to

15.0m)

<>>

Uluktau Area

<<

--  Sampling at four surface trenches completed

--  All trenches intersected mineralization

--  Results include trench UK-63 with intervals such as 9.0m (at)5.9 g/t Au

including 1m (at) 25.6 g/t Au and 2m (at) 7.7 g/t Au

<>>

Doug Kirwin, CEO of Realgold commented, "This is a very positive

exploration milestone at our Uluktau and Alai-Karabiy Project and follows

recent high-grade drill results at Uluktau, where drilling in 2017 in hole

DHU-08-2017 intersected 21m (at) 33.7 g/t Au. Our portfolio, situated in the

southwest area of the democratically elected, politically stable and

pro-mining Kyrgyz Republic, lies within the Tien Shan Mineral Belt. Realgold's

extensive portfolio covers 90% of the newly recognized and underexplored

Carlin-type gold belt in the Kyrgyz Republic and importantly, our drilling and

trenching programs continue to confirm the strong potential of our

properties."

Technical Details

Alai Group

The Alai group constitutes a land package that covers 10,909 ha. Alai is

comprised of four separate areas; The North Karabiy area is part of the Alai

group. Karabiy is located on the western extent of Alai and North Karabiy

represents the northern region of Karabiy. The property is located is

immediately east of the city of Kadamjai. Access to the area is from Osh via

the national highway to Kadamjai and then by provincial roads. The area is sub

mountainous terrain with local relief ranging from 1100m to 1900m.

The Alai-Karabiy area and environs were extensively studied and

prospected by the Soviets for antimony, mercury and arsenic, with the most

modern exploration conducted in the 1960s through to the 1990s.

Antimony-mercury mineralisation was established within a set of

stibnite-cinnabar-bearing jasperoid horizons associated with thrust faults.

These jasperoidal rocks form relatively thin, brecciated, lenticular horizons

associated with thrust faults and proximal to the contact of the Carboniferous

limestone formation with the Siluro-Devonian sediments. These thrust faults

are offset laterally in an imbricate NE-SW pattern by a later stage of

faulting.

North KarabiyIn 2019, a total of five core holes were completed on North

Karabiy, which included 2 restarts where holes were prematurely terminated due

to technical issues with drilling. In addition, a total of 5 surface trenches

were exposed, mapped and channel sampled. Results of the drill holes are

detailed in Table 1 and results of surface trenching are detailed in Table 2.

Composite Parameters

<<

1. Minimum Width: No minimum width

2. Interval Grade Cut-Off: 1 g/t Au

3. Maximum Interval Internal Dilution: Maximum of 1 sample <1 g/t Au

<>>

Composite Parameters

<<

1. Minimum Width: No minimum width

2. Interval Grade Cut-Off: 1 g/t Au

3. Maximum Interval Internal Dilution: Maximum of 1 sample <1 g/t Au

<>>

Hole by Hole Summary

DH-NK-1-19DH-NK-1-19 was collared as a vertical hole designed to confirm

the mineralized zone exposed in trench 19NKT01 (9 m (at) 8.37 g/t) and completed

to a depth of 40.0m. A weakly anomalous gold value was intersected within a

brecciated and jasperoidized limestone. Anomalous pathfinder elements of

arsenic, thallium and antimony were encountered in this interval grading; 180

ppm As, 3 ppm Tl and 3,022 ppm Sb.

DH-NK-2-19DH-NK-2-19 was an angled hole designed to confirm the

mineralized zone exposed in trench 19NKT03 (6 m (at) 2.02 g/t) and completed to

a

depth of 29.0m. Gold mineralization was intersected in the top of the hole

from 0m to 1m depth grading 1m (at) 1.2 g/t Au in jasperoids. Anomalous

pathfinder elements of arsenic, thallium and antimony are present in this

mineralized interval with grades over the interval of; 1,756 ppm As, 9 ppm Tl

and 850 ppm Sb. From 0m to 6m, a zone of weakly anomalous gold mineralization

returned 6m (at) 0.6 g/t Au. Anomalous pathfinder elements of arsenic, thallium

and antimony are present in this mineralized interval with grades over the

interval of; 1,525 ppm As, 7.5 ppm Tl and 647 ppm Sb.

DH-NK-3-19DH-NK-3-19 was an angled hole designed to confirm the

mineralized zone exposed in trench 19NKT02 (3 m (at) 5.48 g/t) completed to a

depth of 38m. Gold mineralization was encountered in a zone of tectonic

breccia with fragments of siltstone and sandstone and limonitization in

fractures within fault gouge in an interval of 2m (at) 6.2 g/t Au (0.0m to 2.0m

depth). Anomalous pathfinder elements of arsenic, thalium and antimony were

elevated in this zone with grades of; 2,249 ppm As, 11.5 ppm Tl and 58 ppm Sb.

DH-NK-4-19DH-NK-4-19 was an angled hole designed to confirm the

mineralized zone exposed in trench 19NKT04 (14 m (at) 7.79 g/t), terminated at a

depth of 5.7m due to problems with the drill. Gold mineralization was

encountered in a narrow interval 0f 0.7m (at) 4.6 g/t Au within jasperoid.

Anomalous pathfinder elements of arsenic, thalium and antimony were

encountered in this interval with interval grades of; 2,391 ppm As, 12 ppm Tl

and 92 ppm Sb. The entire interval consisted of tectonic breccia with blocks

and fragments of jasperoid.

DH-NK-4bis-19 DH-NK-4bis-19 was an angled hole designed to confirm the

mineralized zone exposed in trench 19NKT04 (14 m (at) 7.79 g/t) and completed to

a depth of 50.6m. The hole was a redrill of DH-NK-4-19. Two intervals of gold

mineralization were encountered in this hole. Gold mineralization was

intersected from 4.0m to 9.0m grading 5m (at) 2.7 g/t Au in a tectonic breccia

with jasperoid. Within this interval, elevated pathfinder pathfinder elements

of arsenic, thallium and antimony graded; 758 ppm As, 5 ppm Tl and 21,543 ppm

Sb . Stibnite was found throughout this interval (as pockets and veinlets and

also disseminated). A second zone of gold mineralization was intersected from

36m to 38m grading 2m (at) 1.6 g/t Au within a limonitized tectonic breccia

consisting of sandstone and siltstone fragments. Within this interval elevated

pathfinder elements of arsenic, thallium and antimony graded; 2,669 ppm As,

12.5 ppm Tl and 105 ppm Sb.

DH-NK-5-19DH-NK-5-19 was collared as a vertical hole designed to confirm

the mineralized zone intersected by historic Soviet-era DH 46 (7.83 g/t Au (at)

7.95 m) and DH46? (7.4 g/t Au (at) 15.3 m). The hole was terminated at 164.0m

due

to drill breakdown. No downhole survey data exists for these historic Soviet

holes, so positioning of historic mineralization is unknown. Gold

mineralization was encountered in a 3m wide interval from 148m to 151m (3.0m

(at)

1.1 g/t Au) hosted in a brecciated black, carbonaceous and pyritized shale

above jasperoids. Within this interval elevated pathfinder elements of

arsenic, thallium and antimony graded; 1,035 ppm As; 8 ppm Tl; and 75 ppm Sb.

DH-NK-5bis-19DH-NK-5bis-19 was collared as a vertical hole designed to

confirm the mineralized zone intersected by historic Soviet-era DH 46 and

DH46?. No downhole survey data exists for these historic Soviet holes, so

positioning of mineralization is unknown. The hole was completed to a depth of

184.8m. Three zones of gold mineralization were encountered ranging in widths

from 2m to 3m in this hole. An upper zone was intersected in jasperoid breccia

above limestone from 138m to 140m grading 2m (at) 2.6 g/t Au and with elevated

pathfinder elements of arsenic, thallium and antimony grading; 1,230 ppm As, 9

ppm Tl, and 1,857 ppm Sb. A unit of tectonic breccia with fragments of black,

silicified, carbonaceous and weakly pyritized shale, jasperoid and sandstone

was encountered from 149m to 166m with elevated pathfinder elements of

arsenic, thallium and antimony grading; 929 ppm As, 6.35 ppm Tl, and 2,059 ppm

Sb over the interval. Two gold mineralized zones were encountered in this

interval with the upper zone (149m to 152m) grading 3m (at) 5.3 g/t Au and the

lower interval (163m to 166m) grading 3m (at) 1.4 g/t Au.

Slantsevyi KlinThe Slantsevyi Klin area is part of the Karabiy area of

the Alai group. Slantsevyi Klin is located on the western region of Karabiy.

In 2019, a total of six core holes were completed on North Karabiy, which

included 1 restart where the hole was prematurely terminated due to technical

issues with drilling. Results of the drill holes are detailed in Table 3.

Composite Parameters

<<

1. Minimum Width: No minimum width

2. Interval Grade Cut-Off: 1 g/t Au

3. Maximum Interval Internal Dilution: Maximum of 1 sample <1 g/t Au

<>>

Hole by Hole Summary

DH-SK-1-19DH-SK-1-19 was an angled hole designed to confirm the

mineralized zone exposed in trench 18SKR02 (29 m (at) 2.95 g/t) and completed to

a depth of 46.2m. The hole encountered jasperoid from surface to 9.15m and

followed by limestone from 9.15 to the end of hole. Anomalous gold values of

10m (at) 0.3 g/t Au in jasperoid were encountered in the top of the hole.

Anomalous important pathfinder elements including arsenic, thallium and

antimony are present within this mineralized interval with average interval

grades; of 1,023 ppm As, 13.7 ppm Tl, 79.3 ppm Sb.

DH-SK-2-19DH-SK-2-19 was an angled hole designed to confirm the

mineralized zone exposed in trench 18SKR02 and completed to a depth of 23.5m.

The hole was drilled south to north towards hole DH-SK-1-19. The hole

encountered jasperoid from surface to 13.4m and followed by limestone from

13.4m to the end of hole. A mineralized interval of 3m (at) 1.3 g/t Au was

intersected in jasperoid at the top of the hole. Anomalous important

pathfinder elements including arsenic, thallium and antimony are elevated

within this interval with average grades of; 1,372 ppm As, 15.7 ppm Tl; and

1,936 ppm Sb.

DH-SK-3-19DH-SK-3-19 was an angled hole designed to confirm the

mineralized zone exposed in trench 18SKR10 (71 m (at) 2.74 g/t) and completed to

a depth of 80.0m. The hole intersected jasperoid from the top of hole to 43m

and followed by limestone from 43m to end of hole. Multiple zones of gold

mineralization were encountered in the jasperoid between 3.0 to 43.0m depth

with interval widths ranging from 4.0m to 15.0m wide. Anomalous pathfinder

elements of arsenic, thallium and antimony were present throughout the entire

jasperoid unit with grades over a 40m interval (from 3.0m to 43.0m) of; 1,767

ppm As, 13.86 ppm Tl, and 1,049 ppm Sb.

DH-SK-4-19DH-SK-4-19 was an angled hole designed to confirm the

mineralized zone exposed in trench 18SKR10 and completed to a depth of 55.0m.

The hole was drilled from south to north towards DH-SK-3-19. The hole

intersected jasperoid from 1.55m to 9.2m and limestone from 9.2m to end of

hole. Anomalous gold values of 9m (at) 0.13 g/t Au within the jasperoid at the

top of the hole was encountered. In addition, pathfinder elements of arsenic,

thallium and antimony were elevated in this interval with average grades of;

791 ppm As, 7.1 ppm Tl and 127 ppm Sb.

DH-SK-5-19DH-SK-5-19 was an angled hole designed to confirm the potential

of a north-south trending feeder structure, located west of the mineralized

zone exposed in trench 18SKR10 and terminated at a depth of 16.0m due to

technical problems with the drill. Jasperoid was encountered at the top of

hole to the end. A narrow zone of gold mineralization of 1m (at) 1.2 g/t Au was

encountered at the top of hole. Anomalous pathfinder elements of arsenic,

thallium and antimony were elevated in this interval, with grades of; 1,626

ppm As, 7 ppm Tl, and 872 ppm Sb.

DH-SK-5bis-19 DH-SK-5bis-19 was an angled hole designed to retest the

target from DH-SK-5-19 and terminated at a depth of 18.9m due to technical

drilling problems. Jasperoid was encountered at the top of hole to the end.

Gold grades did not exceed 0.096 g/t.

DH-SK-6-19DH-SK-6-19 was a vertical hole designed to confirm the

mineralized zone exposed in trench 18SKR18 (17 m (at) 1.86 g/t) and completed to

a depth of 45.3m. The hole intersected jasperoid from the top of hole to 8.7m

and followed by limestone from 8.7m to end of hole. Gold mineralization was

encountered in jasperoids near the top of the hole from 1.2m to 3.0m depth

(1.8m (at) 1.8 g/t Au). Anomalous pathfinder elements of arsenic, thallium and

antimony were elevated within this interval with grades of; 1,638 ppm As; 12

ppm Tl; and 183 ppm Sb.

Surface trenches were exposed and sampled on both the North Karabiy andUluktau

areas, with strongly mineralized results encountered on both

properties. Results of trenching are detailed below in Tables 3 and 4.

Uluktau Group

The Uluktau area covers 6,147 ha. The property is located 38 km west

southwest of the provincial capital city Osh. Access to the area is via

national highway and provincial roads. The area is largely rolling hills with

the steeper parts associated with limestone outcrops.

Uluktau contains the abandoned Chonkoi mercury mine in the eastern part

of the license. The mine operated during Soviet times, which in turn led to an

extensive drilling campaign in the surrounding district in search of other

mercury deposits by the Soviets. This work included geologic surface and

underground mapping at various scales and drilling (485 diamond drill holes

within or nearby the Uluktau license area).

In 2019, a total of 4 surface trenches were exposed, mapped and sampled.

Results of trench sampling is detailed in Table 4.

Composite Parame

" BC-RockwealthDrillResult   10-18 3234

#News release via Canada NewsWire, Vancouver 604-669-7764 -ME-

Attention co.331768:

^Rockwealth Announces Realgold's Results from Drill and Surface Trench

Program at Uluktau and Alai-Karabiy Gold Project@

Initial exploration program successfully targets Carlin type gold

mineralization at three areas in the Kyrgyz Republic

KELOWNA, BC, Oct. 18, 2019 /CNW/ - Rockwealth Resources Corp.

("Rockwealth" or the "Company") (TSXV: RWR) is pleased to announce that

Realgold Resources Corp. ("Realgold") has updated results for its ongoing 2019

exploration on its Carlin-type gold portfolio project located in the southwest

Kyrgyz Republic (the "Uluktau and Alai-Karabiy Gold Project"). This update

provides gold assay results received to date from core drilling and channel

sampling of surface trenches on three areas on Realgold's high priority

Uluktaui and Alai-Karabiy (North Karabiy and Slantsevyi Klin) project. A total

of twelve drill holes have been successfully completed from surface and all

assays received; five holes at North Karabiy and seven holes at Slantsevyi

Klin. Of particular note is hole DH-SK-03_19 at Slantsevyi Klin, which hit

multiple intervals including 15m (at) 3.2 g/t Au including 4m (at) 6.1 g/t Au.

Assays have also been received from channel sampling on nine trenches: five at

North Karabiy and four trenches at Uluktau. Of particular note is trench NK-4

at North Karabiy, which returned an interval of 14m (at) 7.8 g/t Au including 3m

(at) 27.1 g/t Au and trench UK-54 at Uluktau, which returned an interval of 5.2m

(at) 15.3 g/t Au.

Program Highlights

North Karabiy Area

<<

--  Five holes completed in 512.1m with two holes restarted due to technical

problems with drilling

--  All holes intersected mineralized intervals in widths ranging from 1.0

to 5.0m

--  Results include hole DH-NK-5bis: 3m (at) 5.3 g/t Au (149.0m to 152.0m),

including 1m (at) 10.3 g/t Au (149.0m to 150.0m)

--  Sampling at five surface trenches completed

--  All trenches intersected gold mineralization

--  Results include trench NK-4 with intervals such as 14.0m (at)7.8 g/t Au

including 3m (at) 27.1 g/t Au

<>>

Slantsevyi Klin Area

<<

--  Seven holes completed in 284.9m including one hole restarted due to

technical problems with drilling

--  Four holes intersected gold mineralized intervals in widths ranging from

1.0 to 15.0m

--  Results include hole DH-SK-03_19 which hit multiple intervals such as

15m (at) 3.2 g/t Au (9.0m to 24.0m) including 4m (at) 6.1 g/t Au (11.0m to

15.0m)

<>>

Uluktau Area

<<

--  Sampling at four surface trenches completed

--  All trenches intersected mineralization

--  Results include trench UK-63 with intervals such as 9.0m (at)5.9 g/t Au

including 1m (at) 25.6 g/t Au and 2m (at) 7.7 g/t Au

<>>

Doug Kirwin, CEO of Realgold commented, "This is a very positive

exploration milestone at our Uluktau and Alai-Karabiy Project and follows

recent high-grade drill results at Uluktau, where drilling in 2017 in hole

DHU-08-2017 intersected 21m (at) 33.7 g/t Au. Our portfolio, situated in the

southwest area of the democratically elected, politically stable and

pro-mining Kyrgyz Republic, lies within the Tien Shan Mineral Belt. Realgold's

extensive portfolio covers 90% of the newly recognized and underexplored

Carlin-type gold belt in the Kyrgyz Republic and importantly, our drilling and

trenching programs continue to confirm the strong potential of our

properties."

Technical Details

Alai Group

The Alai group constitutes a land package that covers 10,909 ha. Alai is

comprised of four separate areas; The North Karabiy area is part of the Alai

group. Karabiy is located on the western extent of Alai and North Karabiy

represents the northern region of Karabiy. The property is located is

immediately east of the city of Kadamjai. Access to the area is from Osh via

the national highway to Kadamjai and then by provincial roads. The area is sub

mountainous terrain with local relief ranging from 1100m to 1900m.

The Alai-Karabiy area and environs were extensively studied and

prospected by the Soviets for antimony, mercury and arsenic, with the most

modern exploration conducted in the 1960s through to the 1990s.

Antimony-mercury mineralisation was established within a set of

stibnite-cinnabar-bearing jasperoid horizons associated with thrust faults.

These jasperoidal rocks form relatively thin, brecciated, lenticular horizons

associated with thrust faults and proximal to the contact of the Carboniferous

limestone formation with the Siluro-Devonian sediments. These thrust faults

are offset laterally in an imbricate NE-SW pattern by a later stage of

faulting.

North KarabiyIn 2019, a total of five core holes were completed on North

Karabiy, which included 2 restarts where holes were prematurely terminated due

to technical issues with drilling. In addition, a total of 5 surface trenches

were exposed, mapped and channel sampled. Results of the drill holes are

detailed in Table 1 and results of surface trenching are detailed in Table 2.

Composite Parameters

<<

1. Minimum Width: No minimum width

2. Interval Grade Cut-Off: 1 g/t Au

3. Maximum Interval Internal Dilution: Maximum of 1 sample <1 g/t Au

<>>

Composite Parameters

<<

1. Minimum Width: No minimum width

2. Interval Grade Cut-Off: 1 g/t Au

3. Maximum Interval Internal Dilution: Maximum of 1 sample <1 g/t Au

<>>

Hole by Hole Summary

DH-NK-1-19DH-NK-1-19 was collared as a vertical hole designed to confirm

the mineralized zone exposed in trench 19NKT01 (9 m (at) 8.37 g/t) and completed

to a depth of 40.0m. A weakly anomalous gold value was intersected within a

brecciated and jasperoidized limestone. Anomalous pathfinder elements of

arsenic, thallium and antimony were encountered in this interval grading; 180

ppm As, 3 ppm Tl and 3,022 ppm Sb.

DH-NK-2-19DH-NK-2-19 was an angled hole designed to confirm the

mineralized zone exposed in trench 19NKT03 (6 m (at) 2.02 g/t) and completed to

a

depth of 29.0m. Gold mineralization was intersected in the top of the hole

from 0m to 1m depth grading 1m (at) 1.2 g/t Au in jasperoids. Anomalous

pathfinder elements of arsenic, thallium and antimony are present in this

mineralized interval with grades over the interval of; 1,756 ppm As, 9 ppm Tl

and 850 ppm Sb. From 0m to 6m, a zone of weakly anomalous gold mineralization

returned 6m (at) 0.6 g/t Au. Anomalous pathfinder elements of arsenic, thallium

and antimony are present in this mineralized interval with grades over the

interval of; 1,525 ppm As, 7.5 ppm Tl and 647 ppm Sb.

DH-NK-3-19DH-NK-3-19 was an angled hole designed to confirm the

mineralized zone exposed in trench 19NKT02 (3 m (at) 5.48 g/t) completed to a

depth of 38m. Gold mineralization was encountered in a zone of tectonic

breccia with fragments of siltstone and sandstone and limonitization in

fractures within fault gouge in an interval of 2m (at) 6.2 g/t Au (0.0m to 2.0m

depth). Anomalous pathfinder elements of arsenic, thalium and antimony were

elevated in this zone with grades of; 2,249 ppm As, 11.5 ppm Tl and 58 ppm Sb.

DH-NK-4-19DH-NK-4-19 was an angled hole designed to confirm the

mineralized zone exposed in trench 19NKT04 (14 m (at) 7.79 g/t), terminated at a

depth of 5.7m due to problems with the drill. Gold mineralization was

encountered in a narrow interval 0f 0.7m (at) 4.6 g/t Au within jasperoid.

Anomalous pathfinder elements of arsenic, thalium and antimony were

encountered in this interval with interval grades of; 2,391 ppm As, 12 ppm Tl

and 92 ppm Sb. The entire interval consisted of tectonic breccia with blocks

and fragments of jasperoid.

DH-NK-4bis-19 DH-NK-4bis-19 was an angled hole designed to confirm the

mineralized zone exposed in trench 19NKT04 (14 m (at) 7.79 g/t) and completed to

a depth of 50.6m. The hole was a redrill of DH-NK-4-19. Two intervals of gold

mineralization were encountered in this hole. Gold mineralization was

intersected from 4.0m to 9.0m grading 5m (at) 2.7 g/t Au in a tectonic breccia

with jasperoid. Within this interval, elevated pathfinder pathfinder elements

of arsenic, thallium and antimony graded; 758 ppm As, 5 ppm Tl and 21,543 ppm

Sb . Stibnite was found throughout this interval (as pockets and veinlets and

also disseminated). A second zone of gold mineralization was intersected from

36m to 38m grading 2m (at) 1.6 g/t Au within a limonitized tectonic breccia

consisting of sandstone and siltstone fragments. Within this interval elevated

pathfinder elements of arsenic, thallium and antimony graded; 2,669 ppm As,

12.5 ppm Tl and 105 ppm Sb.

DH-NK-5-19DH-NK-5-19 was collared as a vertical hole designed to confirm

the mineralized zone intersected by historic Soviet-era DH 46 (7.83 g/t Au (at)

7.95 m) and DH46? (7.4 g/t Au (at) 15.3 m). The hole was terminated at 164.0m

due

to drill breakdown. No downhole survey data exists for these historic Soviet

holes, so positioning of historic mineralization is unknown. Gold

mineralization was encountered in a 3m wide interval from 148m to 151m (3.0m

(at)

1.1 g/t Au) hosted in a brecciated black, carbonaceous and pyritized shale

above jasperoids. Within this interval elevated pathfinder elements of

arsenic, thallium and antimony graded; 1,035 ppm As; 8 ppm Tl; and 75 ppm Sb.

DH-NK-5bis-19DH-NK-5bis-19 was collared as a vertical hole designed to

confirm the mineralized zone intersected by historic Soviet-era DH 46 and

DH46?. No downhole survey data exists for these historic Soviet holes, so

positioning of mineralization is unknown. The hole was completed to a depth of

184.8m. Three zones of gold mineralization were encountered ranging in widths

from 2m to 3m in this hole. An upper zone was intersected in jasperoid breccia

above limestone from 138m to 140m grading 2m (at) 2.6 g/t Au and with elevated

pathfinder elements of arsenic, thallium and antimony grading; 1,230 ppm As, 9

ppm Tl, and 1,857 ppm Sb. A unit of tectonic breccia with fragments of black,

silicified, carbonaceous and weakly pyritized shale, jasperoid and sandstone

was encountered from 149m to 166m with elevated pathfinder elements of

arsenic, thallium and antimony grading; 929 ppm As, 6.35 ppm Tl, and 2,059 ppm

Sb over the interval. Two gold mineralized zones were encountered in this

interval with the upper zone (149m to 152m) grading 3m (at) 5.3 g/t Au and the

lower interval (163m to 166m) grading 3m (at) 1.4 g/t Au.

Slantsevyi KlinThe Slantsevyi Klin area is part of the Karabiy area of

the Alai group. Slantsevyi Klin is located on the western region of Karabiy.

In 2019, a total of six core holes were completed on North Karabiy, which

included 1 restart where the hole was prematurely terminated due to technical

issues with drilling. Results of the drill holes are detailed in Table 3.

Composite Parameters

<<

1. Minimum Width: No minimum width

2. Interval Grade Cut-Off: 1 g/t Au

3. Maximum Interval Internal Dilution: Maximum of 1 sample <1 g/t Au

<>>

Hole by Hole Summary

DH-SK-1-19DH-SK-1-19 was an angled hole designed to confirm the

mineralized zone exposed in trench 18SKR02 (29 m (at) 2.95 g/t) and completed to

a depth of 46.2m. The hole encountered jasperoid from surface to 9.15m and

followed by limestone from 9.15 to the end of hole. Anomalous gold values of

10m (at) 0.3 g/t Au in jasperoid were encountered in the top of the hole.

Anomalous important pathfinder elements including arsenic, thallium and

antimony are present within this mineralized interval with average interval

grades; of 1,023 ppm As, 13.7 ppm Tl, 79.3 ppm Sb.

DH-SK-2-19DH-SK-2-19 was an angled hole designed to confirm the

mineralized zone exposed in trench 18SKR02 and completed to a depth of 23.5m.

The hole was drilled south to north towards hole DH-SK-1-19. The hole

encountered jasperoid from surface to 13.4m and followed by limestone from

13.4m to the end of hole. A mineralized interval of 3m (at) 1.3 g/t Au was

intersected in jasperoid at the top of the hole. Anomalous important

pathfinder elements including arsenic, thallium and antimony are elevated

within this interval with average grades of; 1,372 ppm As, 15.7 ppm Tl; and

1,936 ppm Sb.

DH-SK-3-19DH-SK-3-19 was an angled hole designed to confirm the

mineralized zone exposed in trench 18SKR10 (71 m (at) 2.74 g/t) and completed to

a depth of 80.0m. The hole intersected jasperoid from the top of hole to 43m

and followed by limestone from 43m to end of hole. Multiple zones of gold

mineralization were encountered in the jasperoid between 3.0 to 43.0m depth

with interval widths ranging from 4.0m to 15.0m wide. Anomalous pathfinder

elements of arsenic, thallium and antimony were present throughout the entire

jasperoid unit with grades over a 40m interval (from 3.0m to 43.0m) of; 1,767

ppm As, 13.86 ppm Tl, and 1,049 ppm Sb.

DH-SK-4-19DH-SK-4-19 was an angled hole designed to confirm the

mineralized zone exposed in trench 18SKR10 and completed to a depth of 55.0m.

The hole was drilled from south to north towards DH-SK-3-19. The hole

intersected jasperoid from 1.55m to 9.2m and limestone from 9.2m to end of

hole. Anomalous gold values of 9m (at) 0.13 g/t Au within the jasperoid at the

top of the hole was encountered. In addition, pathfinder elements of arsenic,

thallium and antimony were elevated in this interval with average grades of;

791 ppm As, 7.1 ppm Tl and 127 ppm Sb.

DH-SK-5-19DH-SK-5-19 was an angled hole designed to confirm the potential

of a north-south trending feeder structure, located west of the mineralized

zone exposed in trench 18SKR10 and terminated at a depth of 16.0m due to

technical problems with the drill. Jasperoid was encountered at the top of

hole to the end. A narrow zone of gold mineralization of 1m (at) 1.2 g/t Au was

encountered at the top of hole. Anomalous pathfinder elements of arsenic,

thallium and antimony were elevated in this interval, with grades of; 1,626

ppm As, 7 ppm Tl, and 872 ppm Sb.

DH-SK-5bis-19 DH-SK-5bis-19 was an angled hole designed to retest the

target from DH-SK-5-19 and terminated at a depth of 18.9m due to technical

drilling problems. Jasperoid was encountered at the top of hole to the end.

Gold grades did not exceed 0.096 g/t.

DH-SK-6-19DH-SK-6-19 was a vertical hole designed to confirm the

mineralized zone exposed in trench 18SKR18 (17 m (at) 1.86 g/t) and completed to

a depth of 45.3m. The hole intersected jasperoid from the top of hole to 8.7m

and followed by limestone from 8.7m to end of hole. Gold mineralization was

encountered in jasperoids near the top of the hole from 1.2m to 3.0m depth

(1.8m (at) 1.8 g/t Au). Anomalous pathfinder elements of arsenic, thallium and

antimony were elevated within this interval with grades of; 1,638 ppm As; 12

ppm Tl; and 183 ppm Sb.

Surface trenches were exposed and sampled on both the North Karabiy andUluktau

areas, with strongly mineralized results encountered on both

properties. Results of trenching are detailed below in Tables 3 and 4.

Uluktau Group

The Uluktau area covers 6,147 ha. The property is located 38 km west

southwest of the provincial capital city Osh. Access to the area is via

national highway and provincial roads. The area is largely rolling hills with

the steeper parts associated with limestone outcrops.

Uluktau contains the abandoned Chonkoi mercury mine in the eastern part

of the license. The mine operated during Soviet times, which in turn led to an

extensive drilling campaign in the surrounding district in search of other

mercury deposits by the Soviets. This work included geologic surface and

underground mapping at various scales and drilling (485 diamond drill holes

within or nearby the Uluktau license area).

In 2019, a total of 4 surface trenches were exposed, mapped and sampled.

Results of trench sampling is detailed in Table 4.

Composite Parameters

<<

1. Minimum Width: No minimum width

2. Interval Grade Cut-Off: 1 g/t Au

3. Maximum Interval Internal Dilution: Maximum of 1 sample <1 g/t Au

<>>

All core measurements reported in this news release are down-hole, and

not representative of true-thickness. At this early stage of exploration, true

thicknesses are unknown.

Quality assurance / quality control (QA/QC) measures included samples

from the drill core are cut in half by core saw in half sections on site and

where possible, samples are standardized at 1.0m down-hole intervals. One-half

of the split sample is sent for assaying and half remains on site for

reference. Channel samples in trenches are continuous samples, and where

possible are standardized at 1.0m intervals. All samples were sent to Stewart

Assay and Environmental Laboratories LLC in Kara Balta, Kyrgyz Republic (which

is independent of Rockwealth and Realgold) for analysis which includes fire

assay for gold, Au-AA25, 30g fire assay-lead collection with AA finish, with a

detection range of 0.01-100ppm. The internal QA/QC program at this independent

laboratory includes routine duplicate, blank, and certified reference standard

analysis throughout the sample preparation and analysis. Realgold did not

detect any significant QA/QC issues during its review of the data.

Qualified PersonThe scientific and technical information contained in

this news release as it relates to Realgold and its properties has been

reviewed and approved by Doug Kirwin, CEO of Realgold and a "Qualified Person"

as defined in National Instrument 43-101 - Standards of Disclosure for Mineral

Projects. Mr. Kirwin verified the data disclosed which includes a review of

the analytical and test data underlying the information and opinions contained

therein.

About Realgold Resources Corp.

Realgold has assembled an extensive land portfolio in the Kyrgyz Republic

with Carlin type potential. The 16 properties in 28 licenses covering 274,124

ha is situated within the prolific Tien Shan gold belt and was acquired by

staking. The orogenic gold deposits of the Tien Shan Mineral Belt include some

of the largest economic gold accumulations in the world. The region is host to

several giant gold deposits, including the world's single largest operating

gold mine (Muruntau in Uzbekistan) to the west and the giant Kumtor mine in

eastern Kyrgyzstan to the east end of the belt. Carlin type deposits are the

most important source of gold in the United States and the Tien Shan belt

within the Kyrgyz Republic is now recognized as hosting the 2nd largest

confirmed Carlin terrain in the world. Realgold licenses cover 90% of newly

recognized and unexplored Carlin type gold belt.

Qualified Person

The scientific and technical information contained in this news release

as it relates to Rockwealth has been reviewed and approved by Ross McElroy, a

director of the Company and a "Qualified Person" as defined in National

Instrument 43-101 - Standards of Disclosure for Mineral Projects.

ON BEHALF OF THE BOARD

Realgold Resources Corp.

Douglas KirwinChief Executive Officer

Rockwealth Resources Corp.

Dev RandhawaChief Executive OfficerPhone: 1-250-868-8177

Neither the TSX Venture Exchange nor its Regulation Services Provider (as

that term is defined in the policies of the TSX Venture Exchange) accepts

responsibility for the adequacy or accuracy of this news release.

Cautionary Statement:

Certain information contained in this press release constitutes

"forward-looking information", within the meaning of Canadian legislation.

Generally, these forward-looking statements can be identified by the use of

forward-looking terminology such as "plans", "expects" or "does not expect",

"is expected", "budget", "scheduled", "estimates", "forecasts", "intends",

"anticipates" or "does not anticipate", or "believes", or variations of such

words and phrases or state that certain actions, events or results "may",

"could", "would", "might" or "will be taken", "occur", "be achieved" or "has

the potential to". Forward looking statements contained in this press release

may include statements regarding the potential of the properties, the future

operating or financial performance of Realgold which involve known and unknown

risks and uncertainties which may not prove to be accurate. Actual results and

outcomes may differ materially from what is expressed or forecasted in these

forward-looking statements. Such statements are qualified in their entirety by

the inherent risks and uncertainties surrounding future expectations. Among

those factors which could cause actual results to differ materially are the

following: market conditions and other risk factors listed from time to time

in our reports filed with Canadian securities regulators on SEDAR at

www.sedar.com. The forward-looking statements included in this press release

are made as of the date of this press release and the Company and Realgold

disclaim any intention or obligation to update or revise any forward-looking

statements, whether as a result of new information, future events or

otherwise, except as expressly required by applicable securities legislation.

SOURCE Rockwealth Resources Corp.

View original content to download multimedia:

http://www.newswire.ca/en/releases/archive/October2019/18/c8112.html

-0-                           10/18/2019

/For further information: please contact: Dev Randhawa, Phone:

1-250-868-8177/

(RWR.)

CO:  Rockwealth Resources Corp.

ST:  British Columbia

IN:  MIN MNG PRM

SU:

-30-

#